# Attachment B:  Waste Disposal

This attachment discusses the emissions sources listed below, which are associated with the disposal and management of various wastes.

Description	EIC Code	CES Number

Biological wastes (unspecified)	199-995-0260-0000	66712

Incinerators-pathological waste	130-130-0266-0000	?

Sewage treatment-liquid waste (unspecified)	110-110-0300-0000	?

Volatile organic waste disposal (evaporation)	199-190-0300-0000	82065

Livestock wastes	620-618-0262-0000	66605

Municipal landfills	120-122-0242-0000	57281

## emissions from biological waste disposal

Medical wastes are incinerated or sterilized and sent to landfills (as treated solid wastes).  Sterilization and incineration may be performed by waste generators or by permitted medical waste treatment facilities.  ARB regulates the allowable emissions from medical waste incinerators, which are issued permits to operate by local Air Quality Management Districts.

In 1990, about 20 percent of the state’s hospitals operated their own onsite incinerators. The remainder either sterilized and landfilled their wastes or shipped them to regional incinerators.  In 1990 ARB adopted a control measure to reduce emissions of dioxins from medical waste incinerators by 99 percent. Section 93104, Title 17 of California Code of Regulations applied the reduction requirement to medical incinerators that incinerate more than 25 tons per year, and required more effective operation of many small incinerators.

The Medical Waste Management Act (California Health and Safety Code Sections 117600 – 118360) requires local agencies to produce a Medical Waste Management Plan and requires registration of generators, transfer stations, treatment facilities, and haulers of medical waste with local agencies.  In addition, treatment facilities and transfer stations must be inspected periodically by enforcement agencies.  The California Health Services Department (DHS) maintains a database of entities registered as treatment facilities, transfer stations, and small or large generators of medical wastes.  Generators of medical wastes qualify as large generators if they produce 200 or more pounds of medical wastes in any month.  The majority of medical practices registered small generators.  Most of these entities use medical-waste haulers and off-site treatment.

As of Oct. 17, 2001 DHS reported that there are 11 permitted offsite medical waste treatment facilities in California.  Only one of these employs incineration (rather than steam or microwave sterilization).  This facility is located in Alameda County (in the city of Oakland), which is not among the CCOS II participating counties.  According to Mr. Jack McGurk, chief of the DHS Environmental Management Branch, only 10 to 12 on-site incinerators currently remain in use at hospitals statewide.  STI investigated those that were said to be located within any of the CCOS II participating counties.  Two on-site incinerators were determined to have been closed, which left only one remaining in operation.  Through telephone contact, the Ukiah Valley Hospital (in Mendocino County) was found to incinerate about 10,000 lb/yr of medical waste (or 5 tpy).  In California, about 35,000 tons of medical waste are processed annually across the state (according to Mr. McGurk).

All of EPA’s recommended emission factors for medical waste incineration are less than 5 lb of air pollutant per ton of waste incinerated (EPA, 1995a).  (This applies to all pollutants of interest: NOx, SOx, CO, TOC, PM.)  Therefore, among CCOS II participating counties, emissions from medical waste incineration are insignificant given the small quantities of wastes that are processed.

## emissions from Pathological Waste Incinerators

This category includes emissions from human and animal crematories.  The Cremation Association of North America (CANA), a business association, estimated that 108,617 human cremations would be performed in California during the year 2000 (CANA, 2001).  The Emission Inventory Improvement Program provides an emission factor of 5.59  10-5 lb PM per cremation (Emission Inventory Improvement Program, 2001).  Thus, statewide year-2000 PM emissions from human cremations are estimated to be negligible as follows.

108,617 cremations  (5.59  10-5 lb PM per cremation) = 6.1 lb PM

Recent emissions studies have found that other criteria air pollutants are emitted at rates not above one order of magnitude larger than the PM emission rate (CANA, 2000; AEA Technology Environment, 2002).  In addition, national-scale inventories have included emissions estimates for animal crematories that are much smaller than those for human crematories (Eastern Research Group, 2001).  Therefore by inference, emissions from human crematories of non-PM criteria pollutants and emissions from animal crematories are considered to be negligible.

## emissions from Unspecified Sewage Treatment of Liquid Waste

Emissions from industrial and publicly-owned municipal sewage treatment facilities are treated as point sources of TOG and ROG in the California inventory.  (EPA’s emissions model, WATER9, is the recommended emissions estimation method.)  Distributed sewer systems for publicly-owned treatment facilities also are sources of TOG and ROG emissions, but these are treated area sources.  To date, it appears that distributed sewer systems have been a largely neglected emissions source category.  We have been unable to identify existing emissions inventories that include this category nor factors that could be scaled according to readily available information, such as process throughputs or sizes of populations served.  Available emissions estimation techniques, such as the use of WATER9, require detailed information about the sewage chemical composition, environmental conditions, and physical design structures of sewers.  The entities who are in the best position to acquire these types of data are city sewer districts, or city engineering and planning departments.  A list of 169 publicly-owned treatment facilities in the CCOS II counties was obtained from the State Water Resources Control Board (see Table 1).

| Table 1. Sewage treatment facilities that handle municipal waste streams in the CCOS II participating counties. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Facility ID | NPDES No. | Agency | Facility Name | Facility City | Facility County | Design Flow (Mgd) | Baseline Flow (Mgd) |
| 5B030100001 |  | Amador, City Of | Amador City WW Export System | Amador City | Amador | 0 | 0 |
| 5B030102001 |  | Ione, City Of | City Of Ione WWTP | Ione | Amador | 0 | 0 |
| 5B031009001 |  | East Bay Mud | Pardee Recreation Area | Ione | Amador | 0 | 0 |
| 5B030109001 |  | Amador Reg.San. Authority | Amador Co.Regional Outfall | Ione | Amador | 0 | 1.2 |
| 5B030103001 | CA0079391 | Jackson, City Of | City Of Jackson WWTP | Jackson | Amador | 0.71 | 0.424 |
| 5B030106001 |  | Plymouth, City Of | Plymouth Wastewater Trt Plant | Plymouth | Amador | 0.17 | 0.1 |
| 5B030110001 |  | Amador Co Pwd | River Pines WWTP | River Pines | Amador | 3.5 | 0 |
| 5B030104001 |  | Sutter Creek, City Of | Sutter Creek WWTP | Sutter Creek | Amador | 0.48 | 0.35 |
| 5A040100001 | CA0078930 | Biggs, City Of | Biggs WWTP | Biggs | Butte | 0.53 | 0.219 |
| 5A040102001 | CA0079081 | Chico, City Of | Chico Regional WWTF | Chico | Butte | 9 | 7.2 |
| 5A041041001 |  | Butte Csa #135 | Keefer Creek Subdiv, Phase II | Chico | Butte | 0.006 | 0.006 |
| 5A041045001 |  | Butte Csa #169 | Pheasant Landing Subdivision | Chico | Butte | 0 | 0 |
| 5A040103001 |  | Gridley, City Of | Gridley WWTF | Gridley | Butte | 1 | 0 |
| 5A040108001 |  | Butte Csa #21 | Butte Csa #21 | Oroville | Butte | 0.002 | 0.002 |
| 5A040121001 |  | Butte Csa #141 | Mops Inc | Oroville | Butte | 0.014 | 0.014 |
| 5A040106001 | CA0079235 | Sewer Comm - Oroville Region | Oroville WWTP | Oroville | Butte | 6.5 | 3 |
| 5A040105001 |  | Richvale Sd | Richvale STP | Richvale | Butte | 0.25 | 0.03 |
| 5A040113001 |  | Butte Csa #82 | Stirling City Ponds | Stirling City | Butte | 0 | 0 |
| 5B050100002 |  | Angels, City Of | Angels Camp WWTP | Angels Camp | Calaveras | 0.4 | 0.33 |
| 5B050107008 |  | Calaveras Co Water Dist | Mill Woods WWTF | Arnold | Calaveras | 0.088 | 0 |
| 5B050107005 |  | Calaveras Co Water Dist | Arnold WWTP | Arnold | Calaveras | 0.175 | 0.031 |
| 5B050106001 |  | Calaveras Co Water Dist | Big Trees Country Houses | Camp Connell | Calaveras | 0.002 | 0.0005 |
| 5B050105001 |  | Calaveras Co Water Dist | Copper Cove WWTP | Copper Cove Village | Calaveras | 0.12 | 0.081 |
| 5B051003001 |  | Calaveras Co Water Dist | Douglas Flat/Vallecito WWTP | Douglas Flat | Calaveras | 0.065 | 0.025 |
| 5B050101001 |  | Mokelumne Hill Sanitary Dist. | Mokelumne Hill WWTF | Mokelumne Hill | Calaveras | 0 | 0 |
| 5B051003002 |  | Calaveras Co Water Dist | Indian Rock Vineyards Subdiv. | Murphys | Calaveras | 0.006 | 0 |
| 5B050107004 |  | Calaveras Co Water Dist | Forest Meadows | Murphys | Calaveras | 0.045 | 0.027 |
| 5B050102001 |  | Murphys Sanitary District | Murphys WWTF | Murphys | Calaveras | 1.08 | 0.23 |
| 5B050103001 | CA0079464 | San Andreas Sanitary Dist. | San Andreas WWTF | San Andreas | Calaveras | 0.4 | 0.3 |
| 5B051022001 |  | East Bay Mud | Ebmud Pardee Center WWTF | Valley Springs | Calaveras | 0.003 | 0 |
| 5B050104001 |  | Valley Springs Sd | Valley Springs Sd WWTF | Valley Springs | Calaveras | 0.065 | 0.037 |
| 5B051003004 |  | Calaveras Co Water Dist | Southworth Ranch Estates WWTF | Wallace | Calaveras | 0.017 | 0 |
| 5B050107007 |  | Wallace Community Service Dist | Wallace Lake Estates WWTF | Wallace | Calaveras | 0.04 | 0 |
| 5B052000001 |  | Calaveras Co Water Dist | Wilseyville Comm.Sewage System | Wilseyville | Calaveras | 0 | 0 |
| 5B052009002 |  | Calaveras Co Water Dist | West Point WWTP | Wilseyville | Calaveras | 0.058 | 0.058 |
| 5A010100001 |  | Arbuckle Pud | Arbuckle Pud WWTP | Arbuckle | Colusa | 0.5 | 0.24 |
| 5A060101001 | CA0078999 | Colusa, City Of | Colusa WWTP | Colusa | Colusa | 0.9 | 0.66 |
| 5A060102001 | CA0079987 | Maxwell P.U.D. | Maxwell Pud WWTF | Maxwell | Colusa | 0.2 | 0.14 |
| 5A060104001 |  | Princeton Water Works Dist. | Princeton Wtr Works Dist WWTP | Princeton | Colusa | 0 | 0 |
| 5A060103001 | CA0077933 | Williams, City Of | Williams WWTP | Williams | Colusa | 0.5 | 0.3 |
| 5B090102001 | CA0078662 | El Dorado Id | Deer Creek WWTP | Cameron Park | El Dorado | 2.5 | 2.4 |
| 5A090102010 |  | El Dorado Id | Camino Heights WWTP | Camino | El Dorado | 0 | 0 |
| 5A091007001 |  | Georgetown Divide Pud | Auburn Lk Trls Onsite WW Disp | Cool | El Dorado | 0.0718 | 0.024 |
| 5B090102005 | CA0078671 | El Dorado Id | El Dorado Hills WWTP | El Dorado Hills | El Dorado | 3 | 1.5 |
| 5A090100001 | CA0078956 | Placerville, City Of | Hangtown Creek WWTP | Placerville | El Dorado | 3.9 | 1.038 |
| 5A090102008 |  | El Dorado Id | Gold Ridge Forest Unit 3 | Pollock Pines | El Dorado | 0.0125 | 0.0125 |
| 5A090102003 |  | El Dorado Id | Rancho Ponderso | Rescue | El Dorado | 0 | 0 |
| 6A095900700 |  | South Tahoe Pud | STPud Wastewater Trtment Plant | South Lake Tahoe | El Dorado | 7.7 | 4.5 |
| 5A110103001 |  | Hamilton City Csd | Hamilton City Csd WWTP | Hamilton City | Glenn | 0.5 | 0 |
| 5A110100001 |  | Orland, City Of | City Of Orland WWTP | Orland | Glenn | 2.1 | 1.029 |
| 5A110101001 | CA0078034 | Willows, City Of | Willows WWTP | Willows | Glenn | 1.12 | 1 |
| 5C221012001 | CA0079430 | Mariposa Pud | WWTP | Mariposa | Mariposa | 0.61 | 0.11 |
| 5C221002001 |  | Yosemite West Sd | WWTF | Wawona | Mariposa | 0.1 | 0.022 |
| 1B77162OMEN |  | Calpella Cwd | Calpella Cwd-WWTP | Calpella | Mendocino | 0.04 | 0.027 |
| 1B83009OMEN | CA0023574 | Covelo Csd | Covelo City POTW | Covelo | Mendocino | 0.08 | 0.041 |
| 1B84083OMEN | CA0023078 | Fort Bragg, City Of | Fort Bragg City WWTP | Fort Bragg | Mendocino | 1 | 0.82 |
| 1B89005RMEN |  | Gualala Csd | Gualala WWTF | Gualala | Mendocino | 0.0001 | 0.131 |
| 1B83118OMEN | CA0024040 | Mendo Co Wwd #2 Anchor Bay | Mendocino CWWd#2-Anchor Bay | Gualala | Mendocino | 0.0196 | 0.009 |
| 1B80154OMEN |  | Hopland Pud | Hopland Public Utilities Dist. | Hopland | Mendocino | 0.06 | 0.049 |
| 1B80023OMEN |  | Irish Beach Water District | Irish Beach Subd.Units 1,2,3,4 | Manchester | Mendocino | 0.0001 | 0.0001 |
| 1B82076OMEN |  | Caspar South Wd | Caspar South Water District | Mendocino | Mendocino | 0.013 | 0.006 |
| 1B83129OMEN | CA0022870 | Mendocino City Csd | Mendocino City Csd | Mendocino | Mendocino | 0.3 | 0.142 |
| 1B80045OMEN |  | Point Arena, City Of | Point Arena Wastewater Trt.Plt | Point Arena | Mendocino | 0.13 | 0.028 |
| 1B84029OMEN | CA0022888 | Ukiah, City Of | Ukiah City WWTP | Ukiah | Mendocino | 2.8 | 2.25 |
| 1B75260OMEN |  | Westport Cwd | WeSTPort Cwd | Westport | Mendocino | 0.029 | 0.022 |
| 1B80078OMEN | CA0023060 | Willits, City Of | Willits City WWTP | Willits | Mendocino | 1.3 | 0.7 |
| 6A290002225 |  | Floriston, Community Of | Floriston Waste Trt Facility | Floriston | Nevada | 0 | 0 |
| 5A291025001 |  | Kingsbury Greens Csd | Kingsbury Greens Csd WWTP | Grass Valley | Nevada | 0.013 | 0.011 |
| 5A290100001 | CA0079898 | Grass Valley, City Of | Grass Valley, City Of STP | Grass Valley | Nevada | 3.8 | 1.32 |
| 5A290104003 | CA0081612 | Nevada Co Sd #1 | Lake Of The Pines WWTP | Lake Of The Pines | Nevada | 0.72 | 0.54 |
| 5A290104001 | CA0077828 | Nevada Co Sd #1 | Lake Wildwood WWTP | Lake Wildwood | Nevada | 1.12 | 0.47 |
| 5A290107002 | CA0083241 | Nevada Co Sd #1 | Cascade Shores WWTP | Nevada City | Nevada | 0.021 | 0.014 |
| 5A290106002 |  | Nevada Id | Scotts Flat Reservoir Rec Area | Nevada City | Nevada | 0 | 0 |
| 5A290101001 | CA0079901 | Nevada City, City Of | Nevada City WWTP | Nevada City | Nevada | 0.69 | 0.41 |
| 5A290107001 |  | Nevada Co Sd #1 | North San Juan WW Facility | North San Juan | Nevada | 0.024 | 0.024 |
| 5A290104006 |  | Nevada Co Sd #1 | Penn Valley WWTP | Penn Valley | Nevada | 0.09 | 0.09 |
| 5A290105001 | CA0081621 | Donner Summit Public Utility | Donner Summit WWTP | Soda Springs | Nevada | 0.82 | 0.15 |
| 6A290011000 |  | Tahoe Truckee Sanitation Agncy | Tahoe Truckee Sanitation Agen | Truckee | Nevada | 4.83 | 4.83 |
| 5A310104012 |  | Placer Co Facility Services 1 | Applegate WTF, Zone 24 | Applegate | Placer | 0 | 0 |
| 5A310104007 | CA0079316 | Placer Co Facility Services 1 | Placer Co Smd No 1 | Auburn | Placer | 0.95 | 1.202 |
| 5A310100001 | CA0077712 | Auburn, City Of | Auburn WWTP | Auburn | Placer | 1.67 | 1.35 |
| 5A290106001 |  | Nevada Id | Rollins Reservoir Rec Areas | Colfax | Placer | 0 | 0 |
| 5A310101001 | CA0079529 | Colfax, City Of | Colfax STP | Colfax | Placer | 0.16 | 0.12 |
| 5A310104018 | CA0079367 | Placer Co Facility Services 1 | Placer Co Smd No 3 | Granite Bay | Placer | 0.75 | 0.1 |
| 5A310108001 |  | South Sutter Water District | Camp Far West Res South Side | Lincoln | Placer | 0 | 0.075 |
| 5A31NP00011 | CA0084476 | Lincoln, City Of | City Of Lincoln WWTP | Lincoln | Placer | 1.4 | 0.82 |
| 5A310102002 |  | Lincoln, City Of | City Of Lincoln WWTP | Lincoln | Placer | 1.4 | 0.82 |
| 5A310103001 |  | Newcastle Sanitary District | Newcastle STP | Newcastle | Placer | 0 | 0 |
| 5A310104016 |  | Placer Co Facility Services 1 | Sunset Septage Receiving Fac | Rocklin | Placer | 0 | 0 |
| 5A310106007 | CA0084573 | Roseville, City Of | Pleasant Grove WWTP | Roseville | Placer | 12 | 6 |
| 5A310106001 | CA0079502 | Roseville, City Of | Dry Creek WWTP | Roseville | Placer | 18 | 10 |
| 5A310106006 |  | Roseville, City Of | Master Reclamation Permit | Roseville | Placer | 6 | 0.0001 |
| 5A310104011 | CA0079341 | Placer Co Facility Services 1 | Sa No 28, Zone No.6 | Sheridan | Placer | 0.1 | 0.1 |
| 5A320104003 |  | Beckwourth Csa | Beckwourth WW Ponds | Beckwourth | Plumas | 0 | 0 |
| 5A320104004 |  | Plumas Eureka Csd | Plumas Eureka Estates WWTF | Blairsden | Plumas | 0 | 0 |
| 5A321021001 |  | Gold Mountain Csd | Gold Mountain WWTF | Blairsden | Plumas | 0.1 | 0.05 |
| 5A320104002 |  | West Almanor Csd | Lake Almanor West | Chester | Plumas | 0 | 0 |
| 5A320100001 | CA0077747 | Chester Pud | Chester STP | Chester | Plumas | 0.75 | 0.5 |
| 5A321018001 |  | Whitehawk Ranch Mutual Wtr Co | Whitehawk Ranch WWTP | Clio | Plumas | 0.134 | 0.134 |
| 5A321010001 | CA0081744 | Grizzly Lake Resort Imp Dist | Dellecker WWTP | Dellecker | Plumas | 0.01 | 0.001 |
| 5A321019001 |  | Gold Ridge Estates Ha | Gold Ridge Ha, Unit III | Graeagle | Plumas | 0.007 | 0.007 |
| 5A320101001 |  | Indian Valley Csd | Greenville WW Ponds | Greenville | Plumas | 0.4 | 0.4 |
| 5A320102001 | CA0077844 | Portola, City Of | Portola WWTP | Portola | Plumas | 0.35 | 0.35 |
| 5A321001001 |  | Grizzly Lake Resort Imp Dist | Crocker Mountain Estates STP | Portola | Plumas | 0.2 | 0.002 |
| 5A320103001 | CA0078981 | Quincy Csd | Quincy WWTP | Quincy | Plumas | 2.5 | 0.5 |
| 5A320104007 |  | Indian Valley Csd | Taylorsville STP | Taylorsville | Plumas | 0 | 0 |
| 5B340107005 |  | Sacramento Co Dpw-Elk Grove | Courtland WWTF | Courtland | Sacramento | 0 | 0 |
| 5A340114004 |  | Sacramento Regional Csd-Elk Gv | Regional Reclamation Facility | Elk Grove | Sacramento | 10 | 5 |
| 5A340108002 | CA0077682 | Sacramento Regional Csd-Sacto | Sacramento Regional WWTP | Elk Grove | Sacramento | 181 | 145 |
| 5A34NP00016 | CA0084816 | Folsom, City Of | Folsom Sewage Collection Sys | Folsom | Sacramento |  |  |
| 5B340101001 | CA0081434 | Galt, City Of | Galt SD | Galt | Sacramento | 3 | 1.45 |
| 5B340103001 |  | Isleton, City Of | City Of Isleton WWTP | Isleton | Sacramento | 0.43 | 0.117 |
| 5B340109001 |  | Rancho Murieta C.S.D. | Rancho Murieta Csd WWT & Rec | Rancho Murieta | Sacramento | 2.4 | 1 |
| 5A340114001 | CA0079111 | Sacramento Regional Csd-Sacto | Combined WW Collection/Trt Sys | Sacramento | Sacramento | 130 | 130 |
| 5A340115001 | CA0082597 | Sacramento Co Water Agency | Sacramento Ms4 Permit | Sacramento | Sacramento | 0.0001 | 0.0001 |
| 5A340107010 | CA0078794 | Sacramento Regional Csd-Elk Gv | Walnut Grove WWTP (Csd1) | Walnut Grove | Sacramento | 0.5 | 0.5 |
| 5A450100001 | CA0077704 | Anderson, City Of | Anderson Wpcp | Anderson | Shasta | 1 | 0.9 |
| 5A450103006 |  | Redding, City Of | Stillwater WWTP Reclamation | Anderson | Shasta | 4 | 2 |
| 5A450103001 | CA0079731 | Redding, City Of | Clear Creek WWTP | Anderson | Shasta | 8.8 | 7.5 |
| 5A450103004 | CA0082589 | Redding, City Of | Stillwater WWTF | Anderson | Shasta | 8 | 1 |
| 5A450107001 |  | Burney Wd | Burney STP | Burney | Shasta | 0.44 | 0.35 |
| 5A450001005 | CA0081507 | Shasta Csa #17 | Cottonwood WWTP | Cottonwood | Shasta | 0.43 | 0.2 |
| 5A520104001 | CA0077852 | Rio Alto Wd | Lake California WWTP | Cottonwood | Shasta | 0.64 | 0.12 |
| 5A450115001 |  | Anderson Union High School Dis | West Valley High School | Cottonwood | Shasta | 0 | 0 |
| 5A450117001 |  | Fall River Mills Csd | Fall River Mills STP | Fall River Mills | Shasta | 0.726 | 0.06 |
| 5A450001004 |  | Shasta Csa #8 | Palo Cedro WWTF | Palo Cedro | Shasta | 0.06 | 0.03 |
| 5A450105001 | CA0079511 | Shasta Lake, City Of | Shasta Lake WWTF | Shasta Lake | Shasta | 0.7 | 0.64 |
| 5A460101001 |  | Sierra Co Serv Area 5 Zone 5a | Sierra Brooks Subdivision |  | Sierra | 0 | 0 |
| 5A460101001 |  | Sierra Co Serv Area 5 Zone 5a | Sierra Brooks Subdivision |  | Sierra | 0 | 0 |
| 5A460100001 |  | Loyalton, City Of | City Of Loyalton WWTP | Loyalton | Sierra | 0 | 0.275 |
| 2 482001001 | CA0038091 | Benicia, City Of | Benicia WWTP | Benicia | Solano | 3 | 2.4 |
| 5A480101001 |  | Dixon, City Of | Dixon STP | Dixon | Solano | 0 | 1.65 |
| 5A480105002 | CA0077691 | Vacaville, City Of | Easterly Sewage Trt Plant | Elmira | Solano | 10 | 6.5 |
| 2 482005001 | CA0038024 | Fairfield-Suisun Sewer Distrct | Subregional WWTP | Fairfield | Solano | 17.5 | 12.7 |
| 2 482005002 |  | Fairfield-Suisun Sewer Distrct | Fairfield-Reclamation | Fairfield | Solano | 0.0001 | 0.0001 |
| 5A480108001 | CA0083771 | Rio Vista & Blackhawk Venture | Summerset At Rio Vista | Rio Vista | Solano | 0.2 | 0.18 |
| 5A480104001 | CA0079588 | Rio Vista, City Of | Rio Vista Waste Trt Facility | Rio Vista | Solano | 0.42 | 0.4 |
| 2 482034001 |  | Twin Creeks | Solano Twin Creek STP | Suisun City | Solano | 0.009 | 0.008 |
| 2 482012001 | CA0037699 | Vallejo San And Flood Cont Dis | Vallejo Sfcd WWTP | Vallejo | Solano | 12.5 | 10.5 |
| 5A510100001 | CA0079022 | Live Oak, City Of | City Of Live Oak WWTP | Live Oak | Sutter | 1.6 | 1.6 |
| 5A510102003 |  | Sutter Co Dpw | Robbins WWTF | Robbins | Sutter | 0.03 | 0 |
| 5A510102002 |  | Sutter Co Dpw | Rio Ramaza WW Disposal Ponds | Sutter County | Sutter | 0.168 | 0 |
| 5A511002001 |  | Sutter Co Dpw | Stonegate Village STP | Yuba City | Sutter | 0.021 | 0.021 |
| 5A510101001 | CA0079260 | Yuba City | Yuba City WW Reclamation Plant | Yuba City | Sutter | 7 | 4.3 |
| 5A520100001 | CA0004995 | Corning, City Of | Corning Indust/Domestic WWTF | Corning | Tehama | 1.38 | 0.83 |
| 5A520105001 |  | Gerber-Las Flores Csd | Gerber STP | Gerber | Tehama | 0.11 | 0.11 |
| 5A520102001 | CA0084069 | Tehama Co Sd #1 | Mineral WWTP | Mineral | Tehama | 0.75 | 0.07 |
| 5A520101001 | CA0078891 | Red Bluff, City Of | Red Bluff WW Reclamation Plant | Red Bluff | Tehama | 2.3 | 1.4 |
| 5A520106001 |  | Golden Meadows Csd | Golden Meadows STP | Red Bluff | Tehama | 0 | 0 |
| 5C550111001 |  | Leland Meadow Water District | Leland Meadow WTP |  | Tuolumne | 0 | 0 |
| 5C550100001 |  | Groveland Comm. Serv. District | Groveland STP | Groveland | Tuolumne | 0.15 | 0.95 |
| 5C550101001 |  | Jamestown Sanitary District | Jamestown Sanitary District | Jamestown | Tuolumne | 1.01 | 0.135 |
| 5C551001001 |  | San Francisco, City & Co Of | Moccasin Powerhouse | Moccasin | Tuolumne | 0 | 0 |
| 5C550106006 |  | Pinecrest Permittees Assoc. | Pinecrest WWTP | Pinecrest | Tuolumne | 0 | 0.08 |
| 5C550106004 |  | Tuolumne Utilities District | Sonora Regional WWTF | Sonora | Tuolumne | 2.6 | 0.6 |
| 5C550106003 |  | Tuolumne Utilities District | Mi-Wuk Village | Sonora | Tuolumne | 0.03 | 0.03 |
| 5C550104001 |  | Tuolumne City Sanitary Dist | Tuolumne STP | Tuolumne | Tuolumne | 0 | 0.07 |
| 5B55NC00004 |  | Tuolumne Utilities District | Twain Harte WWTP | Twain Harte | Tuolumne |  |  |
| 5A570100001 | CA0079049 | Davis, City Of | City Of Davis STP | Davis | Yolo | 7.5 | 5.5 |
| 5A570108001 |  | Esparto Csd | Esparto Csd - Sewage Trt Fac | Esparto | Yolo | 0 | 0 |
| 5A570102001 |  | Madison Service Dist | Madison Service Dist WTF | Madison | Yolo | 0.141 | 0.135 |
| 5B570103001 | CA0079171 | West Sacramento, City Of | West Sacramento STP | West Sacramento | Yolo | 7.5 | 5 |
| 5A570104001 |  | Winters, City Of | Winters WWTP | Winters | Yolo | 0 | 0 |
| 5A570105001 | CA0077950 | Woodland, City Of - Domestic | Woodland WWTP | Woodland | Yolo | 7.5 | 5 |
| 5A570101001 |  | Knights Landing Service Dist. | Knights Landing Serv Dist-WDF | Yolo County | Yolo | 0 | 0 |
| 5A580100002 | CA0079651 | Linda Co Water District | Linda Co Wtr Dist Wpcp | Marysville | Yuba | 1.8 | 1.3 |
| 5A580101001 |  | Marysville, City Of | City Of Marysville WWTP | Marysville | Yuba | 1.7 | 1.51 |
| 5A580102001 | CA0077836 | Olivehurst Pud | Olivehurst WWTP | Olivehurst | Yuba | 1.8 | 1 |
| 5A580103001 |  | Wheatland, City Of | City Of Wheatland WWTP | Wheatland | Yuba | 0.5 | 0.227 |

## Emissions from VOC Waste Disposal

The California Integrated Waste Management Board (IWMB) estimates the quantities of various materials that are generated as landfill waste in each county of California (IWMB, 2001a).  These materials include VOC-rich materials, such as paints, household hazardous materials/solvents, and vehicle/equipment fluids, as well as many other types of low-VOC waste materials.  (Note that negligible VOC contents are assumed for all other reported types of materials, including used asphalt paving/roofing materials, used motor oil, and many others.)

Many counties of California export their landfill wastes out of state for disposal (IWMB 2001b).  The IWMB estimates the ratios of exported wastes to total wastes generated in each county of California (IWMB, 2001b).  In addition, the IWMB estimates quarterly and annual quantities of waste accepted at landfills in each county of California (IWMB, 2001c).

In order to estimate emissions from landfilled VOC wastes, we will employ the following assumptions.

- The majority of emissions occur in the counties that receive landfilled wastes.

- The majority of emissions occur within 3 months of disposal.

The equation below illustrates the method that is proposed to estimate the quantities of VOC-rich wastes accepted by landfills in each county.

Wi=Pi×Gj×(1−Ej)

where:

Wi 	=	Quantity of waste accepted in county i

Gj 	= 	Quantity of waste generated in county j

Pi 	= 	Proportion of total statewide landfill wastes that are accepted in county i

Ej 	= 	Fraction of waste generated in county j that is exported out of state.

In order to estimate emissions from VOC-rich landfilled materials, we will employ the following assumptions.

- The density of disposed paint is approximately 10 lb/gal.

- The VOC content of commercial paints is 330 g/L, which is consistent with the range of VOC contents for solvent-borne paints that was determined by the ARB (1999).

- The VOC content of household paints is 150 g/L, which is consistent with the range of VOC contents for water-borne paints that was determined by the ARB (1999).

- The VOC content of disposed vehicle/equipment fluids is 1.5% by weight.  This is consistent with materials safety data sheets (MSDS) for motor engine oil.

- The VOC content of household solvents/hazardous wastes is 30% by weight.  This is consistent with the ranges of VOC contents that are defined by the ARB and EPA as maximum allowable limits for consumer products.

### Example Calculations

The IWMB (2001a) estimates that Solano County generates 89 tons of commercial paint wastes per year.  In addition, Solano County exports 3.6% of its wastes to out-of-state landfills (IMWB, 2001b).  Thus, the total amounts of commercial paint wastes that are generated in Solano County and disposed within California are calculated as follows.

89 tons  (1 – 0.036) = 85.8 tons per year

Similar estimates were calculated for each of California’s counties and summed to estimate that 924 tons per year of commercial paint wastes were disposed in California’s landfills in year 2000.  During the first quarter of 2000, the IMWB estimates that landfills in Solano County accepted 0.32% of the total annual waste landfilled in California.  VOC emissions from landfilled commercial paint wastes during the first quarter of 2000 are estimated for Solano County as follows.

924 tons  0.0032 10 lb/gallon  330 g/L  454 g/lb  3.785 L/gal = 0.81 tpy VOC

Table 1 tabulates the results of similar calculations that were carried out for each of California’s counties.  The figures in Table 1 represent the sums of emissions for all VOC-rich materials (commercial paints, residential paints, household solvents/hazardous materials, and vehicle/equipment fluids).  These emissions are generally small when compared to total VOC emissions estimates for landfills.  Possible exceptions are Sacramento, Yolo, Calaveras, and Amador counties, where emissions from disposal of VOC-rich materials may account for around 4% to 9% of total landfill emissions.  For counties where emissions of VOC-rich landfill wastes are potentially important, it may be useful to re-allocate a portion of the total VOC emissions from landfills to this more specific category.

| Table 1. Quarterly VOC emissions (tons) due to landfill disposal of VOC-rich wastes. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| County | Annual Total | Quarter 1 | Quarter 2 | Quarter 3 | Quarter 4 |  |
| Alameda | 54.6 | 13.3 | 13.7 | 14.1 | 13.5 |  |
| Amador | 0.9 | 0.2 | 0.2 | 0.4 | 0.2 |  |
| Butte | 3.8 | 0.9 | 1.0 | 0.9 | 0.9 |  |
| Calaveras | 0.8 | 0.2 | 0.2 | 0.2 | 0.2 |  |
| Contra Costa | 21.8 | 4.5 | 5.8 | 5.8 | 5.7 |  |
| Del Norte | 0.4 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| El Dorado | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Fresno | 16.8 | 4.2 | 4.2 | 4.3 | 4.0 |  |
| Glenn | 0.5 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Humboldt | 0.3 | 0.2 | 0.1 | 0.0 | 0.0 |  |
| Imperial | 4.4 | 1.1 | 1.2 | 1.0 | 1.1 |  |
| Inyo | 0.4 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Kern | 19.5 | 4.5 | 4.4 | 4.7 | 5.8 |  |
| Kings | 4.7 | 1.0 | 1.0 | 1.1 | 1.6 |  |
| Lake | 1.1 | 0.2 | 0.3 | 0.3 | 0.3 |  |
| Lassen | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Los Angeles | 251.0 | 61.0 | 64.7 | 63.8 | 61.6 |  |
| Madera | 2.3 | 0.6 | 0.6 | 0.6 | 0.6 |  |
| Marin | 9.2 | 2.4 | 2.3 | 2.3 | 2.2 |  |
| Mariposa | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Mendocino | 1.5 | 0.3 | 0.5 | 0.5 | 0.2 |  |
| Merced | 5.2 | 1.3 | 1.3 | 1.3 | 1.3 |  |
| Mono | 0.7 | 0.1 | 0.2 | 0.2 | 0.1 |  |
| Monterey | 11.0 | 2.4 | 2.8 | 3.0 | 2.8 |  |
| Napa | 1.2 | 0.2 | 0.3 | 0.3 | 0.3 |  |
| Orange | 114.5 | 26.5 | 30.6 | 29.3 | 28.1 |  |
| Placer | 4.8 | 1.2 | 1.3 | 1.2 | 1.2 |  |
| Plumas | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Riverside | 49.1 | 12.0 | 12.6 | 12.2 | 12.4 |  |
| Sacramento | 17.0 | 4.6 | 4.3 | 4.1 | 4.0 |  |
| San Benito | 1.6 | 0.3 | 0.4 | 0.4 | 0.4 |  |
| San Bernardino | 25.7 | 6.1 | 6.7 | 6.8 | 6.1 |  |
| San Diego | 77.1 | 17.4 | 19.6 | 20.5 | 19.7 |  |
| San Joaquin | 34.0 | 6.3 | 7.5 | 9.9 | 10.2 |  |
| San Luis Obispo | 6.1 | 1.4 | 1.6 | 1.6 | 1.5 |  |
| San Mateo | 24.1 | 5.1 | 6.4 | 6.3 | 6.3 |  |
| Santa Barbara | 10.0 | 2.4 | 2.7 | 2.6 | 2.3 |  |
| Santa Clara | 36.6 | 9.6 | 9.2 | 8.9 | 9.0 |  |
| Santa Cruz | 5.2 | 1.4 | 1.6 | 1.6 | 0.6 |  |
| Shasta | 5.1 | 1.4 | 1.3 | 1.3 | 1.1 |  |
| Sierra | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Siskiyou | 0.4 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Solano | 10.7 | 2.8 | 2.7 | 2.5 | 2.6 |  |
| Sonoma | 11.9 | 2.6 | 3.1 | 3.2 | 3.0 |  |
| Stanislaus | 4.6 | 1.1 | 1.0 | 1.3 | 1.2 |  |
| Tehama | 1.1 | 0.3 | 0.3 | 0.3 | 0.3 |  |
| Tulare | 6.3 | 1.6 | 1.7 | 1.6 | 1.5 |  |
| Tuolumne | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Ventura | 22.0 | 5.3 | 5.6 | 5.7 | 5.3 |  |
| Yolo | 4.3 | 1.0 | 1.1 | 1.1 | 1.1 |  |
| Yuba | 4.8 | 1.1 | 1.2 | 1.3 | 1.2 |  |
| Total | 890.1 | 210.7 | 228.1 | 229.2 | 222.1 |  |

## Emissions from Livestock Wastes

As part of its current research and development for ammonia and PM2.5 inventories, the ARB is in the process of developing improved county-level population estimates for cattle and other livestock.  Contacts for the ARB research programs are Mr. Patrick Gaffney and Mr. Michael Benjamin.  In addition, the California Energy Commission (CEC) is responsible for developing comprehensive statewide inventories of greenhouse gas emissions, which includes methane emissions due to livestock manure management.  The contact for the CEC greenhouse inventory is Mr. Guido Franco.

The CEC and ARB are currently working on review draft reports of their emission inventories.  In addition, the CEC is working cooperatively with the ARB to ensure that the CEC’s final version of its emission inventory is reasonably consistent with ARB inventories.  At the time of this writing, the CEC and ARB reports are designated as drafts that cannot be cited or quoted.  In addition, they do not yet agree in all respects (although this issue will be remedied with the final version of the CEC’s inventory).  Thus in the future, it is recommended that counties adopt the CEC’s statewide emissions estimates of methane and apportion these estimates according to county-level cattle populations, which the ARB is developing for the ammonia inventory.

The ARB’s and CEC’s draft research reports present statewide cattle populations that differ by only 10-11% and are based on similar sources of data.  However, the draft results differ significantly in the assignment of subpopulations according to animal types.  The CEC draft report estimates that 63% of cattle are beef cattle and that 37% are dairy cattle.  The ARB draft report estimates that 43% of cattle are beef cattle and 57% are dairy cattle.  At the time of this writing, the ARB seems to have made further progress into adjustments to the cattle populations.  These adjustments were applied in consideration of various factors such as residency times in feedlots, rates of slaughter for market, and temporary inshipments of cattle for grazing in California.  ARB has not yet presented revised population figures for other types of livestock (such as swine and poultry).

The CEC’s current draft of the statewide methane emission inventory for manure management is summarized in Table 2.  The estimates were developed on the basis of guidance from the Emission Inventory Improvement Program (EIIP, 1999).  Table 2 also provides emissions that have been adjusted to account for the differences between the CEC’s and ARB’s interim population estimates.  For the time being, these estimates are recommended for use until the ARB and CEC research projects are complete.

Table 2.   Interim statewide methane emission estimates for manure management.

| Animal Type | CEC Draft Estimatea (tpy) | Population- adjusted Estimate (tpy) |
| --- | --- | --- |
| Beef cattle | 4,670 | 2,940 |
| Dairy cattle | 253,790 | 310,797 |
| Swine | 3,410 | 3,410 |
| Poultry | 7,670 | 7,670 |
| Sheep | 630 | 630 |
| Goats | 110 | 110 |
| Horses | 3,100 | 3,100 |
| Total | 273,380 | 328,657 |

aCEC’s methane inventory is expressed in terms of thousand metric tons of carbon dioxide equivalents, which accounts for methane’s relative effectiveness as a greenhouse gas to carbon dioxide. The “global warming potential” of methane is 21. (See EIIP, 1999b for further definition.)  The unit conversion from the CEC’s units of measure to thousand tons of methane is as follows:

CH4 equivalent/21 CO2 equivalents   1.1025 ton/metric ton

EPA (1999a) recommends a TOC speciation profile for “solid waste – animal waste decomposition” as follows: 70% methane, 20% ethane, and 10% VOC.  Total statewide TOG and VOC emissions are estimated as follows.

328,657 tons CH4 / 0.70 = 469,510 tons TOG

469,510 tons TOG  0.10 = 46,950 tons VOC

Table 3 tabulates the results of similar calculations for all animal types.

Table 3.  Interim statewide TOG and VOC emission estimates for manure management.

| Animal Type | TOG (tpy) | VOC (tpy) |
| --- | --- | --- |
| Beef cattle | 4,200 | 420 |
| Dairy cattle | 443,996 | 44,400 |
| Swine | 4,871 | 487 |
| Poultry | 10,957 | 1,096 |
| Sheep | 900 | 90 |
| Goats | 157 | 16 |
| Horses | 4,429 | 443 |
| Total | 469,510 | 46,950 |

Emissions are apportioned to counties according to the county-level animal populations by type.  ARB’s interim county-level animal populations were used to disaggregate emissions for cattle.  A County Agricultural Statistics Service (1991) publication was consulted for other animal types.  Emissions for goats were disaggregated according to the population patterns for sheep.  Table 4 provides disaggregated county-level emissions of VOC and TOG for manure management.  For counties that are split between air basins, emissions were further disaggregated according to ARB’s spatial surrogate for agricultural land.

| Table 4. Interim annual TOG and VOC emissions for manure management. |  |  |  |
| --- | --- | --- | --- |
| County | Total TOG (tpy) | Total VOC (tpy) |  |
| Alameda | 122 | 12 |  |
| Alpine | 13 | 1 |  |
| Amador | 52 | 5 |  |
| Butte | 547 | 55 |  |
| Calaveras | 93 | 9 |  |
| Colusa | 153 | 15 |  |
| Contra Costa | 997 | 100 |  |
| Del Norte | 985 | 98 |  |
| El Dorado | 96 | 10 |  |
| Fresno | 25,958 | 2,596 |  |
| Glenn | 4,971 | 497 |  |
| Humboldt | 5,337 | 534 |  |
| Imperial | 613 | 61 |  |
| Inyo | 71 | 7 |  |
| Kern | 13,572 | 1,357 |  |
| Kings | 35,061 | 3,506 |  |
| Lake | 34 | 3 |  |
| Lassen | 280 | 28 |  |
| Los Angeles | 1,582 | 158 |  |
| Madera | 8,583 | 858 |  |
| Marin | 4,407 | 441 |  |
| Mariposa | 75 | 8 |  |
| Mendocino | 705 | 71 |  |
| Merced | 58,173 | 5,817 |  |
| Modoc | 244 | 24 |  |
| Mono | 37 | 4 |  |
| Monterey | 1,351 | 135 |  |
| Napa | 156 | 16 |  |
| Nevada | 50 | 5 |  |
| Orange | 128 | 13 |  |
| Placer | 389 | 39 |  |
| Plumas | 62 | 6 |  |
| Riverside | 39,664 | 3,966 |  |
| Sacramento | 5,737 | 574 |  |
| San Benito | 459 | 46 |  |
| San Bernardino | 55,160 | 5,516 |  |
| San Diego | 2,921 | 292 |  |
| San Francisco | 10 | 1 |  |
| San Joaquin | 29,306 | 2,931 |  |
| San Luis Obispo | 486 | 49 |  |
| San Mateo | 93 | 9 |  |
| Santa Barbara | 1,271 | 127 |  |
| Santa Clara | 731 | 73 |  |
| Santa Cruz | 144 | 14 |  |
| Shasta | 251 | 25 |  |
| Sierra | 159 | 16 |  |
| Siskiyou | 623 | 62 |  |
| Solano | 700 | 70 |  |
| Sonoma | 4,221 | 422 |  |
| Stanislaus | 52,955 | 5,295 |  |
| Sutter | 243 | 24 |  |
| Tehama | 1,685 | 169 |  |
| Trinity | 6,054 | 605 |  |
| Tulare | 100,088 | 10,009 |  |
| Tuolumne | 72 | 7 |  |
| Ventura | 227 | 23 |  |
| Yolo | 301 | 30 |  |
| Yuba | 1,050 | 105 |  |
| Total | 469,510 | 46,951 |  |

## Municipal Landfills

Municipal landfills may receive a variety of waste streams, including industrial solid waste, wastewater treatment sludge, and municipal solid waste (MSW).  Uncontrolled emissions from municipal landfills include TOG (which is almost entirely methane) and trace amounts of ROG.  EPA (1999a) reports the following hydrocarbon speciation profile for Class II solid waste landfills: 98.7% methane, 0.1% ethane, and 1.2% VOCs.  If the landfill gas is recovered and recycled or combusted for pollutant control, combustion by-products also will be emitted (PM, PM10, SOx, NOx, and CO).  Emissions of combustion by-products are discussed in Attachment J, Resource Recovery.

The U.S. EPA and the EIIP recommend the Landfill Air Emissions Estimation Model to estimate emissions (LandGEM [EPA 1999b, 1999c]).  At minimum, LandGEM requires the average rate of waste input to the landfill (Mg/yr), the time period since landfill closure (if any), and the time period since landfill opening.  LandGEM uses the following simple equation, which can easily be reproduced in a spreadsheet or database.

QCH4 = L0 R (ekc  ekt)

Q	=	Methane generation rate at time t, m3 CH4/yr

L0	=	Methane generation potential, m3 CH4 /Mg refuse, 100 m3/Mg

R	=	Average annual refuse acceptance rate during active life, Mg/yr

k	=	Methane generation rate constant, 0.02/yr

c	=	Time since landfill closure, years (c = 0 for active landfills)

t	=	Time since the initial refuse placement, years

The IWMB (2001c) maintains detailed records of the quantities of wastes accepted annually at California landfills since 1990.  For these relatively new wastes, the disposal data are input to the LandGEM equation to estimate emissions.  However, according to the LandGEM equation, the methane emission rate deteriorates slowly over time.  Waste that has been landfilled for 10 years emits at a rate that is more than 50% of its initial emission rate.  Therefore, it also is necessary to estimate the emissions for wastes that were landfilled prior to 1990.

The IWMB (2001d) maintains a comprehensive database of all landfills, called the Solid Waste Information System (SWIS) Database.  SWIS lists 52 landfills and solid waste disposal sites within the CCOS II participating counties that have accepted municipal solid waste, have had a regulatory status (i.e., operated since the early 1970s), have not been “clean closed” (i.e., closed without waste in place), or have been confirmed to exist by contacts at local Departments of Health.  Most facilities that operated prior to the enactment of regulations in the early 1970s were “burn dumps,” where wastes were combusted and not left in place.

Unfortunately, many records in the IWMB database do not have the minimum information needed to use the LandGEM equation.  In order to estimate emissions for landfilled wastes that are older than 10 years, the following assumptions will be employed.

- If the actual mass or volume throughput is unavailable in the IWMB database, then the permitted throughput (if available) will be used to estimate emissions.

- Local enforcement agencies (mostly county health departments) were contacted to determine throughputs, dates of closures, and dates of openings.  For those counties that did not respond to inquiries and for which dates are not available in the IWMB database, it will be conservatively assumed that landfills opened in 1970 and remain open today.  Unless an earlier date of closure is known, an end year of 1989 will be input to the LandGEM equation in order to model emissions from wastes that were deposited from 1970 through 1989.

- The EIIP (1997) recommends that a density of 1,160 pounds per cubic yard of waste should be assumed if site-specific information is not available.

Adjustments must also be made to account for recovered landfill gases, which may be flared or used as an energy source.  Four landfills were determined to have gas-to-energy recovery systems (see Attachment J):  the Sacramento City Landfill, the Sacramento County Landfill (Kiefer), the Yolo County Central Landfill, and the Union Mine Disposal Site in El Dorado County.  For these facilities, it was determined that 85% of landfill methane gases are recovered and combusted with an efficiency of 98%.  Thus, methane, VOC, and TOC emissions for these three facilities were reduced by roughly 83%.  In addition, the quantities of flared landfill gases were estimated for each county of California (see Attachment J).  These quantities cannot be associated with individual landfills, but instead are presented as countywide emissions corrections (Table 5), which are applied in the final step of emissions calculations.

| Table 5. Corrections to county emissions for landfill gas recovery flares (tons/yr). |  |  |  |  |
| --- | --- | --- | --- | --- |
| County | CH4 | TOG | VOC |  |
| Amador | -328 | -332 | -4 |  |
| Butte | -2,106 | -2,133 | -26 |  |
| Calaveras | -380 | -385 | -5 |  |
| Colusa | -73 | -74 | -1 |  |
| El Dorado | 0 | 0 | 0 |  |
| Glenn | -274 | -278 | -3 |  |
| Mariposa | -148 | -150 | -2 |  |
| Mendocino | -681 | -689 | -8 |  |
| Nevada | -115 | -116 | -1 |  |
| Placer | -224 | -226 | -3 |  |
| Plumas | -90 | -92 | -1 |  |
| Sacramento | -1,381 | -1,399 | -17 |  |
| Shasta | -2,634 | -2,669 | -32 |  |
| Sierra | -34 | -34 | 0 |  |
| Solano | -5,096 | -5,163 | -62 |  |
| Tehama | -501 | -508 | -6 |  |
| Tuolumne | -163 | -166 | -2 |  |
| Yolo | -132 | -134 | -2 |  |
| Yuba | -1,876 | -1,901 | -23 |  |

### Example Calculation

Below is an example calculation for the Amador County Sanitary Landfill (IWMB No. 03-AA-0001).  The local enforcement agency in Amador County was contacted to gather information for this landfill.  It opened in 1972 and still is currently accepting waste.  It is reported to have accepted waste over its history at rates of 11 to 90 tons per day (midpoint = 51 tons per day, or 18,615 tons per year).  From 1990 through 2000, this landfill accepted the annual tonnages of waste that are reported in Table 6.  The year-2000 methane emission rate is estimated as follows for wastes that were deposited in 1995.

100 m3/Mg  (19,614 tons/yr  Mg/1.1025 ton)  (e-0.02(2000-1995) – e-0.02(2000-1994))

 1000 L/m3  mol/22.4 L  16 g/mol  10-6 Mg/g  1.1025 ton/Mg = 25 tons CH4/yr

The sum of emissions from wastes that were deposited from 1990 through 2000 at the Amador County Sanitary Landfill is 356 tons CH4/yr (see Table 6).

Table 6.  Waste accepted from 1972-2000 and associated emissions for the

Amador County Sanitary Landfill.

| Year(s) | Total Tons | Year-2000 Emissions (ton) |  |  |
| --- | --- | --- | --- | --- |
| Deposited | Accepted | CH4 | TOG | VOC |
| 2000 | 39,339 | 56 | 56 | 0.68 |
| 1999 | 2,708 | 4 | 4 | 0.05 |
| 1998 | 11,653 | 16 | 16 | 0.19 |
| 1997 | 10,617 | 14 | 14 | 0.17 |
| 1996 | 19,271 | 25 | 25 | 0.31 |
| 1994 | 19,939 | 25 | 25 | 0.31 |
| 1995 | 19,614 | 25 | 25 | 0.30 |
| 1993 | 25,022 | 31 | 31 | 0.37 |
| 1991 | 40,192 | 51 | 52 | 0.63 |
| 1992 | 42,714 | 47 | 48 | 0.58 |
| 1990 | 52,951 | 61 | 62 | 0.75 |
| Subtotal | 284,020 | 356 | 360 | 4.3 |
| 1972 through 1989 | 335,070 | 308 | 312 | 3.7 |
| Grand Total | 619,090 | 663 | 672 | 8 |

For wastes that were deposited from 1972 through 1989, emissions are estimated for the Amador County Sanitary Landfill as follows.

100 m3/Mg  (18,615 tons/yr  Mg/1.1025 ton)  (e-0.02(2000-1989) – e-0.02(2000-1972))

 1000 L/m3  mol/22.4 L  16 g/mol  10-6 Mg/g  1.1025 ton/Mg = 308 tons CH4/yr

The sum total of emissions from all wastes that were deposited from 1970 through 2000 at the Amador County Sanitary Landfill is 663 tons CH4/yr (see Table 7).  Emissions of TOG and VOC are estimated according to the EPA’s recommended speciation profile.

663 tons CH4/yr 0.987 = 672 tons TOG/yr

672 tons TOG/yr  0.012 = 8.1 tons VOC/yr

Tables 8 and 9 tabulate emissions for all landfills within the CCOS II participating counties for deposits made during the 1990-2000 and 1970-1989 periods, respectively.  Table 10 presents the sum totals of emissions for each landfill.  Table 11 presents the net totals of emissions for each county, which account for flared landfill gases (see Table 5).

In conducting this analysis, we noted that some counties with small populations appear to have relatively large emissions from this source category.  This is because we quantified relatively large quantities of waste deposits in the landfills for these small counties and relatively large corrections for controls for some of the larger counties that have LFGTE facilities.  For example, in the CCOS II area alone, about 40% of wastes were quantified to be deposited in Sacramento County landfills; 15% in Solano County; 8% in Shasta and Yolo Counties; and 6% to 7% each in Placer, Yuba, and Butte Counties.  Sacramento and Yolo Counties both have facilities with LFGTE, and therefore have fairly large emissions reductions applied to their inventories.  Thus, only about 12% and 4% of CCOS II total TOG emissions are attributable to Sacramento and Yolo County landfills; whereas 16% to 21% are attributable to Solano, Yuba, and Plumas Counties.

| Table 8. Waste accepted from 1990-2000 and associated annual emissions for landfills in CCOS II participating counties. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IWMB No. | County | Estimated Total Waste Accepted (Mg) | CH4 (tpy) | TOC (tpy) | VOC (tpy) |  |
| 03-AA-0001 | Amador | 284,020 | 356 | 360 | 4.3 |  |
| 04-AA-0002 | Butte | 1,687,479 | 2,174 | 2,203 | 26.4 |  |
| 04-AA-0009 | Butte | 138,448 | 177 | 180 | 2.2 |  |
| 05-AA-0014 | Calaveras | 30,970 | 36 | 36 | 0.4 |  |
| 05-AA-0023 | Calaveras | 298,409 | 386 | 391 | 4.7 |  |
| 06-AA-0001 | Colusa | 62,990 | 75 | 76 | 0.9 |  |
| 09-AA-0003 | El Dorado | 427,382 | 88 | 89 | 1.1 |  |
| 11-AA-0001 | Glenn | 237,815 | 304 | 308 | 3.7 |  |
| 22-AA-0001 | Mariposa | 128,245 | 165 | 167 | 2.0 |  |
| 23-AA-0003 | Mendocino | 20,868 | 25 | 25 | 0.3 |  |
| 23-AA-0007 | Mendocino | 88,499 | 120 | 122 | 1.5 |  |
| 23-AA-0008 | Mendocino | 944 | 1.1 | 1.1 | 0.01 |  |
| 23-AA-0012 | Mendocino | 11,524 | 13 | 14 | 0.2 |  |
| 23-AA-0018 | Mendocino | 12,908 | 17 | 17 | 0.2 |  |
| 23-AA-0019 | Mendocino | 343,076 | 446 | 452 | 5.4 |  |
| 23-AA-0021 | Mendocino | 112,314 | 142 | 143 | 1.7 |  |
| 29-AA-0001 | Nevada | 99,461 | 117 | 119 | 1.4 |  |
| 31-AA-0210 | Placer | 1,990,356 | 2,554 | 2,588 | 31.1 |  |
| 31-AA-0560 | Placer | 193,826 | 235 | 238 | 2.9 |  |
| 32-AA-0007 | Plumas | 11,971 | 16 | 16 | 0.2 |  |
| 32-AA-0008 | Plumas | 40,205 | 48 | 49 | 0.6 |  |
| 32-AA-0009 | Plumas | 26,293 | 32 | 32 | 0.4 |  |
| 34-AA-0001 | Sacramento | 9,316,957 | 1,974 | 2,000 | 24.0 |  |
| 34-AA-0007 | Sacramento | 63,401 | 80 | 81 | 0.98 |  |
| 34-AA-0018 | Sacramento | 835,504 | 165 | 167 | 2.0 |  |
| 34-AA-0020 | Sacramento | 1,133,835 | 1,478 | 1,498 | 18.0 |  |
| 45-AA-0019 | Shasta | 13,552 | 16 | 16 | 0.2 |  |
| 45-AA-0020 | Shasta | 906,719 | 1,146 | 1,161 | 13.9 |  |
| 45-AA-0021 | Shasta | 10,092 | 12 | 12 | 0.1 |  |
| 45-AA-0022 | Shasta | 22,697 | 27 | 27 | 0.3 |  |
| 45-AA-0043 | Shasta | 1,222,861 | 1,577 | 1,597 | 19.2 |  |
| 45-AA-0058 | Shasta | 108,696 | 140 | 141 | 1.7 |  |
| 46-AA-0001 | Sierra | 29,207 | 37 | 38 | 0.5 |  |
| 48-AA-0002 | Solano | 1,247,735 | 1,616 | 1,637 | 19.6 |  |
| 48-AA-0004 | Solano | 11,890 | 14 | 14 | 0.2 |  |
| 48-AA-0075 | Solano | 3,159,813 | 4,102 | 4,156 | 49.9 |  |
| 52-AA-0001 | Tehama | 428,200 | 556 | 563 | 6.8 |  |
| 52-AA-0002 | Tehama | 5,635 | 7 | 7 | 0.08 |  |
| 52-AA-0009 | Tehama | 792 | 1.0 | 1.0 | 0.01 |  |
| 55-AA-0001 | Tuolumne | 12,435 | 15 | 15 | 0.2 |  |
| 55-AA-0002 | Tuolumne | 129,340 | 157 | 159 | 1.9 |  |
| 57-AA-0001 | Yolo | 1,979,768 | 417 | 423 | 5.1 |  |
| 57-AA-0004 | Yolo | 114,543 | 147 | 149 | 1.8 |  |
| 58-AA-0001 | Yuba | 26,921 | 32 | 33 | 0.4 |  |
| 58-AA-0002 | Yuba | 5,423 | 6 | 6 | 0.08 |  |
| 58-AA-0005 | Yuba | 759,993 | 933 | 945 | 11.3 |  |
| 58-AA-0006 | Yuba | 16,246 | 19 | 20 | 0.2 |  |
| 58-AA-0011 | Yuba | 818,287 | 1,119 | 1,133 | 13.6 |  |

| Table 9. Waste accepted from 1970-1989 and associated annual emissions for landfills in CCOS II participating counties. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IWMB No. | County | Estimated Year Open | Estimated End Year | Estimated Throughput (Mg/yr) | CH4 (tpy) | TOC (tpy) | VOC (tpy) |  |
| 03-AA-0001 | Amador | 1972 | 1989 | 16,888 | 308 | 312 | 3.7 |  |
| 04-AA-0002 | Butte | 1970 | 1989 | 198,677 | 3,969 | 4,022 | 48 |  |
| 05-AA-0014 | Calaveras | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 05-AA-0023 | Calaveras | 1990 | 1989 | 33,113 | 0 | 0 | 0 |  |
| 06-AA-0001 | Colusa | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 06-AA-0002 | Colusa | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 09-AA-0003 | El Dorado | 1970 | 1989 | 99,338 | 298 | 301 | 3.6 |  |
| 11-AA-0001 | Glenn | 1970 | 1989 | 21,192 | 423 | 429 | 5.1 |  |
| 22-AA-0001 | Mariposa | 1970 | 1989 | 15,563 | 311 | 315 | 3.8 |  |
| 23-AA-0003 | Mendocino | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 23-AA-0008 | Mendocino | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 23-AA-0018 | Mendocino | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 23-AA-0019 | Mendocino | 1970 | 1989 | 91,060 | 1,819 | 1,843 | 22 |  |
| 23-AA-0021 | Mendocino | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 29-AA-0001 | Nevada | 1970 | 1989 | 59,603 | 1,191 | 1,207 | 14 |  |
| 29-CR-0010 | Nevada | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 31-AA-0140 | Placer | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 31-AA-0210 | Placer | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 31-AA-0520 | Placer | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 31-AA-0540 | Placer | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 31-AA-0560 | Placer | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 31-AA-0625 | Placer | 1970 | 1989 | 39,735 | 794 | 804 | 10 |  |
| 32-AA-0007 | Plumas | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 32-AA-0008 | Plumas | 1970 | 1989 | 231,790 | 4,631 | 4,692 | 56 |  |
| 32-AA-0009 | Plumas | 1970 | 1989 | 231,790 | 4,631 | 4,692 | 56 |  |
| 34-AA-0001 | Sacramento | 1970 | 1989 | 1,477,493 | 4,880 | 4,944 | 59 |  |
| 34-AA-0004 | Sacramento | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 34-AA-0005 | Sacramento | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 34-AA-0018 | Sacramento | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 34-AC-0001 | Sacramento | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 45-AA-0014 | Shasta | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 45-AA-0019 | Shasta | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 45-AA-0020 | Shasta | 1977 | 1989 | 149,008 | 2,009 | 2,036 | 24 |  |
| 45-AA-0022 | Shasta | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 45-AA-0023 | Shasta | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 45-AA-0043 | Shasta | 1982 | 1989 | 208,611 | 1,722 | 1,745 | 21 |  |
| 46-AA-0001 | Sierra | 1970 | 1989 | 2,649 | 53 | 54 | 0.6 |  |
| 48-AA-0001 | Solano | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 48-AA-0002 | Solano | 1970 | 1989 | 155,630 | 3,109 | 3,150 | 38 |  |
| 48-AA-0004 | Solano | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 48-AA-0075 | Solano | 1970 | 1989 | 423,844 | 8,468 | 8,580 | 103 |  |
| 52-AA-0001 | Tehama | 1970 | 1989 | 45,696 | 913 | 925 | 11 |  |
| 55-AA-0001 | Tuolumne | 1970 | 1989 | 4,967 | 99 | 101 | 1.2 |  |
| 55-AA-0002 | Tuolumne | 1970 | 1989 | 30,795 | 615 | 623 | 7.5 |  |
| 55-AA-0005 | Tuolumne | 1970 | 1985 | 0 | 0 | 0 | 0 |  |
| 55-CR-0007 | Tuolumne | 1970 | 1975 | 0 | 0 | 0 | 0 |  |
| 57-AA-0001 | Yolo | 1970 | 1989 | 397,022 | 1,311 | 1,328 | 16 |  |
| 57-AA-0002 | Yolo | 1970 | 1989 | 9,603 | 192 | 194 | 2.3 |  |
| 57-AA-0004 | Yolo | 1970 | 1989 | 28,477 | 569 | 576 | 6.9 |  |
| 58-AA-0002 | Yuba | 1970 | 1989 | Insufficient Data |  |  |  |  |
| 58-AA-0005 | Yuba | 1970 | 1989 | 165,564 | 3,308 | 3,351 | 40 |  |
| 58-AA-0006 | Yuba | 1970 | 1989 | 0 | 0 | 0 | 0 |  |
| 58-AA-0011 | Yuba | 1970 | 1989 | 342,717 | 6,847 | 6,937 | 83 |  |

| Table 10. Total annual emissions (tons/yr) for each landfill in CCOS II participating counties. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IWMB No. | County | Facility Name | Latitude | Longitude | CH4 | TOC | VOC |
| 03-AA-0001 | Amador | Amador Co Slf/Buena Vista Class II LF | 38.29806 | -120.91200 | 663 | 672 | 8.1 |
| 04-AA-0002 | Butte | Neal Road Landfill | 39.67889 | -121.72500 | 6,144 | 6,225 | 75 |
| 04-AA-0009 | Butte | Louisiana-Pacific Landfill | 39.63615 | -121.58477 | 177 | 180 | 2.2 |
| 05-AA-0014 | Calaveras | Red Hill Sanitary Landfill | 38.07333 | -120.49167 | 36 | 36 | 0.44 |
| 05-AA-0023 | Calaveras | Rock Creek Landfill | 38.03861 | -120.84361 | 386 | 391 | 4.7 |
| 06-AA-0001 | Colusa | Evans Road Landfill | 39.03167 | -122.12333 | 75 | 76 | 0.91 |
| 06-AA-0002 | Colusa | Stonyford Disposal Site | 39.36167 | -122.54833 | 0 | 0 | 0 |
| 09-AA-0003 | El Dorado | Union Mine Disposal Site | 38.65333 | -120.82867 | 385 | 390 | 4.7 |
| 11-AA-0001 | Glenn | Glenn County Landfill Site | 39.63333 | -122.28806 | 727 | 737 | 8.8 |
| 22-AA-0001 | Mariposa | Mariposa County Sanitary Landfill | 37.50500 | -120.00306 | 476 | 482 | 5.8 |
| 23-AA-0003 | Mendocino | Caspar Refuse Disposal Site | 39.36118 | -123.78151 | 25 | 25 | 0.30 |
| 23-AA-0007 | Mendocino | Harwood Products Wood Waste Disposal Site | 39.70820 | -123.55952 | 120 | 122 | 1.5 |
| 23-AA-0008 | Mendocino | Laytonville Refuse Disposal Site | 39.64619 | -123.45473 | 1.1 | 1.1 | 0.01 |
| 23-AA-0012 | Mendocino | Covelo Fill Site "B" | 39.80000 | -123.25000 | 13 | 14 | 0.16 |
| 23-AA-0018 | Mendocino | South Coast Solid Waste Site | 38.83389 | -123.54389 | 17 | 17 | 0.20 |
| 23-AA-0019 | Mendocino | Ukiah Solid Waste Disposal Site | 39.16944 | -123.16417 | 2,265 | 2,295 | 28 |
| 23-AA-0021 | Mendocino | City Of Willits Disposal Site | 39.44265 | -123.35316 | 142 | 143 | 1.7 |
| 29-AA-0001 | Nevada | Mccourtney Landfill | 39.21667 | -121.06667 | 1,308 | 1,325 | 16 |
| 29-CR-0010 | Nevada | Nevada City Municipal Dump | 39.25000 | -121.01667 | Insufficient data |  |  |
| 31-AA-0140 | Placer | Loomis Sanitary Landfill | 38.83333 | -121.18333 | Insufficient data |  |  |
| 31-AA-0210 | Placer | Western Regional Landfill | 38.83583 | -121.34472 | 2,554 | 2,588 | 31 |
| 31-AA-0520 | Placer | Meadow Vista Sanitary Landfill | 39.00000 | -121.03333 | Insufficient data |  |  |
| 31-AA-0540 | Placer | Foresthill Sanitary Landfill | 39.01667 | -120.81667 | Insufficient data |  |  |
| 31-AA-0560 | Placer | Eastern Regional Landfill | 39.28563 | -120.21905 | 235 | 238 | 2.9 |
| 31-AA-0625 | Placer | Eastern Regional Mrf | 39.16667 | -120.21667 | 794 | 804 | 10 |
| 32-AA-0007 | Plumas | City Of Portola Landfill | 39.95306 | -121.03389 | 16 | 16 | 0.19 |
| 32-AA-0008 | Plumas | Gopher Hill Sanitary Landfill | 39.95333 | -121.03333 | 4,679 | 4,741 | 57 |
| 32-AA-0009 | Plumas | Chester Sanitary Landfill | 40.32667 | -121.13833 | 4,663 | 4,724 | 57 |
| 34-AA-0001 | Sacramento | Sacramento County Landfill (Kiefer) | 38.51667 | -121.18667 | 6,854 | 6,944 | 83 |
| 34-AA-0004 | Sacramento | Elk Grove Disposal Site | 38.41667 | -121.36667 | Insufficient data |  |  |
| 34-AA-0005 | Sacramento | Grand Island Disposal Site | 38.15000 | -121.60000 | Insufficient data |  |  |
| 34-AA-0007 | Sacramento | Dixon Pit Landfill | 38.43333 | -121.37028 | 80 | 81 | 1.0 |
| 34-AA-0018 | Sacramento | Sacramento City Landfill | 38.58583 | -121.46389 | 165 | 167 | 2.0 |
| 34-AA-0020 | Sacramento | L & D Landfill Co | 38.52639 | -121.38361 | 1,478 | 1,498 | 18 |
| 34-AC-0001 | Sacramento | City Of Folsom Corporation Yard | 38.68333 | -121.18333 | Insufficient data |  |  |
| 45-AA-0014 | Shasta | Buckeye Disposal Site | 40.67000 | -122.39667 | Insufficient data |  |  |
| 45-AA-0019 | Shasta | City Of Redding Benton Landfill | 40.57167 | -122.41000 | 16 | 16 | 0.19 |
| 45-AA-0020 | Shasta | Anderson Landfill | 40.41639 | -122.36000 | 3,155 | 3,197 | 38 |
| 45-AA-0021 | Shasta | Simpson Paper Co.,Dersch Road Landfill | 40.47833 | -122.21667 | 12 | 12 | 0.14 |
| 45-AA-0022 | Shasta | Intermountain Landfill | 40.93584 | -121.66913 | 27 | 27 | 0.32 |
| 45-AA-0023 | Shasta | Aubrey Ridge Landfill | 40.88167 | -121.71333 | Insufficient data |  |  |
| 45-AA-0043 | Shasta | West Central Landfill | 40.48778 | -122.53639 | 3,299 | 3,342 | 40 |
| 45-AA-0058 | Shasta | Twin Bridges Landfill | 40.49722 | -122.19694 | 140 | 141 | 1.7 |
| 46-AA-0001 | Sierra | Loyalton Landfill | 39.67056 | -120.22000 | 90 | 92 | 1.1 |
| 48-AA-0001 | Solano | Solano Garbage Company | 38.22694 | -121.97833 | 0 | 0 | 0 |
| 48-AA-0002 | Solano | B & J Dropbox Sanitary Landfill | 38.31361 | -121.83722 | 4,725 | 4,788 | 57 |
| 48-AA-0004 | Solano | Rio Vista Sanitary Landfill | 38.17750 | -121.68944 | 14 | 14 | 0.2 |
| 48-AA-0075 | Solano | Potrero Hills Landfill | 38.22333 | -121.97778 | 12,570 | 12,736 | 153 |
| 52-AA-0001 | Tehama | Red Bluff Sanitary Landfill | 40.20972 | -122.29306 | 1,469 | 1,488 | 18 |
| 52-AA-0002 | Tehama | Louisiana-Pacific Disposal Site | 40.18658 | -122.24421 | 6.6 | 6.7 | 0.08 |
| 52-AA-0009 | Tehama | Diamond Landfill | 40.12167 | -122.38167 | 1.0 | 1.0 | 0.01 |
| 55-AA-0001 | Tuolumne | Big Oak Flat Landfill | 37.80750 | -120.22528 | 114 | 116 | 1.4 |
| 55-AA-0002 | Tuolumne | Tuolumne County Central Sanitary Lf | 37.94833 | -120.40167 | 773 | 783 | 9.4 |
| 55-AA-0005 | Tuolumne | Sierra Conservation Center | 37.88958 | -120.53390 | 0 | 0 | 0 |
| 55-CR-0007 | Tuolumne | Columbia County Dump | 38.03333 | -120.40000 | 0 | 0 | 0 |
| 57-AA-0001 | Yolo | Yolo County Central Landfill | 38.59028 | -121.69222 | 1,729 | 1,751 | 21 |
| 57-AA-0002 | Yolo | Esparto Convenience Center | 38.70000 | -122.01667 | 192 | 194 | 2.3 |
| 57-AA-0004 | Yolo | Univ Of Calif Davis Sanitary Landfill | 38.52972 | -121.80556 | 716 | 726 | 8.7 |
| 58-AA-0001 | Yuba | Beale Afb Sanitary Landfill | 39.10000 | -121.38000 | 32 | 33 | 0 |
| 58-AA-0002 | Yuba | Ponderosa Sanitary Landfill | 39.46667 | -121.29000 | 6.3 | 6.4 | 0.08 |
| 58-AA-0005 | Yuba | Yuba-Sutter Disposal Inc | 39.16667 | -121.55500 | 4,241 | 4,296 | 52 |
| 58-AA-0006 | Yuba | Yuba-Sutter Disposal Area | 39.16333 | -121.55833 | 19 | 20 | 0 |
| 58-AA-0011 | Yuba | Ostrom Road Landfill | 39.07306 | -121.39639 | 7,966 | 8,071 | 97 |

Table 11.   Net annual landfill emissions (tpy) by county.

| County | CH4 | TOG | VOC |
| --- | --- | --- | --- |
| Amador | 336 | 340 | 4.1 |
| Butte | 4,216 | 4,271 | 51 |
| Calaveras | 42 | 42 | 0.5 |
| Colusa | 2.5 | 2.5 | 0.03 |
| El Dorado | 385 | 390 | 4.7 |
| Glenn | 453 | 459 | 5.5 |
| Mariposa | 328 | 332 | 4.0 |
| Mendocino | 1,902 | 1,927 | 23 |
| Nevada | 1,193 | 1,209 | 15 |
| Placer | 3,359 | 3,404 | 41 |
| Plumas | 9,267 | 9,389 | 113 |
| Sacramento | 7,197 | 7,292 | 87 |
| Shasta | 4,013 | 4,066 | 49 |
| Sierra | 57 | 57 | 0.69 |
| Solano | 12,214 | 12,374 | 148 |
| Tehama | 975 | 988 | 12 |
| Tuolumne | 724 | 733 | 8.8 |
| Yolo | 2,505 | 2,538 | 30 |
| Yuba | 10,388 | 10,525 | 126 |

## References

AEA Technology Environment (2002) Atmospheric emissions from small carcass incinerators.  Report prepared by AEA Technology Environment, Abingdon, Oxfordshire, England for the Department for Environment, Food and Rural Affairs of the United Kingdom, July.

ARB (1999) 1998 architectural coatings survey results. Final report prepared by the California Air Resources Board, September.

California Agricultural Statistics Service (1991) California livestock statistics bulletin, 1990.  Report published by the California Agricultural Statistics Service, June.

CANA (2000) Emissions testing results.  Web page data summary prepared by the Cremation Association of North America, Chicago, Illinois, available for download from http://www.cremationassociation.org/html/environment.html.

CANA (2001) 2000 Data and projections to the year 2025.  Summary table published by the Cremation Association of North America, Chicago, Illinois, October 29.

Eastern Research Group (2001) Documentation for the 1996 base year national toxics inventory for area sources.  Report prepared for the US Environmental Protection Agency, Research Triangle Park, NC, by Eastern Research Group, Morrisville, NC, May.

Emission Inventory Improvement Program (2001) Uncontrolled emission factor listing for criteria air pollutants.  From the EIIP guidance document series, volume II  estimating point source emissions, chapter 14.  Report no. EPA-68-D-0160 prepared for the Emission Inventory Improvement Program and the US Environmental Protection Agency, Research Triangle Park, NC, by Eastern Research Group, Morrisville, NC, July.

Emission Inventory Improvement Program (1999a) Methods for estimating greenhouse gas emissions from manure management.  From the EIIP guidance document series, volume VII  estimating greenhouse gas emissions, chapter 7.  Report no. EPA-68-D-0160 prepared for the Emission Inventory Improvement Program and the US Environmental Protection Agency, Research Triangle Park, NC, by ICF Consulting, Washington D.C., October.

Emission Inventory Improvement Program (1999b) Introduction to estimating greenhouse gas emissions.  From the EIIP guidance document series, volume VII  estimating greenhouse gas emissions.  Report no. EPA-68-D-0160 prepared for the Emission Inventory Improvement Program and the US Environmental Protection Agency, Research Triangle Park, NC, by ICF Consulting, Washington D.C., October.

Emission Inventory Improvement Program (1997) Methods for estimating greenhouse gas emissions from municipal waste disposal.  From the EIIP guidance document series, volume VII  estimating greenhouse gas emissions, chapter 7.  Report no. EPA-68-D-0160 prepared for the Emission Inventory Improvement Program and the US Environmental Protection Agency, Research Triangle Park, NC, by ICF Consulting, Washington D.C., October.

EPA (1999a) Volatile organic compound/particulate matter speciation data base management system (SPECIATE), version 3.1 (for microcomputers).  Prepared by the Office of Air Quality Planning and Standards, U.S. Environmental Protection Agency, Research Triangle Park, NC October.

EPA (1999b) Landfill gas emissions model (LandGEM), version 2.0.  Software and user’s manual prepared by Eastern Research Group, Inc., Morrisville, NC and Radian Corp., Research Triangle Park, NC. for the Air Pollution Prevention and Control Division of the Environmental Protection Agency, Research Triangle Park, NC.

EPA (1999c). Municipal solid waste landfills.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Supplement E (September 1999), Chapter 2.4.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

EPA (1995a) Medical waste incineration. In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Chapter 13.2.4.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

IWMB (2001a) Solid waste characterization database.  Database maintained by the California Integrated Waste Management Board, Sacramento, and accessed on-line November, 2001 from Internet site http://www.ciwmb.ca.gov/WasteChar/JurisSel.asp.

IWMB (2001b) Percentage of waste exported to total waste disposed for the year 2000 by city and county.  Meeting handout from the August 14-15, 2001 Board meeting for Agenda Item No. 11:  Discussion of and request for direction on proposed change to point of collection of integrated waste management fee for waste exported out of state.  Board meeting materials available on-line at http://www.ciwmb.ca.gov/Agendas/Default.asp.

IWMB (2001c) Landfill tonnage reports. Database maintained by the California Integrated Waste Management Board, Sacramento, and accessed on-line November, 2001 from Internet site http://www.ciwmb.ca.gov/Landfills/Tonnage/default.htm.

IWMB (2001d) Solid waste information system (SWIS) database. Database maintained by the California Integrated Waste Management Board, Sacramento, and accessed on-line November, 2001 from Internet site http://www.ciwmb.ca.gov/SWIS/.