# Attachment C:  Asphalt paving and Roofing

This attachment discusses techniques to estimate emissions from asphalt paving and roofing.  The specific emission inventory categories covered are listed below.  (Note this source category does not include emissions from asphalt plants, which are usually considered a point source.)

Description	EIC Code	CES No.

Cutback asphalt paving	540-560-0400-0000 	46870

Road oils	540-562-0400-0000 	46888

Hot-mix asphalt paving	540-564-0400-0000 	46896

Emulsified asphalt 	540-566-0400-0000 	46904

Asphalt paving (unspecified)	540-995-0400-0000 	82057

Asphalt roofing	540-590-0400-0000	66720

Asphalt and road oils are used to pave, seal, and repair surfaces such as roads, parking lots, drives, walkways, and airport runways.  Asphalt is also used to adhere, repair, and create a smooth finish over manufactured roofing materials (such as asphalt-impregnated or asphalt-coated felt sheets, cap sheets, and flashing squares) on multiple unit dwellings, commercial, and industrial buildings.  Asphalt concrete is a mixture of asphalt cement, comprised of a binder and an aggregate. Asphalt cement is the semi-solid residual material left from petroleum refining after the lighter and more volatile fractions have been distilled out.  The most commonly used forms of asphalt are hot-mix, cutback, and emulsified. Hot-mix asphalt is a mixture of heated asphalt cement and aggregate. Asphalt cutbacks are asphalt cements thinned with petroleum distillates (diluents). Asphalt emulsions are mixtures of asphalt cement with water and emulsifiers.  Each is discussed below.

Hot-Mix Asphalt:  Hot-mix asphalt is the most commonly used paving asphalt for road surfaces 2 to 6 inches thick. Hot-mix asphalt is prepared at a hot-mix asphalt plant by heating asphalt cement before adding the aggregate.

Cutback Asphalt represents about three percent of sales of all asphalt cement types and is used in tack and seal operations, in priming roadbeds for hot-mix application, and for paving operations for pavements up to several inches thick. In preparing cutback asphalt, asphalt cement is blended or “cut back” at the plant with a diluent, typically from 25 to 45 percent by volume of petroleum distillates, depending on the desired viscosity. Because cutback asphalt has a high emission potential, many counties have regulated its use (see table, attached). There are three types of cutback asphalt cement:  (1) Rapid Cure which uses gasoline or naphthas as diluents; (2) Medium Cure which uses kerosene as a diluent; and (3) Slow Cure which uses low volatility fuel solvents as diluents.

Emulsified asphalt represents about seven percent of overall use of all asphalt types and is used in most of the same applications as cutback asphalts but is a lower-emitting, energy-saving, and safer alternative to the cutback asphalts (Moulthrop, et al. 1997). Instead of blending asphalt cement with petroleum distillates, emulsified asphalts use a blend of asphalt cement, water and an emulsifying agent, such as soap. Such blends typically contain one-third water, two-thirds asphalt cement and minor amounts of an emulsifier. Some emulsified asphalts may contain up to 12 percent organic solvents by volume (California SCAQMD Rule 1108.1).

Road oils are liquefied, slow-cure asphalts. They are used to suppress dust, bind gravel, preserve roadways from weathering, and create road-paving mixtures (such as cold patch materials).

##### ESTIMATION METHODS

Emissions from asphalt paving and roofing operations can be estimated by using bottom-up procedures based on locally compiled activity data or by using a top-down approach based on disaggregated statewide or national emission estimates. County-specific activity data can be gathered from state and local Departments of Transportation (DOTs), paving companies (including volumes of materials used and diluent content), and roofing contractors for use in the bottom-up calculation-approach or asphalt usage information may be gathered at the state or national level and disaggregated to individual counties by using a spatially resolved surrogate (e.g., miles of paved roads or number of houses, etc.).  Procedures to complete either approach are presented below.  However, since no surveys were conducted as part of this study, the emission estimates reported here are based on the top-down approach.  If more accurate emissions estimates are desired, local data collection efforts are needed.

### Local Data Collection

The majority of asphalt use is performed or directed by state and local DOTs and roofing contractors.  Most counties have only a handful of local DOTs (which account for the majority of asphalt usage), and some counties also have a manageable number of roofing contractors.  If DOTs and roofing contractors maintain complete records, then activity data is fairly centralized, which facilitates data collection at the county level.  If counties elect to survey local DOTs and roofers, the goals should be (1) to determine the suppliers, types, quantities, VOC contents, and timings of applications of asphalt paving or roofing materials and (2) to follow up with suppliers as a reality check or to fill in missing information.  Contact information for local DOTs and roofing contractors is provided in the attachments. Key survey questions are listed below.

Of the types of asphalt listed below, what asphalts are used by the DOT?  Who are the suppliers of each type (contact information)?  What quantity of each type was used in 2000 (in terms of barrels, pounds, tons, or cubic yards)?  What is the VOC (or diluent) content of each (in terms of volume percent or weight percent)?  What is the density of each type of asphalt (in terms of lb/gal or kg/L)? What is the density of any diluent used (in terms of lb/gal or kg/L)?  During which months is each type applied?

- Cutback asphalt (total)

- Cutback asphalt (rapid cure)

- Cutback asphalt (medium cure)

- Cutback asphalt (slow cure)

- Emulsified asphalt (total)

- Emulsified asphalt (rapid setting)

- Emulsified asphalt (medium setting)

- Emulsified asphalt (slow setting)

- Hot-mix asphalt

- Other asphalt

- Road oils or “tack”

Of the types of roofing materials listed below, what types are used by the contractor and how were they adhered (with hot or cold asphalt) and finished (with smooth asphalt, gravel, or other)? Who are the suppliers or manufacturers of each type (contact information and product ID)?  What quantities of hot- and cold-mix asphalt were used for roof installation, repairs, and finishing in 2000 (in terms of barrels, pounds, tons, or cubic yards)?  Alternately, what are the typical quantities used of hot- and cold-mix asphalt used per square yard or per square of each type of roofing material? What are the VOC (or diluent) contents of the hot- and cold-mix asphalts (in terms of volume percent or weight percent)?  What is the density of each type of asphalt (in terms of lb/gal or kg/L)? What is the density of any diluent used (in terms of lb/gal or kg/L)?  During which months where asphalts used or what are the contractor’s busiest seasons?

- Felt sheet squares

- Cap sheet squares

- Flashing squares

- Bitumen sheets

- Other

If the survey is extended to local asphalt suppliers, this will help to improve the accuracy, quality, and confidence in the estimated emissions.  Questions should be posed to verify the responses provided by local DOTs or their contractors, and to identify any other important asphalt consumers that might need to be contacted.

Of the types of asphalt, what asphalts are sold by your enterprise? What quantity of each type was sold in 2000 (in terms of barrels, pounds, tons, or cubic yards)?  What is the VOC (or diluent) content of each (in terms of volume percent or weight percent)? What is the density of each type of asphalt (in terms of lb/gal or kg/L)? What is the density of any diluent used (in terms of lb/gal or kg/L)?  During which months was each type sold?  Other than the DOT, do you know of any other large-scale consumers of asphalt? What is the split in your asphalt sales between DOTs, roofers, and other types of clients (roughly speaking, if necessary)?

It is more difficult to survey consumers of paving materials for private roads and parking lots, military roads, Bureau of Land Management roads, Forest Service roads, and/or roads on tribal lands.  However, in most cases these activities are likely to be negligible in comparison to maintenance and construction of public roads. The EIIP (EIIP, 2001) suggests that these emissions may be safely omitted from most inventories.  However, they may be somewhat important considerations for the inventories of Yuba County (Beale Air Force Base), Shasta County (Lassen National Park, Whiskeytn-Shasta-Trinity National Recreation Areas), and other counties that have large federally managed areas.

Roofing and paving activities for National Parks may be obtained from each park’s facility manager.  Contact information for individual parks can be obtained via the internet at  http://www.nps.gov.  Once at that web site, click on “visit your parks”, then select “map search”, then select California, and select the desired park from the state map.  Each park’s homepage should provide the mailing and email address and phone number for the point of contact.

Roofing and paving activities at other federally managed lands may be obtained via freedom of information act (FOIA) requests.  At Beale AFB for example, information may be requested from the “records officer” at (530) 634-2964.

The Emission Inventory Improvement Program (EIIP) provides guidance for the selection of default densities of paving asphalts (EIIP, 2001).  For cutback diluents, 0.7 kg/L for rapid cure, 0.8 kg/L for medium cure, and 0.9 kg/L for slow cure are recommended default values. Also, the density of emulsified asphalt is estimated to be are similar to that of water (1.0 to 1.1 kg/L).

### Top-Down Activity Data Collection – Asphalt Paving

Mendocino County undertook a survey of DOTs and asphalt vendors in 1997.  Several other California counties performed surveys of asphalt suppliers between 1987 and 1991.  From the Mendocino County survey, it was estimated that local DOTs and Caltrans used the tonnages of paving asphalts listed below (Wolbach, 2001).  Suppliers in Mendocino County estimated that about 40 percent of their sales were attributable to public entities such as DOTs, thus total asphalt used in Mendocino County in 1997 for paving is likely between 62,000 tons (sum of materials used by public entities) to 150,000 tons (sum of estimated public entity use plus private use).  Asphalt use was divided into the following types:

- Cutback asphalt (medium cure): 150 tons

- Emulsified asphalt (medium set): 8,600 tons

- Hot-mix asphalt: 51,000 tons

- Other asphalt: 2,000 tons

- Road oils or “tack” - unknown

Mendocino County has approximately 9,000 miles of paved public roadways, a population of around 93,000 (U.S. Census Bureau, 2001), and 1997 employment for highway and street construction workers (SIC 1610) of around 160 employees.  Nationwide, 1995 asphalt consumption for paving was around 27 million tons (Tasker, 1995), 1994 road mileage was around 4 million miles (FHA, 1994), 1994-95 U.S. population was around 260,000,000 (U.S. Census Bureau, 2001), and 1995 employment for highway and street construction workers was around 200,000 workers (U.S. Census Bureau, 2001).  From these figures, asphalt consumption rates can be calculated from the Mendocino County data as 7 to 17 tons per mile of paved roadway, from the national data as about 7 tons per mile (note that per capita rates vary too much to serve as a useful surrogate).

Since it appears that miles of paved roadway is the best available surrogate for estimating asphalt use for paved roads, STI compiled estimates of paved road mileages for each county (Funk et al, 2001; Teale Data Center, 1997).  These data could be enhanced by surveys if undertaken by individual counties.  However, they serve as a useful default. In the absence of locally compiled usage data we recommend the use of a default asphalt usage of 10 tons per paved road mile in order to estimate the quantity of asphalt consumed in each county and for the State of California.  Note that this estimate can be validated against statewide asphalt sales data (by asphalt type and end use – roofing or paving), which is exclusively available to government agencies from the Asphalt Institute (http://www.asphaltinstitute.org, telephone 859-288-4960).  Thus, two alternative top-down approaches are feasible for computing asphalt consumption (1) dissagregation of statewide consumption or (2) use of default usage per mile of paved roadway.  Additionally, asphalt consumption for the different types of asphalt can be obtained from the Asphalt Institute or by using the assumed distributions discussed above (with the caveat that minimal or no cutback asphalts should be attributed to counties that restrict their use).  For counties that restrict cutback asphalts, it should be assumed that other types of asphalt are used to meet demand.

### Emission Factors – Asphalt Paving

The U.S. EPA’s document, Compilation of Air Pollutant Emission Factors (AP-42) provides emission factors of 88 pounds of VOC per barrel for cutback asphalt and 9.2 pounds of VOC per barrel applied for emulsified asphalt.  No data are published on the emission factors for hot-mix asphalts nor road oils; however, the factor for hot-mix asphalt should be much lower, and thus we have assumed that it is approximately one order of magnitude smaller than that for emulsified asphalts, or about 0.9 pounds of VOC per barrel applied.  In addition, we have assumed that the factor for road oils is approximately the same as that for cutback asphalt, 88 pounds of VOC per barrel.  (Note that one barrel equals 42 gallons.)  Asphalt densities typically lie between 7 to 9 lb per gallon, with hot-mix asphalts at the heavier end of the scale.  For the purposes of estimating TOG emissions, which are mainly due to the evaporation of diluents, diluents will be assumed to contain 100 percent TOG.  AP-42 assumes a diluent content between 25-45% by volume for cutback asphalts and diluent densities of 0.7, 0.8, 0.9 kg/liter for rapid-cure, medium-cure, and slow-cure cutbacks, respectively.  Typically, the diluent contents of emulsified asphalt and hot-mix asphalt are approximately 1 and 2 orders of magnitude (respectively) less than that for cutback asphalts.  The rate of diluent evaporation follows a pattern of logarithmic decay over time.  AP-42 illustrates that for cutback asphalts, the following fractions of total diluent evaporate after 4 months and may continue for a much longer period.

- Rapid cure (RC): 95 percent by weight of the diluent evaporates;

- Medium cure (MC): 70 percent by weight of the diluent evaporates; and

- Slow cure (SC): 25 percent by weight of the diluent evaporates.

If local data on asphalt use can be obtained, emissions can be calculated directly for cutback (or emulsified) by:

Mass Emissions = Weight of Diluent Used   Weight % of Diluent Evaporated, where

Volume of Cutback Used = Weight of Cutback Used ÷ Density of Cutback

Volume of Diluent Used = Volume of Cutback Used   Volume % Diluent in the Cutback

Weight of Diluent Used = Volume of Diluent Used   Density of Diluent

Sacramento Metropolitan Air Quality Management District (SMAQMD) provided the following emission factors for asphalts, which are based on a 1991 survey conducted in that District, where the VOC contents of asphalts are limited by regulations.

Hot Mix: 0.04 lb VOC/ton

Emulsified: 17.9 lbs VOC/ton

Road Oils (SC cutback):  70.4 lbs/ton

Cutback (RC, MC): 268.3 lbs/ton

### Top-Down Activity Data Collection – Asphalt Roofing

In the absence of local surveys of asphalt roofing practices, statewide or national consumption can be used to dissagregate county-wide emissions in California. The EIIP (2000) estimates that 15-60 pounds (typically 20 lb) of asphalt are consumed per 100 square feet of roofing materials used.  This factor in combination with estimates of annual usage of roofing materials can be used to generate a statewide estimate of the amount of asphalt used to install roofing. Statewide asphalt roofing consumption can then be apportioned to counties according to socioeconomic factors (such as building permits, number of dwellings, etc.).  Note that these estimates can be validated against statewide asphalt sales data (by asphalt type and end use – roofing or paving), which is exclusively available to government agencies from the Asphalt Institute (http://www.asphaltinstitute.org, telephone 859-288-4960). Thus, two alternative top-down approaches are feasible for estimating asphalt roofing consumption (1) dissagregation of statewide consumption (difference between total use and that calculated for use in road paving) or (2) use of default usage per 100 square feet of roofing materials.

### Emission Factors – Asphalt Roofing

In the past, the Asphalt Institute has quoted that 93% of roofing asphalt is applied hot, which is the method that is associated with significant emissions. Emissions from asphalt roofing operations can be estimated with the following equation (EIIP, 2000).

Mass Emissions (tons of VOC) = Weight of asphalt melted (tons)  6.2 pounds VOC/ton

### Temporal Distributions for Asphalt Paving and Roofing

Most asphalt applications occur during typical business hours; however, some large projects, emergency building maintenance, and/or highway maintenance operations involve overtime (late hours or 7 days per week).  Activity depends on the weather; road construction tends to peak during warmer and dryer months, while roofing construction and maintenance peaks just before California’s rainy season.  In California, employment for construction of highways, streets, bridges, and tunnels (NAIC code 2341) and employment for roofing, siding, and sheet metal contractors (NAIC code 2356) varied quarterly as listed below (U.S. Census, 2000).  We recommend that annual use of asphalt be distributed by season according to seasonal employment estimates.  Emissions should than be distributed over a period of several months following application and assume that rates of emissions decay logarithmically over time (as illustrated in AP-42 and mentioned above).

| - Asphalt Paving - Quarter 1: 16,132 employees - Quarter 2: 18,546 employees - Quarter 3: 20,167 employees - Quarter 4: 17,872 employees | - Asphalt Roofing - Quarter 1: 21,044 employees - Quarter 2: 20,523 employees - Quarter 3: 23,298 employees - Quarter 4: 24,252 employees |
| --- | --- |

## Calculations of VOC EMISSIONS from Paving and Roofing

### Paving Emissions

As reported by the EIA (2001), 20,366,000 bbl of asphalt were consumed in California for roofing and paving purposes combined during the year 2000.  Tasker (1996) estimated that nationally, 80% of asphalt use is for road paving and 20% is for roofing (i.e., 16,300,000 bbl are used for paving and 4,070,000 bbls are used for roofing).  (Note that a reasonably comparable consumption estimate is produced if the Caltrans (2000) estimate of statewide paved road mileage, 166,972 miles, is multiplied by an asphalt consumption factor of 10 tons per mile and an asphalt density of 8 lbs per gallon.) Statewide asphalt use for road paving applications is subdivided according to the distributions of materials usages that have been reported for past emission inventories in other areas of California (Mendocino County, San Diego, and South Coast Air Quality Management District):

Hot-mix: 80-95%  (88%)

Emulsified: 2-15% (9%)

Road Oils: 1-3%    (2%)

Cutback: <1%        (1%)

Taking the mid-point of the ranges listed above in parentheses, yields:

Hot-mix: 	14,344,000 bbl

Emulsified:	  1,467,000 bbl

Road Oils: 	     326,000 bbl

###### Cutback:	     163,000 bbl

Total		16,300,000 bbl

Applying the emission factors from AP-42 –  0.9, 9.2, 88, and 88 lb VOC/bbl for hot-mix, emulsified, road oils, and cutback, respectively – yields annual statewide VOC asphalt paving emissions as follows, which represents an uncontrolled scenario:

Hot-mix:	6,455 	tons per year

Emulsified:	6,748 	tons per year

Road Oils:	14,344 	tons per year

Cutback:	7,172 	tons per year

Total	34,719	tons per year

Individual county emissions can be apportioned according to the miles of paved roads in each county, which were acquired from the Teale Data Center (1997). Thus, VOC emissions from paved roads for counties without controls can be calculated according to the equation below.  Results are presented in Tables 1 and 2.  For counties that restrict the use of cutback, we have arbitrarily subtracted the majority (85%) of cutback quantities used and added this figure to the quantities of emulsified asphalt used in order to represent to some degree the effects of these restrictions.  In addition, for counties that limit the VOC contents of asphalts we applied the emission factors from SMAQMD – 0.04, 17.9, 70.4, and 268.3 lb VOC/bbl for hot-mix, emulsified, road oils, and cutback, respectively.  Hypothetically, if all counties limited the VOC contents of their asphalts, statewide emissions would be estimated as shown below by applying these SMAQMD emission factors:

Hot-mix: 	50	tons per year

Emulsified:	2,298	tons per year

Road Oils: 	2,008	tons per year

Cutback:	3,827	tons per year

Total	8,183

If all counties furthermore banned the use of cutback, statewide emissions would be calculated as follows by reallocating 85% of cutback quantities to the emulsified quantities:

Hot-mix: 	50	tons per year

Emulsified:	2,515	tons per year

Road Oils: 	2,008	tons per year

Cutback:	574	tons per year

Total	5,147

For counties that are split between air basins, emissions may be further distributed spatially by using an appropriate surrogate, such as population or road miles.  (We used population, acquired from Funk, et al [2001] for this purpose.)

Emissions for County A =  Statewide Emissions×Paved Road MilesCounty APaved Road MilesCalifornia

### Roofing Emissions

As reported by the EIA (2001), 20,366,000 bbl of asphalt were consumed in California during the year 2000 for roofing and paving purposes combined.   If 20% of the total is used for roofing purposes, then it is estimated that 4,070,000 bbls are used for roofing.  Applying the emission factor from AP-42 of 6.2 lb VOC/ton of roofing material yields annual statewide VOC asphalt roofing emissions as follows.

4,070,000 bbl   42 gal/bbl   8.34 lb/gal  1 ton/2000 lb  6.2 lb VOC/ton  1 ton/2000lb =  2,211 tons per year

Individual county emissions can be apportioned according to the distribution of socioeconomic surrogates, such as construction activity, which were acquired from Funk, et al (2001).  Thus, VOC emissions from roofing for counties without controls can be calculated as follows.  Results are presented in Tables 3 and 4.  (Individual counties may choose to adjust the statewide-apportioned values to account for any regulatory restrictions on roofing practices in their respective county.)  For counties that are split between air basins, emissions may be further distributed spatially by using an appropriate surrogate, such as total construction, which we acquired from Funk, et al [2001] for this purpose.

Emissions for County A =  Statewide Emissions×Construction ActivityCounty AConstruction ActivityCalifornia

### Emissions Data for Paving and Roofing

| Table 1a. County allocation factors and uncontrolled VOC emissions for road paving. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| County | Allocation Factor | VOC Emissions (tpy) |  |  |  |
|  |  | Hot-mix | Emulsified | Road Oils | Cutback |
| Alameda | 0.0149 | 96.39 | 100.8 | 214.2 | 107.1 |
| Alpine | 0.0012 | 7.74 | 8.1 | 17.2 | 8.6 |
| Amador | 0.0043 | 27.86 | 29.1 | 61.9 | 31.0 |
| Butte | 0.0118 | 76.02 | 79.5 | 168.9 | 84.5 |
| Calaveras | 0.0081 | 52.15 | 54.5 | 115.9 | 57.9 |
| Colusa | 0.0054 | 34.89 | 36.5 | 77.5 | 38.8 |
| Contra Costa | 0.0132 | 85.28 | 89.2 | 189.5 | 94.8 |
| Del Norte | 0.0054 | 34.83 | 36.4 | 77.4 | 38.7 |
| El Dorado | 0.0152 | 98.15 | 102.6 | 218.1 | 109.1 |
| Fresno | 0.0399 | 257.55 | 269.3 | 572.3 | 286.2 |
| Glenn | 0.0072 | 46.24 | 48.3 | 102.7 | 51.4 |
| Humboldt | 0.0207 | 133.34 | 139.4 | 296.3 | 148.2 |
| Imperial | 0.0154 | 99.54 | 104.1 | 221.2 | 110.6 |
| Inyo | 0.0170 | 109.52 | 114.5 | 243.4 | 121.7 |
| Kern | 0.0727 | 469.05 | 490.4 | 1042.3 | 521.2 |
| Kings | 0.0109 | 70.51 | 73.7 | 156.7 | 78.3 |
| Lake | 0.0053 | 34.38 | 35.9 | 76.4 | 38.2 |
| Lassen | 0.0213 | 137.20 | 143.4 | 304.9 | 152.4 |
| Los Angeles | 0.0840 | 542.31 | 567.0 | 1205.1 | 602.6 |
| Madera | 0.0130 | 83.67 | 87.5 | 185.9 | 93.0 |
| Marin | 0.0065 | 41.98 | 43.9 | 93.3 | 46.6 |
| Mariposa | 0.0062 | 40.01 | 41.8 | 88.9 | 44.5 |
| Mendocino | 0.0221 | 142.51 | 149.0 | 316.7 | 158.3 |
| Merced | 0.0149 | 96.26 | 100.6 | 213.9 | 107.0 |
| Modoc | 0.0187 | 120.59 | 126.1 | 268.0 | 134.0 |
| Mono | 0.0077 | 49.87 | 52.1 | 110.8 | 55.4 |
| Monterey | 0.0237 | 152.96 | 159.9 | 339.9 | 170.0 |
| Napa | 0.0047 | 30.01 | 31.4 | 66.7 | 33.3 |
| Nevada | 0.0089 | 57.43 | 60.0 | 127.6 | 63.8 |
| Orange | 0.0225 | 145.52 | 152.1 | 323.4 | 161.7 |
| Placer | 0.0123 | 79.64 | 83.3 | 177.0 | 88.5 |
| Plumas | 0.0163 | 105.38 | 110.2 | 234.2 | 117.1 |
| Riverside | 0.0414 | 267.20 | 279.3 | 593.8 | 296.9 |
| Sacramento | 0.0135 | 86.83 | 90.8 | 193.0 | 96.5 |
| San Benito | 0.0063 | 40.70 | 42.5 | 90.4 | 45.2 |
| San Bernardino | 0.0817 | 527.20 | 551.2 | 1171.6 | 585.8 |
| San Diego | 0.0392 | 253.05 | 264.5 | 562.3 | 281.2 |
| San Francisco | 0.0032 | 20.90 | 21.9 | 46.5 | 23.2 |
| San Joaquin | 0.0138 | 88.77 | 92.8 | 197.3 | 98.6 |
| San Luis Obispo | 0.0202 | 130.21 | 136.1 | 289.4 | 144.7 |
| San Mateo | 0.0089 | 57.75 | 60.4 | 128.3 | 64.2 |
| Santa Barbara | 0.0190 | 122.81 | 128.4 | 272.9 | 136.5 |
| Santa Clara | 0.0186 | 119.79 | 125.2 | 266.2 | 133.1 |
| Santa Cruz | 0.0066 | 42.35 | 44.3 | 94.1 | 47.1 |
| Shasta | 0.0191 | 123.19 | 128.8 | 273.8 | 136.9 |
| Sierra | 0.0049 | 31.50 | 32.9 | 70.0 | 35.0 |
| Siskiyou | 0.0304 | 196.33 | 205.3 | 436.3 | 218.1 |
| Solano | 0.0073 | 47.08 | 49.2 | 104.6 | 52.3 |
| Sonoma | 0.0135 | 86.83 | 90.8 | 193.0 | 96.5 |
| Stanislaus | 0.0132 | 84.93 | 88.8 | 188.7 | 94.4 |
| Sutter | 0.0049 | 31.77 | 33.2 | 70.6 | 35.3 |
| Tehama | 0.0135 | 87.40 | 91.4 | 194.2 | 97.1 |
| Trinity | 0.0120 | 77.28 | 80.8 | 171.7 | 85.9 |
| Tulare | 0.0204 | 131.84 | 137.8 | 293.0 | 146.5 |
| Tuolumne | 0.0095 | 61.37 | 64.2 | 136.4 | 68.2 |
| Ventura | 0.0150 | 96.62 | 101.0 | 214.7 | 107.4 |
| Yolo | 0.0070 | 45.20 | 47.3 | 100.4 | 50.2 |
| Yuba | 0.0057 | 37.10 | 38.8 | 82.5 | 41.2 |
| State Total | 1.0 | 6,454.8 | 6,748.2 | 14,344.0 | 7,172.0 |

| Table 1b. County allocation factors and controlled VOC emissions for road paving. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| County | Allocation Factor | VOC Emissions (tpy) |  |  |  |
|  |  | Hot-mix | Emulsified | Road Oils | Cutback |
| Alameda | 0.0149 | 0.75 | 37.6 | 30.0 | 8.6 |
| Alpine | 0.0012 | 0.06 | 3.0 | 2.4 | 0.7 |
| Amador | 0.0043 | 27.86 | 29.1 | 61.9 | 31.0 |
| Butte | 0.0118 | 0.59 | 29.6 | 23.6 | 6.8 |
| Calaveras | 0.0081 | 52.15 | 54.5 | 115.9 | 57.9 |
| Colusa | 0.0054 | 0.27 | 13.6 | 10.9 | 3.1 |
| Contra Costa | 0.0132 | 0.66 | 33.2 | 26.5 | 7.6 |
| Del Norte | 0.0054 | 34.83 | 36.4 | 77.4 | 38.7 |
| El Dorado | 0.0152 | 0.76 | 38.2 | 30.5 | 8.7 |
| Fresno | 0.0399 | 257.55 | 269.3 | 572.3 | 286.2 |
| Glenn | 0.0072 | 46.24 | 48.3 | 102.7 | 51.4 |
| Humboldt | 0.0207 | 133.34 | 139.4 | 296.3 | 148.2 |
| Imperial | 0.0154 | 0.77 | 38.8 | 31.0 | 8.9 |
| Inyo | 0.0170 | 0.85 | 42.7 | 34.1 | 9.7 |
| Kern | 0.0727 | 3.65 | 182.7 | 145.9 | 41.7 |
| Kings | 0.0109 | 70.51 | 73.7 | 156.7 | 78.3 |
| Lake | 0.0053 | 34.38 | 35.9 | 76.4 | 38.2 |
| Lassen | 0.0213 | 137.20 | 143.4 | 304.9 | 152.4 |
| Los Angeles | 0.0840 | 4.22 | 211.3 | 168.7 | 48.2 |
| Madera | 0.0130 | 83.67 | 87.5 | 185.9 | 93.0 |
| Marin | 0.0065 | 0.33 | 16.4 | 13.1 | 3.7 |
| Mariposa | 0.0062 | 0.31 | 15.6 | 12.4 | 3.6 |
| Mendocino | 0.0221 | 142.51 | 149.0 | 316.7 | 158.3 |
| Merced | 0.0149 | 96.26 | 100.6 | 213.9 | 107.0 |
| Modoc | 0.0187 | 120.59 | 126.1 | 268.0 | 134.0 |
| Mono | 0.0077 | 0.39 | 19.4 | 15.5 | 4.4 |
| Monterey | 0.0237 | 152.96 | 159.9 | 339.9 | 170.0 |
| Napa | 0.0047 | 0.23 | 11.7 | 9.3 | 2.7 |
| Nevada | 0.0089 | 57.43 | 60.0 | 127.6 | 63.8 |
| Orange | 0.0225 | 1.13 | 56.7 | 45.3 | 12.9 |
| Placer | 0.0123 | 0.62 | 31.0 | 24.8 | 7.1 |
| Plumas | 0.0163 | 105.38 | 110.2 | 234.2 | 117.1 |
| Riverside | 0.0414 | 2.08 | 104.1 | 83.1 | 23.8 |
| Sacramento | 0.0135 | 0.68 | 33.8 | 27.0 | 7.7 |
| San Benito | 0.0063 | 40.70 | 42.5 | 90.4 | 45.2 |
| San Bernardino | 0.0817 | 4.10 | 205.4 | 164.0 | 46.9 |
| San Diego | 0.0392 | 1.97 | 98.6 | 78.7 | 22.5 |
| San Francisco | 0.0032 | 0.16 | 8.1 | 6.5 | 1.9 |
| San Joaquin | 0.0138 | 88.77 | 92.8 | 197.3 | 98.6 |
| San Luis Obispo | 0.0202 | 1.01 | 50.7 | 40.5 | 11.6 |
| San Mateo | 0.0089 | 0.45 | 22.5 | 18.0 | 5.1 |
| Santa Barbara | 0.0190 | 0.96 | 47.8 | 38.2 | 10.9 |
| Santa Clara | 0.0186 | 0.93 | 46.7 | 37.3 | 10.7 |
| Santa Cruz | 0.0066 | 42.35 | 44.3 | 94.1 | 47.1 |
| Shasta | 0.0191 | 0.96 | 48.0 | 38.3 | 11.0 |
| Sierra | 0.0049 | 31.50 | 32.9 | 70.0 | 35.0 |
| Siskiyou | 0.0304 | 1.53 | 76.5 | 61.1 | 17.5 |
| Solano | 0.0073 | 0.37 | 18.3 | 14.6 | 4.2 |
| Sonoma | 0.0135 | 0.68 | 33.8 | 27.0 | 7.7 |
| Stanislaus | 0.0132 | 84.93 | 88.8 | 188.7 | 94.4 |
| Sutter | 0.0049 | 31.77 | 33.2 | 70.6 | 35.3 |
| Tehama | 0.0135 | 0.68 | 34.0 | 27.2 | 7.8 |
| Trinity | 0.0120 | 77.28 | 80.8 | 171.7 | 85.9 |
| Tulare | 0.0204 | 131.84 | 137.8 | 293.0 | 146.5 |
| Tuolumne | 0.0095 | 61.37 | 64.2 | 136.4 | 68.2 |
| Ventura | 0.0150 | 0.75 | 37.6 | 30.1 | 8.6 |
| Yolo | 0.0070 | 0.35 | 17.6 | 14.1 | 4.0 |
| Yuba | 0.0057 | 37.10 | 38.8 | 82.5 | 41.2 |
| State Total | 1.0 | 2,213.7 | 3,944.8 | 6,175.3 | 2,802.9 |

Table 2.   Air basin allocation factors and VOC emissions for road paving in counties

that are split between air basins (controlled emissions).

| County | Air Basin | VOC Emissions (tons/yr) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Allocation | Hot-mix | Emulsified | Road Oils | Cutback |
| El Dorado | Tahoe | 0.12 | 0.09 | 4.5 | 3.6 | 1.0 |
|  | Mountain Counties | 0.88 | 0.67 | 33.7 | 26.9 | 7.7 |
| Placer | Tahoe | 0.02 | 0.02 | 0.8 | 0.6 | 0.2 |
|  | Sacramento Valley | 0.89 | 0.55 | 27.6 | 22.0 | 6.3 |
|  | Mountain Counties | 0.09 | 0.05 | 2.7 | 2.1 | 0.6 |
| Solano | San Francisco Bay Area | 0.74 | 0.27 | 13.6 | 10.9 | 3.1 |
|  | Sacramento Valley | 0.26 | 0.09 | 4.7 | 3.8 | 1.1 |

| Table 3. County allocation factors and VOC emissions for roofing. |  |  |  |
| --- | --- | --- | --- |
| County | Allocation Factor | VOC Emissions (tons/yr) |  |
| Alameda | 0.053 | 116.5 |  |
| Alpine | 0.000 | 0.1 |  |
| Amador | 0.001 | 2.6 |  |
| Butte | 0.007 | 16.5 |  |
| Calaveras | 0.002 | 3.6 |  |
| Colusa | 0.001 | 1.4 |  |
| Contra Costa | 0.029 | 65.1 |  |
| Del Norte | 0.001 | 1.9 |  |
| El Dorado | 0.004 | 9.8 |  |
| Fresno | 0.033 | 73.1 |  |
| Glenn | 0.001 | 2.1 |  |
| Humboldt | 0.004 | 9.3 |  |
| Imperial | 0.004 | 8.6 |  |
| Inyo | 0.001 | 1.3 |  |
| Kern | 0.022 | 48.6 |  |
| Kings | 0.003 | 7.6 |  |
| Lake | 0.002 | 4.4 |  |
| Lassen | 0.001 | 2.1 |  |
| Los Angeles | 0.187 | 413.2 |  |
| Madera | 0.004 | 8.3 |  |
| Marin | 0.009 | 20.3 |  |
| Mariposa | 0.001 | 1.4 |  |
| Mendocino | 0.003 | 7.4 |  |
| Merced | 0.008 | 16.7 |  |
| Modoc | 0.000 | 0.6 |  |
| Mono | 0.001 | 1.4 |  |
| Monterey | 0.037 | 81.0 |  |
| Napa | 0.005 | 11.0 |  |
| Nevada | 0.004 | 8.8 |  |
| Orange | 0.056 | 123.4 |  |
| Placer | 0.011 | 24.5 |  |
| Plumas | 0.001 | 1.6 |  |
| Riverside | 0.029 | 63.1 |  |
| Sacramento | 0.041 | 90.9 |  |
| San Benito | 0.005 | 10.3 |  |
| San Bernardino | 0.035 | 77.4 |  |
| San Diego | 0.104 | 229.5 |  |
| San Francisco | 0.040 | 88.2 |  |
| San Joaquin | 0.016 | 34.6 |  |
| San Luis Obispo | 0.010 | 22.5 |  |
| San Mateo | 0.026 | 58.3 |  |
| Santa Barbara | 0.013 | 29.4 |  |
| Santa Clara | 0.071 | 157.1 |  |
| Santa Cruz | 0.021 | 46.9 |  |
| Shasta | 0.006 | 13.5 |  |
| Sierra | 0.000 | 0.3 |  |
| Siskiyou | 0.002 | 3.4 |  |
| Solano | 0.012 | 25.7 |  |
| Sonoma | 0.017 | 36.6 |  |
| Stanislaus | 0.016 | 34.3 |  |
| Sutter | 0.002 | 4.9 |  |
| Tehama | 0.002 | 4.3 |  |
| Trinity | 0.000 | 0.9 |  |
| Tulare | 0.013 | 28.0 |  |
| Tuolumne | 0.002 | 4.5 |  |
| Ventura | 0.016 | 34.5 |  |
| Yolo | 0.006 | 13.4 |  |
| Yuba | 0.002 | 4.4 |  |
| State Total | 1.0 | 2211 |  |

Table 4.   Air basin allocation factors and VOC emissions for roofing in counties

that are split between air basins.

| County | Air Basin | Allocation | VOC Emissions (tons/year) |
| --- | --- | --- | --- |
| El Dorado | Tahoe | 0.11 | 1.1 |
|  | Mountain Counties | 0.89 | 8.8 |
| Placer | Tahoe | 0.02 | 0.4 |
|  | Sacramento Valley | 0.91 | 22.4 |
|  | Mountain Counties | 0.07 | 1.6 |
| Solano | San Francisco Bay Area | 0.73 | 18.7 |
|  | Sacramento Valley | 0.27 | 7.0 |

## References and Useful Source of Information

Asphalt Roofing Manufacturers Association, 4041 Power Mill Road, Suite 404, Calverton, Maryland 20705, Telephone: 301.348.2002, Fax: 301.348.2020, http://www.asphaltroofing.org/.

Caltrans (2000) Mileage of maintained public roads in each county by jurisdiction as of December 31, 1999.

EIA (2001)  Petroleum Supply Annual 2000, Volume 1. Stocks of crude oil and petroleum products by pad district, 2000. table 12. pad district vsupply, disposition, and ending stocks of crude oil and petroleum products, 2000 (table 30).  Report prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, June.  http://www.eia.doe.gov/oil_gas/petroleum/data_publications/petroleum_supply_annual/psa_volume1/psa_volume1.html

EIIP. 2001. Emission Inventory Improvement Program Guidance for Emission Inventory Development, Volume III, Chapter 17, Asphalt Paving.  January. http://www.epa.gov/ttn/chief/eiip/techreport/volume03/iii17_apr2001.pdf

EIIP. 2000.  Abstract – Asphalt Roofing Kettles.  September. http://www.epa.gov/ttn/chief/eiip/techreport/volume03/asphalt.pdf

EPA. 1996. Compilation of Air Pollution Emission Factors - Volume I: Stationary Point and Area Sources, Fifth Edition and Supplements, AP-42. U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards. (GPO-055-000-00251-7). Research Triangle Park, North Carolina.

EPA. 1994a. Final Report - Evaluation of Emissions from Paving Asphalts. U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards, EPA-600/R-94-135. Research Triangle Park, North Carolina

EPA. 1994b. AIRS Database. U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards. Research Triangle Park, North Carolina.

EPA. 1991. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I: General Guidance for Stationary Sources. U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards, EPA-450/4-91-016. Research Triangle Park, North Carolina.

Federal Highway Administration. 1994.  Highway Statistics.  U.S. Department of Transportation, Washington D.C., FHWA-PL-95-042.

Funk T.H., Stiefer P.S., and Chinkin L.R. (2001) Development of gridded spatial allocation factors for the state of California. Technical memorandum prepared for California Air Resources Board, Sacramento, CA by Sonoma Technology, Inc., Petaluma, CA, STI-900201/999542-2092-TM, July.

Moulthrop, J.S., R.G. Hicks, W.R. Ballou. 1997. Emulsion: The Future of Pavement Maintenance? Emulsified Asphalt seeks its niche in the paving industry. The Asphalt Contractor. February. p. 49.

Puzinauskas, V.P. 1979. Emissions from Asphalt Roofing Kettles. Research Report No. 79-2, The Asphalt Institute, College Park, MD, November, 1979.

State of California South Coast Air Quality Management District Rule 1108.1. Emulsified Asphalt. Adopted August 3, 1979, amended December 4, 1981, November 4, 1983. Paragraph (b) Requirements.

Tasker, Nasreen P. 1996. What do the cards hold for the asphalt industry?  Presentation at the joint meeting of the International Slurry Surfacing Association and the Asphalt Emulsion Manufacturers Association, Phoenix, Arizona, February. http://www.srv.net/~whone/ia/iagen.html

Teale Data Center.  1997.  ROADSA Coverage [data file] for California.  Stephen P Teale Data Center, Rancho Cordova, CA.

U.S. Census Bureau. 2001.  Population data downloaded from the Internet at http://www.census.gov/population/estimates/nation/intfile1-1.txt.

U.S. Census Bureau. 2000. 1997 Economic Census, Construction - Geographic Area Series for California. http://www.census.gov/prod/ec97/97c23-ca.pdf.

Wolbach, Dean.  2001.  Personal communication. Mendocino County Air Pollution Control District.

Status of cutback asphalt restrictions in California.

| Air Quality Districts | Restriction on cutback asphalt? | Rule No. |
| --- | --- | --- |
| Amador County Air Pollution Control District | No |  |
| Antelope Valley Air Pollution Control District | Yes | 1108 |
| Bay Area Air Quality Management District | Yes | 8-15 |
| Butte County Air Quality Management District | Yes | 241 |
| Calaveras County Air Pollution Control District | No |  |
| Colusa County Air Pollution Control District | Yes | 2.33 |
| El Dorado County Air Pollution Control District | Yes | 224 |
| Feather River Air Quality Management District | No |  |
| Glenn County Air Pollution Control District | Yes | 99.1 |
| Great Basin Unified Air Pollution Control District | No |  |
| Imperial County Air Pollution Control District | Yes | 426 |
| Kern County Air Pollution Control District | Yes | 410.5 |
| Lake County Air Quality Management District | No |  |
| Lassen County Air Pollution Control District | No |  |
| Mariposa County Air Pollution Control District | No |  |
| Mendocino County Air Quality Management District | No |  |
| Modoc County Air Pollution Control District | No |  |
| Mojave Desert Air Quality Management District | Yes | 1103 |
| Monterey Bay Unified Air Pollution Control District | Yes | 425 |
| North Coast Unified Air Quality Management District | No |  |
| Northern Sierra Air Quality Management District | No |  |
| Northern Sonoma County Air Pollution Control District | No |  |
| Placer County Air Pollution Control District | Yes | 217 |
| Sacramento Metropolitan Air Quality Management District | Yes | 453 |
| San Diego County Air Pollution Control District | Yes | 67.7 |
| San Joaquin Valley Unified Air Pollution Control District | Yes | 4641 |
| San Luis Obispo County Air Pollution Control District | Yes | 420 |
| Santa Barbara County Air Pollution Control District | Yes | 329 |
| Shasta County Air Quality Management District | Yes | 3.15 |
| Siskiyou County Air Pollution Control District | No |  |
| South Coast Air Quality Management District | No | 1108 |
| Tehama County Air Pollution Control District | Yes | 4.26 |
| Tuolumne County Air Pollution Control District | No |  |
| Ventura County Air Pollution Control District | Yes | 74.4 |
| Yolo-Solano Air Quality Management District | Yes | 2.28 |

Note: This information is from May 1999.

| Office | Telephone | Address | City | County Name |
| --- | --- | --- | --- | --- |
| Amador County | 209-223-6429 | 500 Argonaut Ln | Jackson | Amador |
| Gridley | 530-846-3631 | 685 Kentucky St | Gridley | Butte |
| Butte County | 530-538-7681 | 7 County Center Dr | Oroville | Butte |
| Oroville | 530-538-2420 | 1735 Montgomery St | Oroville | Butte |
| Paradise | 916-872-6288 | 5555 Skyway | Paradise | Butte |
| Calaveras | 209-754-6402 | 891 Mountain Ranch Rd | San Andreas | Calaveras |
| Colusa County | 530-458-0466 | 1215 Market St | Colusa | Colusa |
| Placerville | 530-642-5250 | 487 Main St | Placerville | El Dorado |
| South Lake Tahoe | 916-542-6030 | 1900 Lake Tahoe Blvd | So. Lake Tahoe | El Dorado |
| Orland | 916-865-7232 | Parks Roosevelt Ave | Orland | Glenn |
| Orland | 916-865-9053 | 815 4th St | Orland | Glenn |
| Glenn County | 530-934-6530 | 777 N Colusa St | Willows | Glenn |
| Willows | 530-934-7041 | 201 N Lassen St | Willows | Glenn |
| Mariposa County | 209-966-5356 |  | Mariposa | Mariposa |
| Mendocino Center - Road Division | 707-983-6811 | 23290 Airport Rd | Covelo | Mendocino |
| Mendocino Center - Road Division | 707-984-6716 | 1825 Branscomb Rd | Laytonville | Mendocino |
| Mendocino Center | 707-463-4363 | 340 Lake Mendocino Dr | Ukiah | Mendocino |
| Ukiah | 707-463-6200 | 300 Seminary Ave | Ukiah | Mendocino |
| Willits | 707-459-4605 | 380 E Commercial St | Willits | Mendocino |
| Grass Valley | 916-274-4360 | 125 E Main St | Grass Valley | Nevada |
| Nevada County | 530-265-1411 | 950 Maidu Ave | Nevada City | Nevada |
| Truckee | 916-582-7700 | 11570 Donner Pass Rd | Truckee | Nevada |
| Auburn | 530-823-4211 | 1225 Lincoln Way | Auburn | Placer |
| Placer County | 530-889-7500 | 11444 B Ave | Auburn | Placer |
| Lincoln | 916-645-8576 | 1530 3rd St | Lincoln | Placer |
| Loomis | 916-652-1848 | 3770 Taylor Rd | Loomis | Placer |
| Rocklin | 916-632-4130 | 3980 Rocklin Rd | Rocklin | Placer |
| Roseville - Administration | 916-774-5331 | 316 Vernon St | Roseville | Placer |
| Placer - Engineering | 916-581-6226 | 565 West Lake Bl | Tahoe City | Placer |
| Portola | 916-832-5194 | 120 Main | Portola | Plumas |
| Plumas County | 530-283-6268 | 1834 E Main St | Quincy | Plumas |
| Galt City Engineer | 209-745-0575 | 495 Industrial Dr | Galt | Sacramento |
| Sacramento | 916-264-7100 | 915 I St # 200 | Sacramento | Sacramento |
| Sacramento | 916-433-6220 | 5730 24th St | Sacramento | Sacramento |
| Sacramento | 916-264-5214 | 20 28th St | Sacramento | Sacramento |
| Sacramento | 916-264-7995 | 1231 I St # 200 | Sacramento | Sacramento |
| Sacramento | 916-566-6477 | 3550 Marysville Blvd | Sacramento | Sacramento |
| Sacramento - Administration | 916-264-7110 | City Hall | Sacramento | Sacramento |
| Sacramento - Engineering | 916-264-5707 | 915 I St | Sacramento | Sacramento |
| Sacramento - Traffic Engineering | 916-264-5307 | 1000 I St # 200 | Sacramento | Sacramento |
| Sacramento - Transportation | 916-366-2227 | 906 G St Ste | Sacramento | Sacramento |
| Sacramento - Administrator | 916-552-8132 | 827 7th Street | Sacramento | Sacramento |
| Sacramento County | 916-875-2700 | 3030 Explorer Dr # 5 | Sacramento | Sacramento |
| Sacramento County | 916-875-5555 | 9700 Goethe Rd # C | Sacramento | Sacramento |
| Sacramento County | 916-874-6651 | 609 9th St | Sacramento | Sacramento |
| Sacramento County | 916-875-5296 | 4101 Branch Center Rd | Sacramento | Sacramento |
| Sacramento County | 916-875-5000 | 827 7th St | Sacramento | Sacramento |
| Redding | 530-225-4170 | 760 Parkview Ave | Redding | Shasta |
| Shasta County | 916-225-5661 | 1855 Placer St | Redding | Shasta |
| Sierra - Maintenance Shop | 916-289-3412 | Highway | Downieville | Sierra |
| Sierra County | 530-289-3201 | 101 Courthouse Sq | Downieville | Sierra |
| Sierra County | 916-993-0423 | 100 Courthouse Sq | Downieville | Sierra |
| Benicia - Administration | 707-746-4240 | 250 E L St | Benicia | Solano |
| Dixon | 916-678-7050 | 285 E Chestnut St | Dixon | Solano |
| Fairfield - Administration | 707-428-7485 | City Hall 1000 Webster | Fairfield | Solano |
| Navy Center | 707-425-0827 | 2550 Hilborn Rd | Fairfield | Solano |
| Rio Vista | 707-374-6747 | 600 Saint Francis Way | Rio Vista | Solano |
| Vacaville - Administration | 707-449-5169 | 650 Merchant St | Vacaville | Solano |
| Vallejo - Director | 707-648-4315 | 555 Santa Clara St | Vallejo | Solano |
| Vallejo - Maintenance | 707-648-4319 | 111 Amador St | Vallejo | Solano |
| Winters - Administration | 916-795-2820 | 318 1st St | Winters | Solano |
| Live Oak | 530-695-3436 | 9955 Live Oak Blvd | Live Oak | Sutter |
| Sutter - Director | 916-741-7450 | 1160 Civic Center Blvd | Yuba City | Sutter |
| Sutter County | 530-822-7450 | 1160 Civic Center Blvd # D | Yuba City | Sutter |
| Yuba City | 530-822-4626 | 1201 Civic Center Blvd | Yuba City | Sutter |
| Corning | 916-824-7029 | 794 3d St | Corning | Tehama |
| Tehama County | 530-385-1462 | 9380 San Benito Ave | Gerber | Tehama |
| Sonora | 209-532-2922 | 111 School St | Sonora | Tuolumne |
| Tuolumne | 209-533-5601 | 48 Yaney Ave | Sonora | Tuolumne |
| Davis – Engineering | 916-757-5686 | 233 Russell Blvd | Davis | Yolo |
| West Sacramento | 916-373-5850 | 1951 S River Rd | West Sacramento | Yolo |
| Woodland | 916-666-8920 | 300 1st St | Woodland | Yolo |
| Woodland | 530-661-5961 | 300 1st St | Woodland | Yolo |
| Yolo County | 916-666-8775 | 292 W Beamer St | Woodland | Yolo |
| Yolo County | 916-744-1299 | 51230 Clarksburg Rd | Courtland | Yolo |
| Marysville | 530-741-6644 | 526 C St | Marysville | Yuba |
| Yuba - Maintenance Yard | 916-741-6536 | 5390 Fruitland Rd | Marysville | Yuba |
| Yuba County | 530-741-6421 | 938 14th St | Marysville | Yuba |

| Company Name | Telephone | Address | City | County Name |
| --- | --- | --- | --- | --- |
| Rec Ski Equipment | 209-295-7665 | 26690 Wagon Wheel Dr | Pioneer | Amador |
| Tober Heating & Air Inc | 209-223-1730 | 840 Hwy | Jackson | Amador |
| Sun Mountain Roofing | 209-296-5374 | 19405 Cherokee Ln | Volcano | Amador |
| Wooden Man The | 209-245-6954 | 9490 Pacific | Plymouth | Amador |
| Mountain Air | 209-296-7497 | Corner Of Taves & Hwy | Pine Grove | Amador |
| Sutter Creek Roof Contractors | 209-223-4496 | 757 N Main St | Jackson | Amador |
| Amador Sheet Metal | 209-267-5595 | 22 Broad | Sutter Creek | Amador |
| American Fence Co | 916-533-8039 | 2552 5th Ave | Oroville | Butte |
| Sierra Glass | 916-895-3630 | 2990 Esplanade | Chico | Butte |
| Bi-Rite Steel & Fabrication | 916-342-6491 | 2209 Park Ave | Chico | Butte |
| Scott Mobile Enterprises | 916-877-2866 | 1210 Severns Ln | Paradise | Butte |
| Erwin Roofing Co Inc | 916-533-1366 | 36 Canyon Highlands Dr | Oroville | Butte |
| Ridgetop Roofing | 916-873-5000 | 14825 Del Oro Dr | Magalia | Butte |
| Hoffman Mark A Construction | 916-343-8218 | 715 Sequoyah Ave | Chico | Butte |
| Sweems Air Conditioning | 916-533-8060 | 2581 Liability Ln | Oroville | Butte |
| Miskella John J Miskella John Jas Roofing | 916-343-9031 | 888 Lindo Ln | Chico | Butte |
| Brown Ken Construction | 916-873-1215 | 14559 Skyway | Magalia | Butte |
| Moore Sheet Metal Works | 916-342-4597 | 431 W 9th St | Chico | Butte |
| Harward & Company | 916-893-5898 | 3851 Morrow Ln | Chico | Butte |
| Oak Tree Construction | 916-898-1001 | 1824-1/2 Esplanade | Chico | Butte |
| Four Counties Roofing Co | 916-343-1416 | 3 Crusader Ct | Chico | Butte |
| Gene M Jessee Inc | 916-342-2909 | 1627 Nord Ave | Chico | Butte |
| North Valley Raingutter | 916-894-3347 | 27 Fairgate Ln | Chico | Butte |
| George Roofing | 916-533-6393 | 1875 Wyandotte Ave | Oroville | Butte |
| Double J Roofing | 916-846-2791 | 280 Sheldon Ave | Gridley | Butte |
| Jeff George Roofing | 916-589-2215 | 1090 Hurleton Rd | Oroville | Butte |
| Cobb Elvie D Roofing | 916-876-0153 | PO Box | Magalia | Butte |
| Majestic Land Works | 916-589-2741 | 95 Shane Ct | Berry Creek | Butte |
| Versatile Roofing Co | 916-898-9106 | 6256 Steiffer Rd | Magalia | Butte |
| Alchemy Painting | 916-877-9076 | PO Box | Paradise | Butte |
| All Roofing Co | 916-872-0315 | 697 Buschmann Rd | Paradise | Butte |
| Alladin Roofing | 916-533-2934 | 11595 Miller Peak Rd | Oroville | Butte |
| Nelson J Patrick Roofng Contr | 916-877-8008 | 5722 Newman Ave | Paradise | Butte |
| Abc Seamless Siding Of Chico | 916-894-5115 | PO Box | Forest Ranch | Butte |
| Aalgaard Construction | 916-877-6021 | 9001 Skyway | Paradise | Butte |
| Weir Roofing | 916-533-6016 | 2849 Foothill Blvd | Oroville | Butte |
| A-Jacks Roofing Co | 916-896-1217 | PO Box | Chico | Butte |
| Parr Roofing & Associates | 209-728-3551 | PO Box | Murphys | Calaveras |
| D R I Supply | 209-754-3479 | PO Box | San Andreas | Calaveras |
| Scott Jim Heating & Sheet Metal | 209-795-3268 | 1231 Linebaugh Rd | Arnold | Calaveras |
| Scott Jim Roofing & Heating Inc | 209-795-1577 | 1231 Linenbaugh Rd | Arnold | Calaveras |
| Wellmaker Roofing | 209-728-8357 | 3428 Brice Station Rd | Murphys | Calaveras |
| Calaveras Roofing | 209-795-4856 | 702 Yuroc Dr | Dorrington | Calaveras |
| Dolin David Roofing | 209-772-9850 | 6279 Cane Ln | Valley Springs | Calaveras |
| Rcm Heating & Air Conditioning | 209-754-1711 | 5469 Old Emigrant Trl E | Mountain Ranch | Calaveras |
| C & B Heating Air Conditioning & Rain Gutters | 209-795-7124 | PO Box | Arnold | Calaveras |
| Arts Sheet Metal | 209-728-3373 | 969 Highway | Douglas Flat | Calaveras |
| Armfield Ken Metal Work | 916-473-2580 | 2542 Davis Rd | Williams | Colusa |
| Park West Home Supply | 916-622-0991 | 6100 Enterprise Dr | Diamond Springs | El Dorado |
| Alpine Sheet Metal | 916-541-4180 | 2152 Ruth Ave | South Lake Tahoe | El Dorado |
| Lakehills Roofing Company | 916-933-3199 | PO Box | El Dorado Hills | El Dorado |
| Jurgens Construction | 916-677-5103 | 27121lacanadact | Camino | El Dorado |
| Placerville Roofing | 916-644-2486 | PO Box | Camino | El Dorado |
| Sun Roofing Systems | 916-677-4291 | 5050highcrestdr | Shingle Springs | El Dorado |
| Placerville Glass Inc | 916-622-6417 | 3030 Chapel St | Placerville | El Dorado |
| Hooker R Roofing Co | 916-541-5895 | 3748 Larch Ave | South Lake Tahoe | El Dorado |
| King Roofing | 916-577-4342 | 1186 Normuk | Tahoe Paradise | El Dorado |
| D & D Precision Sheet Metal | 916-626-4118 | 331 Industrial Dr | Diamond Springs | El Dorado |
| Lindsey Roofing | 916-642-8766 | 3794 Kismet Ln | Placerville | El Dorado |
| Mac-Tek II | 916-622-5816 | 5810 Mother Lode Dr | Placerville | El Dorado |
| Snyder Pat Roofing | 916-542-1686 | 2700 Knox Ave | South Lake Tahoe | El Dorado |
| Paradise Roofing & Repair | 916-573-1306 | 1802 Santa Fe Rd | South Lake Tahoe | El Dorado |
| Messina Metal Mfg | 916-933-1966 | 5050 Hillsdale Cir | El Dorado Hills | El Dorado |
| Skyline Enterprises Ofc | 916-933-2773 | 2222 Francisco Dr | El Dorado Hills | El Dorado |
| Custom Roofing | 916-676-4425 | 3958 Cambridge Rd | Cameron Park | El Dorado |
| Guild Harold Roofing | 916-644-7496 | 2975 Camino Heights Dr | Camino | El Dorado |
| Mott Gordon Roofing Co Inc | 916-622-1199 | 6601 Merchandise Way | Diamond Springs | El Dorado |
| Central State Roofing Co | 916-677-2864 | 3766 Calderwood Rd | Shingle Springs | El Dorado |
| El Dorado Roofing | 916-626-1616 | 1900 Highway | Placerville | El Dorado |
| Breeding Neil L Inc Painting Contractor | 916-626-7784 | 1792 Beals Rd | Placerville | El Dorado |
| Scott Roofing | 916-621-1026 | PO Box | Placerville | El Dorado |
| Tru-Truss Engineering | 916-933-3478 | 1783 Arroyo Vista Way | El Dorado Hills | El Dorado |
| Sequoia Roofing | 916-626-1280 | 4610 Enterprise Ct | Diamond Springs | El Dorado |
| Ruperts Roofing | 916-544-1484 | 806 Jeffery St | South Lake Tahoe | El Dorado |
| All Dry Roofing | 916-577-9228 | PO Box | South Lake Tahoe | El Dorado |
| El Dorado Truss Co Inc | 916-622-1264 | 300 Industrial Dr | Placerville | El Dorado |
| Tahoe Sheet Metal | 916-541-3768 | 2178 Eloise Ave | South Lake Tahoe | El Dorado |
| Shingle Springs Heating & Air Conditioning | 916-933-0454 | 4020 Durock Rd | Shingle Springs | El Dorado |
| High Sierra Builders Inc | 916-544-5600 | 1943 Kokanee Way | South Lake Tahoe | El Dorado |
| Applied-Rite Roofing Systems | 916-621-0769 | PO Box | El Dorado | El Dorado |
| R W E Weatherproofing | 916-676-1727 | 2160 Marden Dr | Rescue | El Dorado |
| Sheet Metal Shop The | 916-541-2266 | Highway 50 & | South Lake Tahoe | El Dorado |
| Bippus Roofing Co | 916-458-7003 | PO Box | Orland | Glenn |
| Rackley Company Inc | 916-865-9619 | 3772 County Road | Orland | Glenn |
| Bippus Roofing Co | 916-865-3921 | 616 8th St | Orland | Glenn |
| Sinclair Fred Sinclair Heat & Air Conditioning | 916-934-3261 | 346 N Tehama St | Willows | Glenn |
| Hefty Sheet Metal | 916-865-9677 | 740 E Walker St | Orland | Glenn |
| Greeley Hill Roofing | 209-878-3144 | 5708 Cuneo Rd | Coulterville | Mariposa |
| Jerrys Custom Truss Mfg | 209-966-2326 | 4052 Silver Bar Rd | Mariposa | Mariposa |
| Mariposa Mobile Power Wash | 209-966-4343 | 4050 Live Oak Rd | Mariposa | Mariposa |
| Steve Roberts Roofing | 707-462-9125 | 780 N State St | Ukiah | Mendocino |
| Quinliven Melvin Ukiah Roofing Co | 707-463-0489 | 1194 S Dora St | Ukiah | Mendocino |
| Advanced Mfg & Development | 707-459-9451 | 300 E Hill Rd | Willits | Mendocino |
| Doug Warner Roofing | 707-964-7807 | 21344 S Petaluma Ave | Fort Bragg | Mendocino |
| Hookins Ernie Valley Roofing | 707-462-5166 | 212 Thompson St | Ukiah | Mendocino |
| J & M Sheet Metal | 707-459-6912 | 375 E San Francisco Ave | Willits | Mendocino |
| Sheet Metal Works | 707-462-3729 | 275 Cherry St | Ukiah | Mendocino |
| Loomis Tim Construction Co | 707-961-0114 | 19249 Benson Ln | Fort Bragg | Mendocino |
| Big River Construction | 707-463-2550 | 21511 Orr Springs Rd | Ukiah | Mendocino |
| Ocean Glass & Sash Whse | 707-884-4016 | 38001 Old Stage Rd | Gualala | Mendocino |
| Valley Mechanical | 707-462-1656 | 966 Mazzoni St | Ukiah | Mendocino |
| Branesky Sheet Metal | 707-964-0691 | 636 N Franklin St | Fort Bragg | Mendocino |
| Fagundes Ron Roofng Contr | 707-459-5110 | PO Box | Willits | Mendocino |
| Gibbs Roof Co | 707-964-3533 | PO Box | Fort Bragg | Mendocino |
| Mendocino Roofing Inc | 707-485-7102 | PO Box | Redwood Valley | Mendocino |
| Adco Sheet Metal | 707-462-6551 | 390 N Main St | Ukiah | Mendocino |
| Piedmont Lumber Co-Truss Division | 707-485-8781 | 6301 N State | Calpella | Mendocino |
| Redwood Roofers | 707-937-1700 | PO Box | Mendocino | Mendocino |
| Ely Roofing Inc | 916-343-7663 | 13291 Contractors Dr | Chicago Park | Nevada |
| Innovative Metal Fabrication | 916-272-4191 | 179 Clydesdale Ct | Grass Valley | Nevada |
| Homeowners Resource The | 916-894-7777 | 1950 E-20th Chico | Chicago Park | Nevada |
| Truckee Heating Sheet Metal & Plumbing | 916-587-2511 | 10745 W River St | Truckee | Nevada |
| Ferris Heating | 916-587-5343 | 16025 Melbourne Ct | Truckee | Nevada |
| Nelson Roofing & Removal | 916-274-1515 | 15140 Forrest Rd | Grass Valley | Nevada |
| Welch Roofing & Supply | 916-272-8433 | 12800 Ridge Rd | Grass Valley | Nevada |
| Unique Building | 916-343-8680 | 920 W-11th Ave | Chicago Park | Nevada |
| Wright Gutters | 916-477-6151 | 15022 Beyers Ln | Grass Valley | Nevada |
| Johnson Brett Roofing Co | 916-587-7113 | 10544 Pine Cone Dr | Truckee | Nevada |
| Truss-Fab | 916-823-6166 | 12600 Locksley Ln | Auburn | Nevada |
| Butte Roofing Co | 916-342-6553 | PO Box | Chicago Park | Nevada |
| Sierra Pre-Bilt | 916-477-8787 | 11352 E Bennett Rd | Grass Valley | Nevada |
| Johnson Gregg Roofing | 916-587-1220 | 10008 High St | Truckee | Nevada |
| Reeds Motherlode Roofing | 916-645-9520 | 3884 Mount Pleasant Rd | Lincoln | Placer |
| Hagans Roofing | 916-786-3553 | 222 Evelyn Ave | Roseville | Placer |
| Rinkydink Builders | 916-663-3387 | 390 Buena Vista Ave | Newcastle | Placer |
| James Construction | 916-645-1449 | 1665 Kilaga Springs Rd | Lincoln | Placer |
| Holland Dave Roofing | 916-663-4930 | 2555 Newcastle Rd | Newcastle | Placer |
| A Clean America | 916-624-2850 | 3735 Clover Valley Rd | Rocklin | Placer |
| Abc Supply Co Inc | 916-784-0100 | 9020 Atkinson St | Roseville | Placer |
| Carvalho Roofing Company | 916-624-2942 | PO Box | Rocklin | Placer |
| Casey A James Roofing | 916-583-7555 | River Road Trade Center | Tahoe City | Placer |
| Advanced Deck Designs | 916-781-3325 | 460 Tahoe Ave | Roseville | Placer |
| Alpine Roofing | 916-888-6191 | 248 Sutherland Dr | Auburn | Placer |
| Applegate Mobile Home Service | 916-878-0381 | PO Box | Applegate | Placer |
| Auburn Roofing Inc | 916-823-9900 | 150 Elm Ave | Auburn | Placer |
| Auburn Sheet Metal Works | 916-878-7256 | 87 Apple Ln | Applegate | Placer |
| Baker Roofing | 916-878-1778 | PO Box | Meadow Vista | Placer |
| Factory Direct | 916-885-5167 | 11808 Kemper Rd | Auburn | Placer |
| Joyce Sheet Metal | 916-823-5155 | 58 P And S Ln | Newcastle | Placer |
| Builders Prestain | 916-652-0471 | 3415 Swetzer Rd | Loomis | Placer |
| Independent Maintenance & Roofing | 916-546-3025 | PO Box | Tahoe Vista | Placer |
| Roseville Precision Inc | 916-645-1628 | 3620 Cincinnati Ave | Rocklin | Placer |
| Jones Johnnie Roofing | 916-652-9737 | 3445 Sugarloaf Mountain Rd | Loomis | Placer |
| Creative Sheet Metal | 916-823-6210 | 336 Sacramento St | Auburn | Placer |
| Millers Heating Air Conditioning & Sheet Metal | 916-885-6172 | 2112 Drive In Way | Auburn | Placer |
| Roghers Roofing | 916-587-2154 | PO Box | Carnelian Bay | Placer |
| Roof Life Co | 916-624-4951 | 2252 Sierra Meadows Dr | Rocklin | Placer |
| Forster Heating Air Cond | 916-888-1163 | 9405 Crater Hill Rd | Auburn | Placer |
| C & H Heating & Sheet Metal | 916-546-7700 | PO Box | Kings Beach | Placer |
| Incline Glass Inc | 916-546-2749 | 250 Village Bl | Brockway | Placer |
| Roseville Air Conditioning & Heating | 916-786-6532 | 1312 Church St | Roseville | Placer |
| Gray Construction | 916-652-7602 | PO Box | Auburn | Placer |
| Cole Heating & Air Conditioning Inc | 916-791-5655 | 4608 Crombie Ct | Roseville | Placer |
| Brad Strong Roofing | 916-652-3259 | PO Box | Loomis | Placer |
| Nogleberg Heating & Air Conditioning | 916-346-8827 | 25135 Pineview Dr | Colfax | Placer |
| Rose Roofing & Supply | 916-823-3632 | 144 Teamtrack Rd | Auburn | Placer |
| Triple S Roofing | 916-885-4691 | 12075 Holly Vista Way | Auburn | Placer |
| Custom Fabrication | 916-786-2078 | 213 Kenroy Lne | Roseville | Placer |
| Runkle Steve M Roofing Contractor | 916-783-3787 | 2004 Katherine Pl | Roseville | Placer |
| Wood-Revivers | 916-663-3177 | 7222 English Colony Way | Penryn | Placer |
| Precision Metal Fabricators Inc | 916-632-1224 | 575 Menlo Dr | Rocklin | Placer |
| Newman & Sons | 916-645-2110 | 2533 Mccourtney Rd | Lincoln | Placer |
| Degrazier Roofing | 916-587-3672 | PO Box | Truckee | Placer |
| Summit Roofing | 916-782-4664 | 502 Giuseppe Ct | Roseville | Placer |
| North Tahoe Quality Roofing Inc | 916-583-5900 | 690 Ward Creek Bl | Tahoe City | Placer |
| T R C Roofing | 916-887-0404 | 1191 Star Ln | Auburn | Placer |
| Daniels Roofing | 916-583-1513 | PO Box | Tahoe City | Placer |
| Almanor Energy Plus | 916-596-3128 | 3732 Big Springs Rd | Lake Almanor | Plumas |
| Bradshaw Max Roofing | 916-832-5697 | 309 Portola Ave | Portola | Plumas |
| Johnsons Heating Air Cond & Sheet Metal Fabr | 916-836-1374 | 999 Johnsville Rd | Blairsden | Plumas |
| Cml Heatin G & Refrigeration | 916-283-1036 | 7071 Bucks Lake Rd | Meadow Valley | Plumas |
| Lassen Truss & Crane | 916-258-2021 | 160 Richardson Wy | Chester | Plumas |
| Sierra Valley Truss Co | 916-832-5159 | 190 Industrial Wy | Beckwourth | Plumas |
| Circle Roofing | 916-283-0452 | PO Box | Quincy | Plumas |
| Paul Arlan & Wyrna | 916-284-7665 | N Valley Rd | Greenville | Plumas |
| Madden Sheet Metal | 916-283-4489 | 2289 E Main St | Quincy | Plumas |
| Miller Sheet Metal | 916-283-5131 | 1750 Lee Rd | Quincy | Plumas |
| Hayes Construction | 916-331-7086 | 3432 Pinehill Way | Sacramento | Sacramento |
| Home Team-Tom Yanceys | 916-979-1888 | 7101 Fair Oaks Blvd | Carmichael | Sacramento |
| Roof Industry Consultants | 916-487-6814 | 3807 Pasadena Ave | Sacramento | Sacramento |
| One Stop Home Improvement | 916-332-0549 | 6525-32d | North Highlands | Sacramento |
| Home Service Center | 916-929-9197 | 1412 Rene Ave | Sacramento | Sacramento |
| T & M Industrial Metals Inc | 916-424-4935 | 3039-52d Ave | Sacramento | Sacramento |
| Roof Rangers Inc | 209-745-5753 | 10393 Live Oak Rd | Galt | Sacramento |
| Olympic Roofing | 916-361-6959 | 10398 Rockingham Dr | Sacramento | Sacramento |
| Tabish Construction Inc | 916-631-8950 | 113 Wesleyan Way | Folsom | Sacramento |
| Home Team-Tom Yanceys Re-Roofing Gutters & Dow | 916-979-1892 | 7101 Fair Oaks Bl | Carmichael | Sacramento |
| Hemm Robert E | 916-362-7970 | 7798-2 Excelsior | Sacramento | Sacramento |
| Housh Roofing & Construction | 916-455-5130 | 8175 Alpine Ave | Sacramento | Sacramento |
| Zimmerman Roofing | 916-454-3667 | 3560 Ramona Ave | Sacramento | Sacramento |
| Zumwalt & Associates | 916-965-7663 | 5000 San Juan Ave | Fair Oaks | Sacramento |
| Briggs Roofing & Repair | 916-339-0615 | 6412 Watt Ave | North Highlands | Sacramento |
| Old Country Roofing | 916-920-6333 | 3951 Development Dr | Sacramento | Sacramento |
| Trus Joist Macmillan | 916-649-6835 | 819w Striker Ave | Sacramento | Sacramento |
| Brazeal Speciality Coating & Construction | 916-685-7961 | 9020 Bradshaw Rd | Elk Grove | Sacramento |
| Urethane Foam Roofing Harry Bunfill | 916-988-0522 | 7446 Beech Ave | Orangevale | Sacramento |
| Brand S Roofing Inc | 916-381-0613 | 6501 Florin Perkins Rd | Sacramento | Sacramento |
| Freeman & Young Construction | 916-485-6803 | 5942 Don Way | Carmichael | Sacramento |
| Hester Roofing | 916-381-8000 | 8120 35th Ave | Sacramento | Sacramento |
| Mcguire Earth Works Landscaping | 916-962-1942 | 9725 Fair Oaks Blvd | Fair Oaks | Sacramento |
| Lifetime Shingle Co Inc | 916-452-3433 | 4000 Deeble St | Sacramento | Sacramento |
| Les Mauch Roofing Company | 916-682-9249 | 7845 Bar Du Ln | Sacramento | Sacramento |
| Larson K L Siding & Window | 916-481-9928 | 2230 Loma Vista Dr | Sacramento | Sacramento |
| Patterson Heating & Air Conditioning | 916-685-3766 | 9055 Locust St | Elk Grove | Sacramento |
| Lane Ervin Co Inc | 209-745-1104 | 13570 Sargent Ave | Galt | Sacramento |
| Kims Roofing Co | 916-369-0310 | 3211 Huntsman Dr | Sacramento | Sacramento |
| Kelly Kevin General Contractors | 916-925-7331 | 1808 Woolley Way | Sacramento | Sacramento |
| Keeney Construction | 916-481-4420 | 3211 El Camino Ave | Sacramento | Sacramento |
| Homes By Pacific | 916-423-3150 | 9723 Railroad St | Elk Grove | Sacramento |
| K-Designers | 916-631-9300 | 11261 Sunrise Park Dr | Rancho Cordova | Sacramento |
| Precision Metal Crafters | 916-928-0558 | 1143 N Market Blvd # W | Sacramento | Sacramento |
| Mcqueens Earl Roofing Co | 916-920-4161 | 2629 Evergreen St | Sacramento | Sacramento |
| Meyers Roofing Service | 916-442-5520 | 860 Onstott Rd | Yuba City | Sacramento |
| Mikes Sheet Metal Products | 916-991-3061 | 7231-26th | Rio Linda | Sacramento |
| Outsiders Construction | 916-974-0370 | 3108 Ellington Cir | Sacramento | Sacramento |
| Ortiz A Jerry Roofing | 916-368-7835 | 6813 Mc Comber St | Sacramento | Sacramento |
| Pro Metals | 916-424-8148 | 189 Otto Cir | Sacramento | Sacramento |
| Pyramid Sheet Metal | 916-944-1733 | 4021 California Ave | Carmichael | Sacramento |
| Hudson R F Roofing | 916-388-1133 | 9588 School St | Elk Grove | Sacramento |
| Sunrise Sheet Metal | 916-851-0181 | 11121 Sun Center Dr | Rancho Cordova | Sacramento |
| Parsons Roofing | 916-381-3487 | 6740 Fruitridge Rd | Sacramento | Sacramento |
| Walker Lumber Co | 916-338-2121 | 6925-30th | North Highlands | Sacramento |
| Sacramento Shingle Co | 916-966-0152 | 7337 Roseville Rd | Fair Oaks | Sacramento |
| Advanced Siding & Roofing | 916-965-5114 | 7210 Palm Ave | Fair Oaks | Sacramento |
| Sierra Spray Foam Roofing | 209-748-2662 | 11750 Borden Rd | Herald | Sacramento |
| Cordova Roofing Supply | 916-635-0358 | 2541 Grennan Ct | Rancho Cordova | Sacramento |
| Comfort Seal Inc | 916-383-0735 | 8146 Junipero St | Sacramento | Sacramento |
| Channell Roofing | 916-991-3564 | 6734 Rio Linda Blvd | Rio Linda | Sacramento |
| Century Homestead Inc | 916-381-7799 | 5665 Power Inn Rd | Sacramento | Sacramento |
| Sierra Vista | 916-851-1630 | 11333 Sunco Dr | Rancho Cordova | Sacramento |
| Cei West Roofing Co | 916-635-9921 | 3257 Fitzgerald Rd | Rancho Cordova | Sacramento |
| Vorhies Construction | 916-387-6407 | 5401 Warehouse Way | Sacramento | Sacramento |
| Casebolt Construction | 916-989-5330 | 102 Spring Water Way | Folsom | Sacramento |
| Alco Siding & Windows | 916-362-1400 | 10255 Old Placerville Rd | Sacramento | Sacramento |
| Adell Bruce N Aiae Consulting Architect | 916-972-0664 | 3417 Arden Way | Sacramento | Sacramento |
| Alex Engardt Roofing & Siding Co | 916-452-7341 | 7700-14th Ave | Sacramento | Sacramento |
| Aci Distribution | 916-929-4401 | 160 Main Ave | Sacramento | Sacramento |
| Ace Air Manufacturing Inc & Mechanical | 916-338-0111 | 7012-34th | North Highlands | Sacramento |
| Walsh Mike Roofing Inc | 916-451-7054 | 3747 W Pacific Ave | Sacramento | Sacramento |
| Warrington Roofing | 916-923-3354 | 1685 Helena Ave | Sacramento | Sacramento |
| Abc Seamless Of Calif | 916-776-0065 | 14165 Market | Walnut Grove | Sacramento |
| Watson Brothers Roofing | 916-481-6293 | 3491 Orange Grove Ave | North Highlands | Sacramento |
| Aaa Roofing | 916-991-5292 | 9500 Newton Rd | Elverta | Sacramento |
| A-Plus Quality Roofcard | 916-988-7880 | 8841 Nimbus Way | Orangevale | Sacramento |
| Watson Roofing Inc | 916-723-3439 | 3491g Orange Grove Wy | Citrus Heights | Sacramento |
| A-Glb Roofing | 916-638-7200 | 11353 Pyrites Way | Rancho Cordova | Sacramento |
| A Roof Doctor Co | 916-721-6130 | 6725 Smithhart St | Orangevale | Sacramento |
| A & D Welding & Manufacturing | 916-858-0184 | 3255 Monier Cir | Rancho Cordova | Sacramento |
| Collier Warehouse-Sacramento Inc | 916-635-0782 | 11225 Trade Center Dr | Rancho Cordova | Sacramento |
| Sierra Lumber & Fence Co | 916-688-8489 | 7849 Stockton Blvd | Sacramento | Sacramento |
| Finley & Sons Construction | 916-989-3234 | 8947 Coan Ln | Orangevale | Sacramento |
| Future Roofing | 916-686-8270 | 9055-A Locust St | Elk Grove | Sacramento |
| Sacramento Valley Contractors | 916-331-9855 | 4069 Palm Ave | Sacramento | Sacramento |
| Eversult Sheet Metal Inc | 916-967-4701 | 8019 Sacramento St | Fair Oaks | Sacramento |
| Valley Roof Removal | 916-731-5600 | 7551-14th Av | Sacramento | Sacramento |
| Bonacci Roofing | 916-332-5505 | 6525-32d N Hilnds | Sacramento | Sacramento |
| Bmb Corp | 916-631-9120 | 11460 Elks Cir | Rancho Cordova | Sacramento |
| Sheet Metal Fabricators And Supply Inc | 916-638-6884 | 11390 Amalgam Way | Rancho Cordova | Sacramento |
| Better Roofs | 916-638-8986 | 10301 Georgetown Dr | Rancho Cordova | Sacramento |
| Berkan & Clark Air Conditioning | 916-485-2665 | 2544 Tesla Way | Sacramento | Sacramento |
| Berco Builders Inc | 916-484-1996 | 4560 Auburn Blvd | Sacramento | Sacramento |
| Ajaxx Supply Co | 916-635-2245 | 2587 Mercantile Dr | Rancho Cordova | Sacramento |
| Shingle Comp Any The Inc | 916-332-1900 | PO Box | North Highlands | Sacramento |
| Fontaine & Bean Inc | 916-443-3789 | 200 27th St | Sacramento | Sacramento |
| Atkins Roofing | 916-988-6955 | 6716 La Junta Dr | Orangevale | Sacramento |
| Artistic Siding | 916-349-9234 | 3919 Weybridge Way | Sacramento | Sacramento |
| Artesia Siding | 916-992-6917 | 199 Artesia Rd | Elverta | Sacramento |
| Armour Guard Roofing | 916-364-0950 | 8911 Gulfport Way | Sacramento | Sacramento |
| Antelope Home Improvement & Construction | 916-725-0309 | 8108 Filifera Way | Antelope | Sacramento |
| Western Wood Fabricators | 916-991-4400 | 3700 Riego Rd | Elverta | Sacramento |
| Amalgamated Building Components | 916-484-1373 | 4216 Roseville Rd | North Highlands | Sacramento |
| Aluminum Builders Of California | 916-635-1467 | 11355 Folsom Bl | Rancho Cordova | Sacramento |
| Witt Bob Roofing | 916-991-3505 | 6904-7th | Rio Linda | Sacramento |
| Allied Enterprises Inc | 916-381-3330 | 8788 Elder Creek Rd | Sacramento | Sacramento |
| All Side Supply Center Alside Supply Center | 916-922-9327 | 1801 Bell Ave | Sacramento | Sacramento |
| Yancey Company | 916-441-1616 | 3009 C St | Sacramento | Sacramento |
| B & H Whole Inc | 916-920-5372 | 2500 Grand Ave | Sacramento | Sacramento |
| Elk Grove Heating & Air Conditioning | 916-685-6130 | PO Box | Wilton | Sacramento |
| A & B Roofing | 916-344-2129 | 5940 Rosebud Ln | Sacramento | Sacramento |
| Cj Little Roofing | 209-745-9625 | 347 Poplar St | Galt | Sacramento |
| Hunter Roofing | 916-649-2356 | 1214 Arcade Blvd | Sacramento | Sacramento |
| Hunt Roofing | 916-423-1072 | 6725 Fleming Ave | Sacramento | Sacramento |
| Clark Roofing | 916-929-3311 | 2076 Acoma St | Sacramento | Sacramento |
| Sacramento Exteriors | 916-369-7433 | 9648 Kiefer Blvd | Sacramento | Sacramento |
| Engineered Metal Systems Inc Ofc | 916-448-0999 | 2727 River Plaza Dr # | Sacramento | Sacramento |
| Cmc Roofing | 916-721-3493 | PO Box | Citrus Heights | Sacramento |
| I M Distributing Corp | 916-381-1800 | 8541 23rd Ave | Sacramento | Sacramento |
| Diamond Roofing | 916-988-5590 | 6004 Hazel Ave | Orangevale | Sacramento |
| Dial One T & G Roofing Inc | 916-965-3032 | 7808 Old Auburn Rd | Citrus Heights | Sacramento |
| Dial One Metro Energy Insulation Co | 916-722-1110 | 5997b Devecchi Ave | Citrus Heights | Sacramento |
| Dial One Metro Energy Insulation | 916-348-0271 | 5997 Devecchi Ave | Citrus Heights | Sacramento |
| Golden State Roofing | 916-451-2460 | 5409-55th | Sacramento | Sacramento |
| Custom Metal Fabricators | 916-334-1022 | 5760 Roseville Rd | Sacramento | Sacramento |
| Cole Heating & Air Conditioning Inc | 916-331-8191 | 6727-32d | North Highlands | Sacramento |
| River City Roofing Inc | 916-929-2028 | 2270 Dale Ave | Sacramento | Sacramento |
| Rfc Manufacturing | 916-381-4742 | 8540 Thys Ct | Sacramento | Sacramento |
| Suburban Roofing Co Inc | 916-457-9888 | 5613 Elvas Ave | Sacramento | Sacramento |
| Golden Aire Heating & Air Conditioning | 916-988-6137 | 9408 Orangevale Ave | Orangevale | Sacramento |
| Stewart Sheetmetal | 916-383-6767 | 9284 Jackson Rd | Sacramento | Sacramento |
| Gold Metal Service-Heating & Air Conditioning | 916-988-0313 | 9477 Greenback Ln | Folsom | Sacramento |
| Rhetta Henry Sh2a Inc Architects | 916-441-0686 | 1718-3d | Sacramento | Sacramento |
| H & M Roofing | 916-635-7400 | 3646 Omec Cir | Rancho Cordova | Sacramento |
| Decker Hess-Tech Inc | 916-349-0352 | 5036 Hemlock St | Sacramento | Sacramento |
| Redwood Refinishers | 916-361-1111 | 4122 Happy Ln | Sacramento | Sacramento |
| Royal Roofing Co Inc | 916-452-7291 | 8004 Hickory Ave | Sacramento | Sacramento |
| Degen Fred Pacific Builders | 916-383-3168 | 5421 84th St | Sacramento | Sacramento |
| Reo Roofing Company | 916-381-2915 | 8520 Kiefer Bl | Sacramento | Sacramento |
| Gudgel-Yancey Roofing Commercial Div | 916-387-6902 | 5321-84th | Sacramento | Sacramento |
| Cigelske Roofing & Construction | 916-721-7330 | 6720c Van Maren Ln | Citrus Heights | Sacramento |
| Trabil Enterprises | 916-484-6252 | 4045 Wayside Ln | Carmichael | Sacramento |
| Roofing Technology Consulting Inc | 916-961-5717 | PO Box | Fair Oaks | Sacramento |
| Imperial Roofing Co | 916-393-1934 | 7437 Cosgrove Way | Sacramento | Sacramento |
| Coffey & Sons Roofing Co | 916-646-4242 | 2156 Surrey Rd | Sacramento | Sacramento |
| Giddens Brothers Inc Mechanical Contractors | 209-745-1697 | 232 S Lincoln Way | Galt | Sacramento |
| Neverkovec R L Roofing | 916-965-9301 | 7140 Spicer Dr | Citrus Heights | Sacramento |
| Standard Structures Inc | 916-988-5816 | 7222 Chestnut Ave | Orangevale | Sacramento |
| Southgate Roofing Co | 916-386-2401 | 7290 Hedge Ave | Sacramento | Sacramento |
| Cal Pac Roofing Of Sacramemto | 916-635-6300 | 11267 Coloma Rd | Rancho Cordova | Sacramento |
| Irwin Jeff Roofing Co | 916-961-2110 | 8801 Piedra Way | Fair Oaks | Sacramento |
| Solar Industries | 916-567-9650 | 731 N Market Blvd | Sacramento | Sacramento |
| Norms Sheet Metal Inc | 916-451-8357 | 4909 Franklin Blvd | Sacramento | Sacramento |
| Specialty Steel Service Co Inc | 916-924-9596 | 2531 Land Ave | Sacramento | Sacramento |
| Curtis Roofing | 916-451-7286 | 7475-14th Ave | Sacramento | Sacramento |
| Nab Construction | 916-961-4350 | 4790 Dewey Dr | Fair Oaks | Sacramento |
| Bunfill Harry Urethane Foam Roofing | 916-928-1620 | 1037w N Market Blvd | Sacramento | Sacramento |
| Gentry Roofing | 916-921-2209 | 506 Peralta Ave | Sacramento | Sacramento |
| Crown Roofing | 916-381-3498 | 8606 Merribrook Dr | Sacramento | Sacramento |
| J D Murray Sheetmetal | 916-338-3776 | 3429 E St | North Highlands | Sacramento |
| Delta Sierra Roofing Co | 916-448-0388 | PO Box | Sacramento | Sacramento |
| James W Stalder Co Inc | 916-482-7683 | 4320 Hazelwood Ave | Sacramento | Sacramento |
| Natural Energy Products Sunoptics Skylights | 916-395-4700 | 6350-27th | Sacramento | Sacramento |
| California Roofing And Inspection Service | 916-388-1305 | 89425 Rovana Cir | Sacramento | Sacramento |
| Cal-Asia | 916-685-5648 | 10547 E Stockton Blvd | Elk Grove | Sacramento |
| Mt Diablo Roofing Co | 916-687-7614 | 10017 Davis Rd | Wilton | Sacramento |
| California Steel | 209-745-2933 | 648 Fairway Dr | Galt | Sacramento |
| Guerrero's Sheetmetal & Fabrication | 916-921-1950 | 4037 Astoria St | Sacramento | Sacramento |
| Custom Exteriors Inc | 916-924-9444 | 3900 Winters St | Sacramento | Sacramento |
| Capital Sheet Metal | 916-443-3761 | 500 16th St | Sacramento | Sacramento |
| Capitol All Steel Canopies | 916-422-6712 | 6500 Belleau Wood Ln | Sacramento | Sacramento |
| California Foam Roofing Co | 916-853-9243 | 3655 Omec Park Dr | Rancho Cordova | Sacramento |
| Moylen Jim Roofing Contractor | 916-392-7189 | 7829 East Pkwy | Sacramento | Sacramento |
| California Cedar Roof Care | 916-635-9318 | 11295 Sunrise Gold Cir | Rancho Cordova | Sacramento |
| Hammon Ken Roofing | 916-241-4100 | 740 Lake Blvd | Redding | Shasta |
| Hunter Roofing Inc | 916-222-2111 | 3777 Meadow View Dr | Redding | Shasta |
| Harbert Roofing | 916-223-3251 | 10274 Maddelein Ln | Palo Cedro | Shasta |
| D T K Company Inc | 916-241-8098 | 4989 Mountain Lakes Blvd | Redding | Shasta |
| Shelter Roof Co | 916-246-8741 | P O Box | Redding | Shasta |
| Astin Roofing | 916-243-6710 | PO Box | Shasta Lake | Shasta |
| Barry J C Skylights Manufacturer | 916-244-6666 | 7032 Danyeur Rd | Redding | Shasta |
| Redding Metal Crafters | 916-222-4400 | 3871 Rancho Rd | Redding | Shasta |
| Bear Mountain Construction | 916-221-4570 | 1023 Roseland St | Redding | Shasta |
| Benson Roofing | 916-222-3838 | 3980 Marvel Ln | Redding | Shasta |
| Coates Roofing | 916-223-4725 | 2494 Castlewood Dr | Redding | Shasta |
| Sierra Pacific Construction | 916-223-1409 | 1864 Keystone Ct | Redding | Shasta |
| Haring Contractors | 916-222-1232 | 9817 Forgotten Trl | Redding | Shasta |
| Foam Experts Co | 916-221-0311 | 696 Grove St | Redding | Shasta |
| Shasta Metal Products | 916-246-7353 | 3245 Veda St | Redding | Shasta |
| Newman Sheet Metal | 916-221-4571 | 2540 Hartnell Ave | Redding | Shasta |
| Pacific West Roofing | 916-221-2575 | 20123 Cindyln | Redding | Shasta |
| Overdeck Roofing | 916-365-9492 | 557 Pleasant View Dr | Anderson | Shasta |
| Metal Siding & Roofing | 916-244-0172 | 5000 Westside Rd | Redding | Shasta |
| North State Gutter Co | 916-549-4661 | 10523 Endview Dr | Palo Cedro | Shasta |
| Builders Door & Window | 916-221-3667 | 850 Commerce St | Redding | Shasta |
| Phelps Construction | 916-241-5504 | 1670 Market St | Redding | Shasta |
| Carrs Seamless Rain Gutters | 916-244-3254 | 120 Branstetter Ln | Redding | Shasta |
| Wait Roofing & Seamless Gutters | 916-221-3223 | 1499 Beltline Rd | Redding | Shasta |
| Burke Roofing | 916-365-7003 | PO Box | Anderson | Shasta |
| Nachtwey Engineering | 916-243-9353 | 4305 Caterpillar Rd | Redding | Shasta |
| Cascade Seamless Gutters | 916-222-2626 | 1231 E Cypress Ave | Redding | Shasta |
| Add-A-Roof | 916-357-2070 | 5345 Pine St | Anderson | Shasta |
| Creative Edge | 916-224-7288 | 2736 Bechelli Ln | Redding | Shasta |
| Horsley Roofing | 916-365-6271 | 21962 Lone Tree Rd | Anderson | Shasta |
| Ark Design Construction | 916-223-1149 | PO Box | Redding | Shasta |
| Systems Plus | 916-378-6800 | 1800 S Barney Rd | Anderson | Shasta |
| North Valley Roofing & Gutters | 916-221-0981 | 2550 Alfreda Way | Redding | Shasta |
| Gard & Sons Heating & Plumbing Inc | 916-221-7494 | 20256 Skypark Dr | Redding | Shasta |
| Dave Cox Restoration | 916-241-7013 | 18121 Clear Creek Rd | Redding | Shasta |
| Cousin Garys Skycrest Building Systems Service | 916-241-5304 | 4991 Eastside Rd | Redding | Shasta |
| Baker & Baker Roofing Company | 707-557-3631 | 22 California St | Vallejo | Solano |
| Monier Roff Tile | 916-795-1360 | 27600 County Road | Winters | Solano |
| Benicia Heating & Air Conditioning | 707-745-6556 | 340 W Channel Rd | Benicia | Solano |
| Northwest Roofing & Waterproofing Inc | 707-447-9301 | 300 County Airport Rd | Vacaville | Solano |
| Wolter Roofing | 916-795-5216 | 208-1st | Winters | Solano |
| Mcdonalds Glass & Mirror | 707-643-0067 | 830 Broadway St | Vallejo | Solano |
| Allied Roofing & Insulation Co | 707-437-5441 | 4989 Noonan Ln | Fairfield | Solano |
| Adams Roofing | 707-554-4287 | 926 York St | Vallejo | Solano |
| Sierra Pacific Construction | 707-446-7550 | 892 Aldridge Rd | Vacaville | Solano |
| Cardinal Roofing Company Incorporated | 707-644-0132 | PO Box | Vallejo | Solano |
| Design Roofing & Siding Inc | 707-745-8300 | 727-1st | Benicia | Solano |
| Eagle Construction | 707-558-8246 | 204 Thomas Ave | Vallejo | Solano |
| Home Repairs R Us | 707-644-5155 | 302 Valle Vista Ave | Vallejo | Solano |
| Vallejo Sheet Metal & Heating Inc | 707-552-4328 | 924 Maine St | Vallejo | Solano |
| Anderson Truss | 916-678-1636 | 8810 Sparling Ln | Dixon | Solano |
| Construcive Services Company | 707-746-7397 | PO Box | Benicia | Solano |
| Allen-Simmons Heating & Sheet Metal Co From Va | 707-644-8957 | 1973 W Cordelia Rd | Vallejo | Solano |
| Tomlinson Construction Inc | 916-755-0438 | 657 Lincoln Rd | Yuba City | Sutter |
| Andys Heating & Air Conditioning Co Inc | 916-674-0486 | 108 Percy Ave | Yuba City | Sutter |
| Titan Truss Inc | 916-674-0663 | 753 N George Washington Blvd | Yuba City | Sutter |
| Feather River Roofing Inc | 916-673-3795 | 365 B St | Yuba City | Sutter |
| Richards Roofing | 916-695-2632 | 2472 Elm St | Live Oak | Sutter |
| Helsem Heating & Air Conditioning Inc | 916-673-1418 | 342 B Yc | Yuba City | Sutter |
| Globe Mechanical Co | 916-673-9698 | 958-A Von Geldern Way | Yuba City | Sutter |
| Tormec | 916-755-1065 | 2258 California St | Sutter | Sutter |
| Sutter Foam & Coating | 916-671-3626 | 710 Cooper Ave | Yuba City | Sutter |
| Hubbart Roofing Co | 916-674-8486 | 3333 Industrial Dr | Yuba City | Sutter |
| Johnson Roofing Service | 916-671-9228 | 1021 Northgate Dr | Yuba City | Sutter |
| Action Roofing Service | 916-673-4067 | 477 Georg Wshngtn B | Yuba City | Sutter |
| Western Foam Inc | 916-673-1261 | 1465 Barry Rd | Yuba City | Sutter |
| Condrey Construction | 916-755-4887 | PO Box | Yuba City | Sutter |
| Haker Roofing | 916-347-4837 | PO Box | Cottonwood | Tehama |
| Cert Welding & Sheet Metal Mobile Welding Serv | 916-824-5028 | South Ave | Corning | Tehama |
| Johnson Bill General Contra | 916-527-7055 | 333 Ash St | Red Bluff | Tehama |
| Hinkle Roofing | 916-385-1153 | 9440 San Benito Ave | Gerber | Tehama |
| Macs Roofing | 916-347-4030 | 3198 Meadow Oak Dr | Cottonwood | Tehama |
| Holman Fred Roofing | 916-527-3686 | 1300 Vista Way | Red Bluff | Tehama |
| Factory Direct Siding Co | 916-529-3742 | PO Box | Red Bluff | Tehama |
| Lang Roofing | 916-824-1215 | 3388 Highway | Corning | Tehama |
| John Rollf | 916-824-1144 | 24830 Florence Ave | Corning | Tehama |
| Griggs Roofing | 916-824-1304 | 263 Rio Verde Ct | Corning | Tehama |
| Nichol Gerald Roofing | 916-347-5975 | David Wy | Cottonwood | Tehama |
| Bennetts Construction | 916-824-0105 | 4245 Kopta Rd | Corning | Tehama |
| Pneumatic Conveying Mfg | 916-529-1212 | 205 Kimball Rd | Red Bluff | Tehama |
| Bailey Elastomeric Roofing | 916-347-6310 | 16266 Gas Point Rd | Cottonwood | Tehama |
| North State Truss | 916-347-9703 | 3660 Main St | Cottonwood | Tehama |
| Aronos Research Club | 209-532-8282 | 37 Elkin St | Sonora | Tuolumne |
| Ringen Roofing | 209-928-3890 | PO Box | Twain Harte | Tuolumne |
| Carter Roofing | 209-984-4212 | 17421 Table Mountain Rd | Jamestown | Tuolumne |
| Kerr Roofing Inc | 209-532-1187 | 17006 Mono Vista Rd | Sonora | Tuolumne |
| Zierenberg Painting | 209-962-7363 | 19059 Raboul Ct | Groveland | Tuolumne |
| Steves Mobile Home Service | 209-532-1167 | 18003 Old Wards Ferry Rd | Sonora | Tuolumne |
| Roof Doctor The | 209-928-3968 | 18769 Carter St | Tuolumne | Tuolumne |
| Groveland Sheet Metal | 209-962-7132 | 19145 Mill Villa Ct | Sonora | Tuolumne |
| Sierra Seamless Gutters | 209-962-4833 | 1921 4 Ferretti Rd | Groveland | Tuolumne |
| Foothill Mechanical | 209-533-4504 | 19451 Village Dr | Sonora | Tuolumne |
| Big Bear Roofing | 209-532-1299 | PO Box | Sonora | Tuolumne |
| Sierra Roofing | 209-586-5255 | 20723 Koteo Ln | Mi Wuk Village | Tuolumne |
| Griggs Roofing | 209-586-7663 | 25029 Highway | Sonora | Tuolumne |
| Vanns Sheet Metal | 209-532-3532 | 19340 Industrial Dr | Sonora | Tuolumne |
| Olstad Jim R & J Roofing | 916-662-8746 | 129 Kern Ave | Woodland | Yolo |
| Curtis Roofing | 916-758-9577 | 7475-14th Av | Davis | Yolo |
| Caluya Enterprises Roofing | 916-371-3744 | 617 N Harbor Blvd | Bryte | Yolo |
| Creative Roofing | 916-372-8728 | 1020 Taber St | Broderick | Yolo |
| Pacific Single Ply Inc | 916-757-2651 | PO Box | Davis | Yolo |
| Paulsen Brothers Inc Roofng Contrs | 916-662-5468 | 543 6th St | Woodland | Yolo |
| Bunfill Rick L R & R Enterprises Roofing Paint | 916-758-2466 | 39973 Barry Rd | Davis | Yolo |
| Custom Bilt Metals | 916-372-7696 | 839 Riske Ln | West Sacramento | Yolo |
| Mc Clintock Metal Fabricators Inc | 916-666-6007 | 455 Harter Ave | Woodland | Yolo |
| Century Roofing Co | 916-744-1350 | 52898 Clarksburg Rd | Clarksburg | Yolo |
| Bunfill Dennis D K B Unlimited | 916-756-5614 | 2219 Anza Ave | Davis | Yolo |
| Sacramento Roofing Company Inc | 916-662-4105 | P O Box | Woodland | Yolo |
| Wilson & Sons Roofing | 916-662-2617 | 1512-6th | Woodland | Yolo |
| Jim Patton Roofing | 916-662-1667 | 8 Grand Ave | Woodland | Yolo |
| Durafoam Contracting | 916-373-0300 | 3834 Commerce Dr | West Sacramento | Yolo |
| Snyder Jack F Roofing | 916-371-5445 | 2751 Rice Ave | West Sacramento | Yolo |
| Barth J Co Sheet Metl Service Dept | 916-662-2166 | 423-1st | Woodland | Yolo |
| Green Metal Fabricating | 916-371-0192 | 536 Houston St | West Sacramento | Yolo |
| Perryman Mechanical Inc | 916-371-8888 | 514 Glide Ave | West Sacramento | Yolo |
| Laukkanen Bros Inc-Plumbing Sheet Metal Heat A | 916-662-9296 | 231 W Main St | Woodland | Yolo |
| D K B Unlimited | 916-661-1004 | 589 N East St | Woodland | Yolo |
| Van Tassels Roofing | 916-743-6629 | 1024 Laurel Ave | Marysville | Yuba |
| Homewood Truss | 916-743-8855 | 7229 Dantoni Rd | Marysville | Yuba |
| North State Coatings | 916-749-8333 | PO Box | Marysville | Yuba |
| Hastey J Rex Inc | 916-743-6662 | 2487 Feather River Blvd | Marysville | Yuba |
| Norcal Lumber Co Inc | 916-742-1804 | 4601 Bernice Ave | Marysville | Yuba |
| Randys Roofing & Waterproofing | 916-741-2737 | 3399 Hammonton Smartville Rd | Marysville | Yuba |
| Crown Solar Products Inc | 916-743-9213 | 1334 Melody Rd | Marysville | Yuba |
| Galaxy Roofing Service | 916-742-7457 | 7684 Highway | Marysville | Yuba |

No emissions have been assigned to this category.

Note that this memorandum does not cover manufacturing of asphalt roofing materials, but only the application of asphalt by construction workers for roof installations and repairs.

The Asphalt Insititute declines to provide this information to consultants and contractors.  However, it is available to state and local agencies.

The Teale Data Center compiled their 1997-published road mileage estimates by scanning cartographic maps to produce digital line graphs (DLG).  Caltrans compiles its road mileage data from reports of paved road mileages submitted by counties, cities, federal agencies, and state agencies in California.  Due to the difference in compilation techniques, the two data sets can differ significantly.  Queries of the Teale data generally produce larger estimates of county road mileages than those reported by Caltrans.  The differences tend to be pronounced in rural counties (around a factor of 2), but insignificant in urbanized counties.  We opted to use the Caltrans data to estimate total state emissions because the reporting agencies are the major consumers of asphalt products in the state.  We opted to disaggregate emissions spatially by using the Teale data because this is consistent with the database of spatial surrogates that are currently being used by the ARB (Funk et al, 2001).