# Attachment D:  Bakeries

This attachment covers small-scale and large-scale bakeries (EIC No. 420-412-6012-0000, CES No. 57349).  Yeast fermentation produces ethanol and small amounts of other VOCs, which are driven off at high temperatures during baking.

Types of baking with yeast:

- Straight-dough process:  All ingredients are mixed together at one time and the dough is allowed to rise (or leaven) for 1-2 hours before baking.  In general, small-scale bakeries use this process.

- Sponge-dough process:  At first, only about half of the flour is mixed together with nearly all of the liquid ingredients and the yeast.  This mixture is allowed to rise very slowly and for a longer period of time (6 to 10 hours), which produces a “sponge”.  The remaining flour and liquid ingredients are then mixed with the sponge to produce the dough for baking.  In general, large-scale bakeries use this process.

Methods To Calculate Emissions

Survey or Self-Reports

AP-42, Chapter 9.9.6 recommends an emission factor calculation that depends upon detailed information about the baking process.

VOC E.F. = 0.95Yi + 0.195ti  0.51S  0.86ts + 1.90	[1]

where:

VOC E.F. 	= 	pounds VOC per ton of baked bread

Yi 	= 	initial baker’s percent of yeast

ti 	= 	total yeast action time in hours

S 	= 	final (spike) baker’s percent of yeast

ts 	= 	spiking time in hours

The “spike” and “spiking time” refer to a second stage of leavening when additional yeast is added to the pre-risen dough.  Spiking is employed very infrequently.  Typical yeast percentages are 1-2%.  Commercial bakeries tend to use the higher end of this range, while small-scale bakers tend to use the lower end.

Example calculation for a sponge-dough recipe (Yi = 2%, ti  = 9 hr):

VOC E.F. = 0.95 (2) + 0.195 (9)  0.51 (0)   0.86 (0) + 1.90 = 5.6 lb/ton

Example calculation for a straight-dough recipe (Yi  = 1%, ti  = 5 hr):

VOC E.F. = 0.95 (1) + 0.195 (5)  0.51 (0)   0.86 (0) + 1.90 = 3.8 lb/ton

Top-Down Calculations

Emission Factors

In the absence of detailed process information, the EIIP recommends emission factors of 1 lb VOC per ton of bread produced by the straight-dough process and 10-16 lb VOC per ton of bread produced by the sponge-dough process (Emission Inventory Improvement Program, 1999).  We plan to apply mid-range emission factors taken from the EIIP document (1 lb/ton for straight-dough baking and 13 lb/ton for sponge-dough baking).

For a 1990 inventory, the Bay Area Air Quality Management District performed source testing of large-scale and small-scale bakeries (Bay Area Air Quality Management District, 1992).  Emission factors for large-scale bakeries were determined to range between 0.6-14 lb VOC per ton of bread produced.  For small-scale bakeries, source testing resulted in an average emission factor of 5.4 lb VOC per ton of bread produced.  These source test results are reasonably consistent the EIIP document, but demonstrate that the uncertainty in the emissions estimates probably ranges between a factor of 5 to 10.

In order to select and apply the emission factors appropriately, we will assume that large-scale bakeries with more than 50 employees employ the sponge-dough process, while all other bakeries employ the straight-dough process.

Activity Data

The required activity parameter is the quantity of leavened bread products baked by small-scale and large-scale establishments in a county for the year 2000.  We will use the following data products from the US Census Bureau to generate these estimates:

- Quantities and values of leavened baked products shipped by retail and commercial manufacturing bakeries in the State of California in 1997.   (Assume shipments equal production.)

- County employment figures and facility counts from 1999 for retail and commercial manufacturing bakeries.

- Sales of baked products produced on-site at retail food stores in the US and county-level sales by food stores in 1997.

- Growth in sales of retail and commercial bread products for the nation and the State of California from 1997 to 2000.

In addition, we are in the process of acquiring from the California Food and Drug Administration a list of commercial baking facilities in the CCOS II counties.  Other information, such as employment at these facilities, may be possible to obtain but is still to be determined.  We will use this information to quality assure the facility counts and/or employment data from the US Census Bureau.

Calculations for the Commercial Bakeries Manufacturing Sector

For the US in 1997, the quantities and values of commercial product shipments of leavened baked goods are listed in Table 1 (US Census Bureau, 1999).  In addition, California’s contribution to the nationwide total was $2,467,079,000 of $21,459,322,000 (or 11.5%).  From the data in Table 1, the ratios of product weight per dollar value for the major product class categories were estimated.  An example for the major product class, Bread (NAIC = 3118121xxx) is shown below and results for all categories are shown in Table 2.

5,609 + 273 + 1,184 + 403 + 1,582 + 64 + 385 + 498 + 1243,480,995 + 105,908 + 967,567 + 308,119 + 1,268,830 + 43,999 + 307,850 + 467,008 + 76,198=0.00144million lbsthousand $

The dollar values of leavened, baked products shipped by California commercial bakeries in 1997 are listed in Table 3 (US Census Bureau, 1999).  By applying the ratios listed in Table 2 to the values shown in Table 3, the quantities of commercial product shipments for California in 1997 are estimated as shown below for the product class, Bread (see Table 4 for all results).

1,005,785  0.00144 = 1,149 million pounds of bread shipped

| Table 1. 1997 US commercial product shipments of leavened baked goods. |  |  |  |
| --- | --- | --- | --- |
| NAICS | Product Description | 1997 Quantity (million pounds) | 1997 Value ($1000) |
| 3118121 | Bread (white, wheat, and rye, etc.), inc. frozen |  |  |
| 3118121111 | White pan bread, except frozen | 5,609 | 3,480,995 |
| 3118121121 | Frozen white pan bread | 273 | 105,908 |
| 3118121231 | White hearth bread, except frozen (inc. french, italian, etc.) | 1,184 | 967,567 |
| 3118121241 | Frozen white hearth bread (inc. french, italian, etc.) | 403 | 308,119 |
| 3118121351 | Whole wheat, cracked wheat, multigrain, and other dark wheat breads, except frozen | 1,582 | 1,268,830 |
| 3118121361 | Frozen whole wheat, cracked wheat, multigrain, and other dark wheat breads | 64 | 43,999 |
| 3118121471 | Rye bread (inc. pumpernickel), except frozen | 385 | 307,850 |
| 31181214G1 | Other variety breads (raisin, potato, self rising, salt free, canned, etc.), except frozen | 498 | 467,008 |
| 31181214J1 | Other frozen variety breads (raisin, potato, self rising, salt free, canned, etc.) | 124 | 76,198 |
| 3118124 | Rolls (bread type), muffins, bagels, and croissants |  |  |
| 3118124111 | Hamburger and wiener rolls, except frozen | 3,442 | 2,294,522 |
| 3118124121 | Frozen hamburger and wiener rolls | 196 | 111,526 |
| 3118124231 | Brown and serve rolls, except frozen | 320 | 258,764 |
| 3118124241 | Frozen brown and serve rolls | 16 | 13,922 |
| 3118124251 | English muffins, except frozen | 379 | 416,170 |
| 3118124261 | Frozen english muffins | 32 | 20,213 |
| 3118124271 | Hearth rolls, except frozen | 281 | 261,202 |
| 3118124281 | Frozen hearth rolls | 62 | 55,048 |
| 3118124291 | Bagels, except frozen | 291 | 241,342 |
| 31181242A1 | Frozen bagels | 818 | 611,887 |
| 31181242B1 | Croissants, except frozen | 21 | 45,162 |
| 31181242C1 | Frozen croissants | 86 | 123,137 |
| 31181242D1 | Other bread type rolls (kaiser except hearth type, parkerhouse, etc.), except frozen | 389 | 369,185 |
| 31181242E1 | Other frozen bread type rolls (kaiser except hearth type, parkerhouse, etc.) | 192 | 101,209 |
| 31181242F1 | Bread stuffing, croutons, and bread crumbs (plain and seasoned) | 590 | 299,443 |
| 311812D1 | Other sweet goods, except frozen | 1,474 | 2,020,919 |
| 311812D111 | Yeast raised doughnuts, except frozen | 229 | 308,632 |
| 311812D181 | All other sweet goods containing yeast, except frozen (inc. sweet rolls and coffeecake) | 455 | 563,879 |

Table 2.  Ratios of product weight per dollar value for major baked product class categories.

| NAICS | Product Class | Ratio (million lbs per $1000) |
| --- | --- | --- |
| 3118121 | Bread (white, wheat, and rye, etc.), including frozen | 0.00144 |
| 3118124 | Rolls (bread ~ type), muffins, bagels, and croissants | 0.00136 |
| 311812D | Other sweet goods, except frozen [those containing yeast only] | 0.00073 [0.00078] |

Table 3.   Monetary values of baked products shipped by

California commercial bakeries in 1997.

|  |  | Value of product shipments ($1,000) |  |
| --- | --- | --- | --- |
| NAICS | Product Class | CA | US |
| 3118121 | Bread (white, wheat, and rye, etc.), including frozen | 1,005,785 | 7,190,313 |
| 3118124 | Rolls (bread type), muffins, bagels, and croissants | 582,654 | 5,276,466 |
| 311812D | Other sweet goods, except frozen [those containing yeast only] | 202,320 [87,350] | 2,046,923 [883,738] |

Table 4.   Estimated quantities of baked products shipped by

California commercial bakeries in 1997.

|  |  | Quantity of product shipments (million lbs.) |  |
| --- | --- | --- | --- |
| NAICS | Product Class | CA | US |
| 3118121 | Bread (white, wheat, and rye, etc.), Including frozen | 1,449 | 10,359 |
| 3118124 | Rolls (bread ~ type), muffins, bagels, and croissants | 793 | 7,185 |
| 311812D | Other sweet goods, except frozen [those containing yeast only] | 148 [68] | 1,493 [692] |
|  | Total leavened products | 2,311 | 18,236 |

California’s commercial sector “Bakeries and Tortilla Manufacturing” increased from $4.36 billion in 1997 to $4.60 billion in 1998 to $5.10 billion in 1999 (US Census Bureau, 2001).  If this growth trend continued into 2000, production of commercial leavened baked goods should have reached 2,878 million pounds (or, 2311  5.43  4.36).

Example Calculations for the Retail Bakeries Manufacturing Sector

Fewer California-specific data are available for retail manufacturing bakeries than for commercial bakeries.  Therefore, nationwide data must be disaggregated to estimate California production of baked goods by retail manufacturing bakeries.  For the US in 1997, the quantities and values of retail product shipments of leavened baked goods are listed in Table 5 (US Census Bureau, 1999b).  Estimated values are indicated with brackets.  In addition, California’s contribution to the nationwide total was $301,615,000 of $1,947,986,000 (or 15.5%).

| Table 5. Monetary values of baked products shipped by US retail manufacturing bakeries in 1997. |  |  |  |  |
| --- | --- | --- | --- | --- |
| NAIC | Product | Value of US Shipments ($1000) | Percent of Total |  |
| 3118110111 | Breads and rolls (excluding bagels) | 289,592 | 15% |  |
| 3118110121 | Bagels | 18,684 | 1% |  |
| 3118110151 | Doughnuts | 51,726 | 3% |  |
|  | [Leavened doughnuts, estimated] | [18,369]b |  |  |
| 31181101V1 | Other sweet goods (sweet rolls, coffeecake, pastries, danishes, muffins, etc) | 168,084 | 9% |  |
|  | [Leavened sweet goods, estimated] | [92,097]b |  |  |
|  | Nonleavened cakes, cookies, and pies | 391,122 | 20% |  |
| 3118110Y | Retail bakery products (not separated by kind) | 996,343 | 52% |  |
|  | [Breads and rolls, estimated] | [313,893]a |  |  |
|  | [Bagels, estimated] | [20,252]a |  |  |
|  | [Doughnuts, estimated] | [56,067]a |  |  |
|  | [Leavened doughnuts, estimated] | [19,910]a,b |  |  |
|  | [Other sweet goods, estimated] | [182,189]a |  |  |
|  | [Leavened sweet goods, estimated] | [99,825]a,b |  |  |
|  | [Nonleavened, estimated] | [423,943]a |  |  |
|  | Total leavened baked goods | 872,622 | 46% |  |
|  | Total baked goods | 1,915,551 | 100% |  |

a  Estimate was calculated from the distribution of products listed in Table 5 that are separated by kind.

b – Estimates were calculated from the ratio of leavened:unleavened observed in the commercial data set.

Similar to the method used to produce Table 2, ratios of product weight per dollar value for the major product class categories can be estimated if the data from Table 1 are sorted to better match the distribution of retail product categories listed in Table 5.  In addition, the fraction of the US total market for baked goods attributable to California (15.5%) can be used to estimate California-specific production of leavened baked goods for 1997.  Example calculations shown below are for the category “breads and rolls (excluding bagels)” and all results are tabulated in Table 6.

Ratio of production per dollar value of breads and rolls:

5,609 + 273 + 1,184 + 403 + 1,582 + 64 + 385 + 498 + 124+3,442 + 196 + 320 + 16 + 379 + 32 + 281 + 62  + 21

Estimated total value of US shipments of breads and rolls:

289,592 + 313,893 = 603,485 thousand dollars

Estimated total quantity of US shipments of breads and rolls:

603,485  0.00142 = 857 million pounds

Estimated total quantity of California shipments of breads and rolls:

857  15.5% = 133 million pounds

Table 6.  Estimated quantities of baked products shipped by

US and California retail bakeries in 1997.

|  |  | Ratio (million lbs | Quantity of product shipments (million lbs) |  |
| --- | --- | --- | --- | --- |
| NAICS | Product Class | per $1000) | CA | US |
| 3118110111 | Breads and rolls (excluding bagels) | 0.0014 | 133 | 857 |
| 3118110121 | Bagels | 0.0013 | 9 | 55 |
| 3118110151 | Doughnuts | 0.00073 | 24 | 153 |
|  | [Leavened doughnuts] | [0.00074] | [8] | [54] |
| 31181101V1 | Other sweet goods | 0.00073 | 77 | 497 |
|  | [Leavened sweet goods] | [0.00081] | [42] | [273] |
|  | Total nonleavened goods | 0.00077 | 179 | 1,157 |
|  | Total leavened baked goods |  | 192 | 1,239 |

Nationwide, 2000 sales for retail bakeries to be valued at 8,012 million dollars, and 1997 sales were valued at 6,096 million dollars (US Census Bureau, 2001b).  Based on this growth trend, nationwide production of retail baked goods in 2000 is estimated to reach 1,628 million pounds (or, 8,012  6096   1,239).  If this trend also holds true for California, 2000 production is estimated to be 252 million pounds (or, 8,012  6096  192).  If the growth rate of California’s retail production more closely resembles that of commercial production, then the estimate would be 239 million pounds (or 5.43  4.36  192), which agrees reasonably well with the former estimate.

Disaggregation of Statewide Manufacturing Production Data to the County Level

Recall that statewide production of leavened baked goods for retail bakeries and commercial bakeries in the manufacturing sector were estimated to be 2,878 million pounds and 252 million pounds for the year 2000, respectively.  The US Census Bureau “County Business Patterns” database has employment and facility count data for commercial and retail manufacturing bakeries by state and county, which were used to disaggregate the production estimates to the county level.  The County Business Patterns database contains 1999 estimates of the following items for the state and counties (US Census Bureau, 2001c).  The 1999 distribution of employment by counties was assumed to be reasonably representative of the distribution for the year 2000.

- Employment in the retail bakery manufacturing sector

- Employment in the commercial bakery manufacturing sector

- Counts of facilities distributed by staff size (commercial and retail bakeries combined)

From the counts of facilities by staff size, we first estimated the relative ratios of employment at small establishments to employment at large establishments.  (A small establishment was defined as having fewer than 50 employees, while a large establishment was defined as having 50 or more employees.)  According to these ratios, we distributed commercial and retail employment figures to estimate retail and commercial employment by size of establishment, small or large (see Table 7).  We apportioned the commercial and retail production of bakery products proportionally employment estimates (see Table 8).

| Table 7. Estimated employment in the commercial and retail baking sectors in California for 1999. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Commercial Employment (no. employees) |  |  | Retail Employment (no. employees) |  |  |
| County |  | at small establishments | at large establishments | at small establishments | at large establishments |  |
| Alameda |  | 229 | 1,746 | 360 | 0 |  |
| Amador |  | - | - | - | - |  |
| Butte |  | 0 | 0 | 10 | 0 |  |
| Calaveras |  | 10 | 0 | 10 | 0 |  |
| Colusa |  | 0 | 0 | 10 | 0 |  |
| Contra Costa |  | 89 | 163 | 104 | 0 |  |
| Del Norte |  | 0 | 0 | 10 | 0 |  |
| El Dorado |  | 0 | 0 | 10 | 0 |  |
| Fresno |  | 89 | 549 | 131 | 0 |  |
| Glenn |  | - | - | - | - |  |
| Humboldt |  | 28 | 146 | 60 | 0 |  |
| Imperial |  | 0 | 0 | 10 | 0 |  |
| Inyo |  | - | - | - | - |  |
| Kern |  | 87 | 88 | 60 | 0 |  |
| Kings |  | 0 | 0 | 10 | 0 |  |
| Lake |  | - | - | - | - |  |
| Lassen |  | - | - | - | - |  |
| Los Angeles |  | 1,714 | 5,780 | 2,302 | 643 |  |
| Madera |  | 0 | 0 | 10 | 0 |  |
| Marin |  | 60 | 0 | 34 | 0 |  |
| Mariposa |  | - | - | - | - |  |
| Mendocino |  | 10 | 0 | 10 | 0 |  |
| Merced |  | 10 | 0 | 10 | 0 |  |
| Modoc |  | - | - | - | - |  |
| Mono |  | 0 | 0 | 10 | 0 |  |
| Monterey |  | 33 | 61 | 165 | 0 |  |
| Napa |  | 63 | 0 | 60 | 0 |  |
| Nevada |  | 10 | 0 | 10 | 0 |  |
| Orange |  | 246 | 1,496 | 545 | 151 |  |
| Placer |  | 10 | 0 | 60 | 0 |  |
| Plumas |  | 0 | 0 | 10 | 0 |  |
| Riverside |  | 96 | 70 | 175 | 0 |  |
| Sacramento |  | 130 | 1,089 | 175 | 0 |  |
| San Benito |  | 0 | 0 | 60 | 0 |  |
| San Bernardino |  | 60 | 0 | 77 | 53 |  |
| San Diego |  | 520 | 683 | 750 | 0 |  |
| San Francisco |  | 194 | 751 | 315 | 59 |  |
| San Joaquin |  | 0 | 405 | 94 | 0 |  |
| San Luis Obispo |  | 63 | 76 | 60 | 0 |  |
| San Mateo |  | 200 | 705 | 125 | 50 |  |
| Santa Barbara |  | 60 | 0 | 77 | 0 |  |
| Santa Clara |  | 121 | 136 | 375 | 0 |  |
| Santa Cruz |  | 142 | 144 | 29 | 120 |  |
| Shasta |  | 0 | 60 | 10 | 0 |  |
| Sierra |  | - | - | - | - |  |
| Siskiyou |  | 0 | 0 | 10 | 0 |  |
| Solano |  | 10 | 0 | 60 | 0 |  |
| Sonoma |  | 188 | 417 | 60 | 0 |  |
| Stanislaus |  | 0 | 0 | 175 | 0 |  |
| Sutter |  | 0 | 0 | 10 | 0 |  |
| Tehama |  | - | - | - | - |  |
| Trinity |  | - | - | - | - |  |
| Tulare |  | 10 | 0 | 10 | 0 |  |
| Tuolumne |  | - | - | - | - |  |
| Ventura |  | 11 | 163 | 175 | 0 |  |
| Yolo |  | 60 | 0 | 10 | 0 |  |
| Yuba |  | 10 | 0 | 10 | 0 |  |
| California |  | 4,557 | 14,728 | 6,836 | 1,076 |  |

Dashes indicate counties for which no manufacturing employment data were reported.  Assumed negligible.

| Table 8. Estimated year-2000 production of leavened bread products for California counties. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Commercial Production (thousand tons) |  |  | Retail Production (thousand tons) |  |  |
| County |  | at small establishments | at large establishments | at small establishments | at large establishments |  |
| Alameda |  | 17.1 | 130.3 | 5.7 | 0.0 |  |
| Amador |  | - | - | - | - |  |
| Butte |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Calaveras |  | 0.7 | 0.0 | 0.2 | 0.0 |  |
| Colusa |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Contra Costa |  | 6.7 | 12.2 | 1.7 | 0.0 |  |
| Del Norte |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| El Dorado |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Fresno |  | 6.6 | 41.0 | 2.1 | 0.0 |  |
| Glenn |  | - | - | - | - |  |
| Humboldt |  | 2.1 | 10.9 | 0.9 | 0.0 |  |
| Imperial |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Inyo |  | - | - | - | - |  |
| Kern |  | 6.5 | 6.5 | 0.9 | 0.0 |  |
| Kings |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Lake |  | - | - | - | - |  |
| Lassen |  | - | - | - | - |  |
| Los Angeles |  | 127.9 | 431.3 | 36.7 | 10.2 |  |
| Madera |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Marin |  | 4.4 | 0.0 | 0.5 | 0.0 |  |
| Mariposa |  | - | - | - | - |  |
| Mendocino |  | 0.7 | 0.0 | 0.2 | 0.0 |  |
| Merced |  | 0.7 | 0.0 | 0.2 | 0.0 |  |
| Modoc |  | - | - | - | - |  |
| Mono |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Monterey |  | 2.5 | 4.5 | 2.6 | 0.0 |  |
| Napa |  | 4.7 | 0.0 | 0.9 | 0.0 |  |
| Nevada |  | 0.7 | 0.0 | 0.2 | 0.0 |  |
| Orange |  | 18.4 | 111.6 | 8.7 | 2.4 |  |
| Placer |  | 0.7 | 0.0 | 0.9 | 0.0 |  |
| Plumas |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Riverside |  | 7.2 | 5.2 | 2.8 | 0.0 |  |
| Sacramento |  | 9.7 | 81.3 | 2.8 | 0.0 |  |
| San Benito |  | 0.0 | 0.0 | 0.9 | 0.0 |  |
| San Bernardino |  | 4.4 | 0.0 | 1.2 | 0.8 |  |
| San Diego |  | 38.8 | 51.0 | 11.9 | 0.0 |  |
| San Francisco |  | 14.5 | 56.0 | 5.0 | 0.9 |  |
| San Joaquin |  | 0.0 | 30.2 | 1.5 | 0.0 |  |
| San Luis Obispo |  | 4.7 | 5.7 | 0.9 | 0.0 |  |
| San Mateo |  | 14.9 | 52.6 | 2.0 | 0.8 |  |
| Santa Barbara |  | 4.4 | 0.0 | 1.2 | 0.0 |  |
| Santa Clara |  | 9.0 | 10.2 | 6.0 | 0.0 |  |
| Santa Cruz |  | 10.6 | 10.8 | 0.5 | 1.9 |  |
| Shasta |  | 0.0 | 4.4 | 0.2 | 0.0 |  |
| Sierra |  | - | - | - | - |  |
| Siskiyou |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Solano |  | 0.7 | 0.0 | 0.9 | 0.0 |  |
| Sonoma |  | 14.0 | 31.1 | 0.9 | 0.0 |  |
| Stanislaus |  | 0.0 | 0.0 | 2.8 | 0.0 |  |
| Sutter |  | 0.0 | 0.0 | 0.2 | 0.0 |  |
| Tehama |  | - | - | - | - |  |
| Trinity |  | - | - | - | - |  |
| Tulare |  | 0.7 | 0.0 | 0.2 | 0.0 |  |
| Tuolumne |  | - | - | - | - |  |
| Ventura |  | 0.8 | 12.2 | 2.8 | 0.0 |  |
| Yolo |  | 4.4 | 0.0 | 0.2 | 0.0 |  |
| Yuba |  | 0.7 | 0.0 | 0.2 | 0.0 |  |
| California |  | 340 | 1,099 | 109 | 17 |  |

Dashes indicate counties for which no manufacturing employment data were reported.  Assumed negligible.

Example Calculations for Grocery and Specialty Food Stores in the Retail Sector

In addition to the manufacturing business areas discussed above, the retail sector includes groceries and specialty food stores that produce baked goods on site for direct sale to the public.  In 1997, the values of US shipments of baked goods from bakeries in the manufacturing sector were as follows (US Census Bureau, 1999a; US Census Bureau, 1999b):

- Manufacturing sector – Retail bakeries  $ 1,947,986,000

- Manufacturing sector – Commercial bakeries  $21,459,322,000

1997 US sales of on-site baked goods in the retail sector were as follows (US Census Bureau, 2001d):

- Retail sector – Groceries – Baked products baked on site   $ 5,335,885,000

- Retail sector – Specialty food stores – Baked products baked on site  $72,490,000

The scale of on-site bakery activities in the retail sector is large enough to warrant consideration because it is comparable to the scale of the manufacturing sector.  Note that we are assuming at this time that the dollar values assigned to manufacturing shipments and retail sales are formulated on a comparable basis (i.e., manufacturing shipments are not reported in terms of wholesale value).  Total 1997 food and beverage store sales in California were valued at $48,767,273,000 (of $ 368,250,471,000 for the US) and are broken down by county in Table 9 (US Census Bureau, 1999c).  These data were used to apportion US retail sales of on-site baked goods to California and its counties as follows.

For the State of California in 1997, value of on-site baked goods:

48,767,273,000  368,250,471,000  (5,335,885,000 + 72,490,000) = $716,229,000

Only a portion of these sales of baked goods are attributable to leavened products. The value of all baked goods shipped by California commercial manufacturing bakeries was $2,467,079,000 and the value of all baked goods shipped by US retail manufacturing bakeries was $1,915,551,000 (US Census Bureau, 1999a; US Census Bureau, 1999b).  In comparison, Table 3 shows that the value of leavened products shipped by California commercial manufacturing bakeries was $1,675,789,000 (or 67.9% of the total for all baked goods) and Table 5 shows that the value of leavened products shipped by US retail manufacturing bakeries was $872,621,000 (or 45.6% of the total for all baked goods).  The figure for retail manufacturing bakeries is probably more representative of specialty food stores and groceries, so we will use that percentage to estimate the value of leavened goods baked on site in California.

For the State of California in 1997, value of leavened on-site baked goods:

$716,229,000  46% = $329,465,000

If we assume the same distribution of products as that shown in Table 6, a weighted average weight-to-dollar ratio for leavened products can be calculated as follows:

[(0.0014  133) + (0.0013  9) + (0.00074  8) + (0.00081  42)] 192

= 0.0012 million lbs per thousand dollars

The quantity of leavened on-site baked goods can be estimated as follows for the State of California:

0.0012  $329,465,000  1000 = 376 million pounds

Table 9 shows the results of all the calculations such as the examples discussed above.

| Table 9. Estimated 1997 sales figures in California for the retail food and beverage stores sector. |  |  |  |  |
| --- | --- | --- | --- | --- |
| County | Sales for Food and Beverage Stores ($1000) | Est. Sales of On-Site Baked Goods ($1000) | Est. Sales of Leavened On-Site Baked Goods ($1000) | Est. Sales of Leavened On-Site Baked Goods (thousand tons) |
| Alameda | 2,476,451 | 36,371 | 16,731 | 10.36 |
| Amador | 60,309 | 886 | 407 | 0.25 |
| Butte | 318,026 | 4,671 | 2,149 | 1.33 |
| Calaveras | D | - | - | - |
| Colusa | D | - | - | - |
| Contra Costa | 1,486,375 | 21,830 | 10,042 | 6.22 |
| Del Norte | 41,191 | 605 | 278 | 0.17 |
| El Dorado | 277,231 | 4,072 | 1,873 | 1.16 |
| Fresno | 1,031,767 | 15,153 | 6,970 | 4.32 |
| Glenn | 27,870 | 409 | 188 | 0.12 |
| Humboldt | 244,097 | 3,585 | 1,649 | 1.02 |
| Imperial | 190,048 | 2,791 | 1,284 | 0.80 |
| Inyo | 33,431 | 491 | 226 | 0.14 |
| Kern | 815,102 | 11,971 | 5,507 | 3.41 |
| Kings | 144,094 | 2,116 | 973 | 0.60 |
| Lake | 96,982 | 1,424 | 655 | 0.41 |
| Lassen | D | - | - | - |
| Los Angeles | 12,671,512 | 186,102 | 85,607 | 53.02 |
| Madera | 161,997 | 2,379 | 1,094 | 0.68 |
| Marin | 553,413 | 8,128 | 3,739 | 2.32 |
| Mariposa | D | - | - | - |
| Mendocino | 185,253 | 2,721 | 1,252 | 0.78 |
| Merced | 224,773 | 3,301 | 1,519 | 0.94 |
| Modoc | D | - | - | - |
| Mono | 30,925 | 454 | 209 | 0.13 |
| Monterey | 574,514 | 8,438 | 3,881 | 2.40 |
| Napa | 247,602 | 3,636 | 1,673 | 1.04 |
| Nevada | 179,588 | 2,638 | 1,213 | 0.75 |
| Orange | 4,273,921 | 62,770 | 28,874 | 17.88 |
| Placer | 446,966 | 6,564 | 3,020 | 1.87 |
| Plumas | 39,628 | 582 | 268 | 0.17 |
| Riverside | 1,985,443 | 29,160 | 13,413 | 8.31 |
| Sacramento | 1,749,686 | 25,697 | 11,821 | 7.32 |
| San Benito | 74,730 | 1,098 | 505 | 0.31 |
| San Bernardino | 2,248,526 | 33,023 | 15,191 | 9.41 |
| San Diego | 3,758,006 | 55,193 | 25,389 | 15.73 |
| San Francisco | 1,265,916 | 18,592 | 8,552 | 5.30 |
| San Joaquin | 709,442 | 10,419 | 4,793 | 2.97 |
| San Luis Obispo | 404,032 | 5,934 | 2,730 | 1.69 |
| San Mateo | 1,257,062 | 18,462 | 8,493 | 5.26 |
| Santa Barbara | 660,836 | 9,705 | 4,465 | 2.77 |
| Santa Clara | 2,617,092 | 38,436 | 17,681 | 10.95 |
| Santa Cruz | 424,114 | 6,229 | 2,865 | 1.77 |
| Shasta | 275,131 | 4,041 | 1,859 | 1.15 |
| Sierra | D | - | - | - |
| Siskiyou | 76,582 | 1,125 | 517 | 0.32 |
| Solano | 561,354 | 8,244 | 3,792 | 2.35 |
| Sonoma | 898,609 | 13,198 | 6,071 | 3.76 |
| Stanislaus | 633,557 | 9,305 | 4,280 | 2.65 |
| Sutter | 138,916 | 2,040 | 938 | 0.58 |
| Tehama | 80,594 | 1,184 | 544 | 0.34 |
| Trinity | 15,320 | 225 | 103 | 0.06 |
| Tulare | 458,762 | 6,738 | 3,099 | 1.92 |
| Tuolumne | 95,525 | 1,403 | 645 | 0.40 |
| Ventura | 1,087,333 | 15,969 | 7,346 | 4.55 |
| Yolo | 253,105 | 3,717 | 1,710 | 1.06 |
| Yuba | 67,663 | 994 | 457 | 0.28 |
| California | 48,767,273 | 716,229 | 329,465 | 204.06 |

D = Data were withheld by the Census Bureau due to confidentiality concerns.  Assumed negligible.

The same 1997-to-2000 growth factor was used as that applied for retail manufacturing bakeries.  Year-2000 retail production is estimated to be 268 million pounds (or, 8,012  6096  204).

Estimation of Emissions

We applied the selected emission factors (13 lb VOC per ton of bread for large establishments and 1 lb VOC per ton of bread for small establishments and for the retail sector) to generate the emissions estimates presented in Table 10.  Note that these are uncontrolled emissions estimates.  Several counties, such as Sacramento, have enacted rules to control VOC emissions from larger bakeries.

| Table 10. Estimated year-2000 uncontrolled VOC emissions from the production of leavened bread products for California counties. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VOC Emissions for Commercial Manufacturing Operations (tons) |  | VOC Emissions for Retail Manufacturing Operations (tons) |  | VOC Emissions for | Total VOC |  |
| County | small establishments | large establishments | small establishments | large establishments | the Retail Sector (tons) | Emissions (tons) |  |
| Alameda | 8.56 | 846.64 | 2.87 | 0.00 | 6.81 | 864.88 |  |
| Amador | - | - | - | - | 0.17 | 0.17 |  |
| Butte | 0.00 | 0.00 | 0.08 | 0.00 | 0.87 | 0.95 |  |
| Calaveras | 0.35 | 0.00 | 0.08 | 0.00 | - | 0.43 |  |
| Colusa | 0.00 | 0.00 | 0.08 | 0.00 | - | 0.08 |  |
| Contra Costa | 3.33 | 78.99 | 0.83 | 0.00 | 4.09 | 87.23 |  |
| Del Norte | 0.00 | 0.00 | 0.08 | 0.00 | 0.11 | 0.19 |  |
| El Dorado | 0.00 | 0.00 | 0.08 | 0.00 | 0.76 | 0.84 |  |
| Fresno | 3.32 | 266.32 | 1.04 | 0.00 | 2.84 | 273.51 |  |
| Glenn | - | - | - | - | 0.08 | 0.08 |  |
| Humboldt | 1.06 | 70.84 | 0.47 | 0.00 | 0.67 | 73.05 |  |
| Imperial | 0.00 | 0.00 | 0.08 | 0.00 | 0.52 | 0.60 |  |
| Inyo | - | - | - | - | 0.09 | 0.09 |  |
| Kern | 3.24 | 42.46 | 0.47 | 0.00 | 2.24 | 48.42 |  |
| Kings | 0.00 | 0.00 | 0.08 | 0.00 | 0.40 | 0.47 |  |
| Lake | - | - | - | - | 0.27 | 0.27 |  |
| Lassen | - | - | - | - | - | - |  |
| Los Angeles | 63.96 | 2803.17 | 18.33 | 66.58 | 34.84 | 2986.88 |  |
| Madera | 0.00 | 0.00 | 0.08 | 0.00 | 0.45 | 0.52 |  |
| Marin | 2.22 | 0.00 | 0.27 | 0.00 | 1.52 | 4.01 |  |
| Mariposa | - | - | - | - | - | - |  |
| Mendocino | 0.35 | 0.00 | 0.08 | 0.00 | 0.51 | 0.94 |  |
| Merced | 0.35 | 0.00 | 0.08 | 0.00 | 0.62 | 1.05 |  |
| Modoc | - | - | - | - | - | - |  |
| Mono | 0.00 | 0.00 | 0.08 | 0.00 | 0.09 | 0.16 |  |
| Monterey | 1.24 | 29.53 | 1.31 | 0.00 | 1.58 | 33.66 |  |
| Napa | 2.35 | 0.00 | 0.47 | 0.00 | 0.68 | 3.51 |  |
| Nevada | 0.35 | 0.00 | 0.08 | 0.00 | 0.49 | 0.92 |  |
| Orange | 9.19 | 725.39 | 4.34 | 15.65 | 11.75 | 766.32 |  |
| Placer | 0.35 | 0.00 | 0.47 | 0.00 | 1.23 | 2.06 |  |
| Plumas | 0.00 | 0.00 | 0.08 | 0.00 | 0.11 | 0.18 |  |
| Riverside | 3.59 | 33.79 | 1.39 | 0.00 | 5.46 | 44.23 |  |
| Sacramento | 4.83 | 528.41 | 1.39 | 0.00 | 4.81 | 539.44 |  |
| San Benito | 0.00 | 0.00 | 0.47 | 0.00 | 0.21 | 0.68 |  |
| San Bernardino | 2.22 | 0.00 | 0.61 | 5.52 | 6.18 | 14.54 |  |
| San Diego | 19.40 | 331.21 | 5.97 | 0.00 | 10.33 | 366.91 |  |
| San Francisco | 7.23 | 364.29 | 2.51 | 6.11 | 3.48 | 383.63 |  |
| San Joaquin | 0.00 | 196.43 | 0.75 | 0.00 | 1.95 | 199.13 |  |
| San Luis Obispo | 2.33 | 37.07 | 0.47 | 0.00 | 1.11 | 40.99 |  |
| San Mateo | 7.45 | 342.02 | 0.99 | 5.18 | 3.46 | 359.10 |  |
| Santa Barbara | 2.22 | 0.00 | 0.61 | 0.00 | 1.82 | 4.65 |  |
| Santa Clara | 4.51 | 65.98 | 2.98 | 0.00 | 7.20 | 80.67 |  |
| Santa Cruz | 5.28 | 70.06 | 0.23 | 12.37 | 1.17 | 89.12 |  |
| Shasta | 0.00 | 28.86 | 0.08 | 0.00 | 0.76 | 29.69 |  |
| Sierra | - | - | - | - | - | - |  |
| Siskiyou | 0.00 | 0.00 | 0.08 | 0.00 | 0.21 | 0.29 |  |
| Solano | 0.35 | 0.00 | 0.47 | 0.00 | 1.54 | 2.37 |  |
| Sonoma | 7.00 | 202.39 | 0.47 | 0.00 | 2.47 | 212.34 |  |
| Stanislaus | 0.00 | 0.00 | 1.39 | 0.00 | 1.74 | 3.13 |  |
| Sutter | 0.00 | 0.00 | 0.08 | 0.00 | 0.38 | 0.46 |  |
| Tehama | - | - | - | - | 0.22 | 0.22 |  |
| Trinity | - | - | - | - | 0.04 | 0.04 |  |
| Tulare | 0.35 | 0.00 | 0.08 | 0.00 | 1.26 | 1.69 |  |
| Tuolumne | - | - | - | - | 0.26 | 0.26 |  |
| Ventura | 0.42 | 79.19 | 1.39 | 0.00 | 2.99 | 83.99 |  |
| Yolo | 2.22 | 0.00 | 0.08 | 0.00 | 0.70 | 2.99 |  |
| Yuba | 0.35 | 0.00 | 0.08 | 0.00 | 0.19 | 0.62 |  |
| State Total | 170 | 7,143 | 54 | 111 | 134 | 7,613 |  |

Dashes indicate unavailable data.  Assumed negligible.

Temporal Adjustments

Seasonal adjustments:  Nationwide, monthly retail trade figures for baked goods were estimated to be 596, 613,  656,  659,  671,  664,  652,  680,  664,  700,  691,  and 766 millions of dollars (January through December, 2000) (US Census Bureau, 2001b).  We will assume that this monthly pattern is valid for production rates.

Diurnal and day-of-week adjustments:  We have not yet found new sources of information for bakeries.  A query is pending with the CA FDA representative, who may be able to suggest a typical work-week for large-scale bakeries.

Air Basin Distribution

In order to distribute emissions for counties that are split between air basins, we considered the county-specific distributions of activity levels for commercial manufacturing and retail manufacturing of baked products (calculated above), and the county-specific distributions of estimated employment in the non-retail and service/commercial sectors, which ARB uses as spatial surrogates to distribute area source emissions (see Table 11).  Emissions for bakeries were distributed as follows:

FA,NRE  DNRE + FA,SCE  DSCE = WFA

WFA = Weighted fraction of bakery emissions for area A, where area A is defined by a county and air basin.

FA,NRE, FA,SCE = Distribution of non retail employment (NRE) or service/commercial employment (SCE) for area A.

DNRE, DSCE = Distribution of NRE-associated bakery activity or SCE-associated bakery activity for the county.

For example, the fraction of total Placer County-wide emissions that occurs in the Sacramento Valley Air Basin is estimated as follows:

WFA = 95.5%  43% + 95.8%  57% = 95.7

Table 11.  Distributions of area source surrogates and emissions.

| County | Air Basin | Distribution of Non-Retail Employment | Distribution of Service and Commercial Employment | Weighted Distribution of Bakery Emissions |
| --- | --- | --- | --- | --- |
| El Dorado | Lake Tahoe | 9.4% | 14.7% | 14.7% |
|  | Mountain Counties | 90.6% | 85.3% | 85.3% |
| Distribution of Bakery Activity |  | 0.0% | 100.0% |  |
| Placer | Lake Tahoe | 1.7% | 1.4% | 1.5% |
|  | Sacramento Valley | 95.5% | 95.8% | 95.7% |
|  | Mountain Counties | 2.8% | 2.8% | 2.8% |
| Distribution of Bakery Activity |  | 43.0% | 57.0% |  |
| Solano | San Francisco Bay Area | 77.0% | 76.2% | 76.5% |
|  | Sacramento Valley | 23.0% | 23.8% | 23.5% |
| Distribution of Bakery Activity |  | 43.0% | 57.0% |  |

Other Information/Considerations

Tips for conducting a survey of bakeries:

- Identify population to be surveyed through the local health department and the state Food and Drug Administration.

- Request production and operating schedule data:  weight of leavened baked products produced, hours and days of operation, typical baker’s percent of yeast used and rising time.  If a secondary leavening is used for some products, how much of the production follows this process?  What are the baker’s percent of yeast used and rising time for the secondary leavening?

References and Contacts

Bay Area Air Quality Management District (1992) Emission inventory methods documentation prepared by P. Samson of the Bay Area Air Quality Management District, San Francisco, CA, for submittal to the California Air Resources Board, Sacramento, CA.

Emission Inventory Improvement Program (1999) Baked Goods at Commercial/Retail Bakeries.  Area Source Method Abstract prepared by the Emission Inventory Improvement Program and the US Environmental Protection Agency, Research Triangle Park, NC, June.

US Census Bureau (2001) Annual Survey of Manufacturers  1999 Geographic Area Statistics, report no. M99(AS)-3 prepared by the US Census Bureau, Washington, D.C., April.  Accessed from web site http://www.census.gov/prod/2001pubs/m99-as3.pdf.

US Census Bureau (2001b) Monthly Retail Trade Survey.  Summary statistics prepared by the Monthly Retail Surveys Branch of the US Census Bureau, Washington, D.C., April.  Accessed from web site http://www.census.gov/mrts/www/data/html/sal00.html.

US Census Bureau, (2001c) CenStats Databases – County Business Patterns Data, database maintained by the US Census Bureau, Washington, D.C., and accessed June 2001 at web site http://tier2.census.gov/cbp_naics/index.html.

US Census Bureau (2001d) Retail Trade Series of the 1997 Economic Census  Merchandise Line Sales.   Report no. EC97R44S-LS(RV) prepared by the US Census Bureau, Washington, D.C., January.  Accessed from web site http://www.census.gov/prod/ec97/97r44-ls.pdf.

US Census Bureau (1999) Manufacturing Industry Series of the 1997 Economic Census – Commercial Bakeries.  Report no. EC97M-3118B prepared by the US Census Bureau, Washington, D.C., November.  Accessed from web site http://www.census.gov/prod/ec97/97m3118b.pdf.

US Census Bureau (1999b) Manufacturing Industry Series of the 1997 Economic Census – Retail Bakeries.  Report no. EC97M-3118A prepared by the US Census Bureau, Washington, D.C., December.  Accessed from web site http://www.census.gov/prod/ec97/97m3118a.pdf.

US Census Bureau (1999c) Retail Trade Series of the 1997 Economic Census  Geographic Area Series: California.   Report no. EC97R44S-LS(RV) prepared by the US Census Bureau, Washington, D.C., August.  Accessed from web site http://www.census.gov/prod/ec97/97r44-ca.pdf.

US War Department (1916) Manual for Army Bakers. Wash, DC: GPO, 1910. 89 p. UC733A5.

In fact, only one of several professional bakers we contacted knew of the term “spiking.”  He had been introduced to the term through AP-42 while he was evaluating emissions control devices for the baking ovens.

Note: This major category includes products made without yeast.