# Attachment F:  Agricultural Processing Losses

This attachment discusses emissions from facilities that process miscellaneous food and agricultural products intended for human or animal consumption (EIC No. 420-420-6000-0000, CES No. 47076).  As a result of an evaluation of economic data, this discussion is limited to three types of processing plants, listed below.  (Corresponding North American Industrial Classification System, or NAICS, codes are shown in parentheses.)

- Grain elevators and related processes (493130) and grain milling (31121X)

- Nuts processing, including salted and roasted nuts and seeds (311911), and almond processing (115114)

- Malt beverage manufacturing (312120)

Other types of food and agricultural products processing facilities, which were considered but not selected for further discussion, are listed below.

- Meat and animal by-products plants (3116XX), including meat packing plants, meat smokehouses, meat rendering plants, manure processing plants, and poultry slaughtering plants

- Natural and processed cheese factories (311513)

- Cotton gins (115111)

- Fruits and vegetables processing plants (31142X), including onion/garlic processing plants, fruits and vegetables canneries, dehydrated fruits and vegetables plants

- Other grain products manufacturing plants, including cereal breakfast food plants (311211), animal feed processors (3111XX), alfalfa dehydration plants (311119), and pasta manufacturers (311823)

- Sugar refiners and processors (31131X),

- Fats and oils processors (31122X), including vegetable oil manufacturers and corn wet mills

- Distilled spirits manufacturers (312140)

- Miscellaneous food and kindred products manufacturing, including fish processors (3117XX), coffee roasters (311920), deep-fried snack chip manufacturing (311919), yeast producers (311999, misc.), spice/flavoring manufacturers (311942, 311930), tobacco producers (3122XX), and leather tanners (316110)

For each industrial category listed above, economic activity was evaluated for the group of counties that are participating in the CCOS II Project.  (Employment and agricultural production data were gathered from the US Census Bureau and the US Department of Agriculture.)  The industries determined to have the greatest levels of employment in the CCOS II group of counties – 1,000 to 3,000 employees in the first quarter of 1999 – were fruit/vegetable canning, roasted nuts manufacturing, grain milling, and breweries (US Census Bureau, 2001a).  Levels of activity within the group of CCOS II counties also were compared to those for the State of California as a whole.  The industries with the greatest relative levels of activity – 25 to 58% of California’s total employment – were grain milling, roasted nuts manufacturing, farm products warehousing (most likely associated with grain or nuts processing), and breweries (US Census Bureau, 2001a).  (The shares fell below 15% for all other industrial categories listed above.)  Only those categories that had high levels of total employment and high levels of relative employment were selected for further analysis.

## Grain Processing

Grain processing facilities include mills (wheat, corn, rye, rice, animal feed, etc.), grain processing plants (barley malt manufacturers, animal feed processors, etc.), and grain storage elevators.  According to a summary of County Agricultural Commissioners’ Reports, rice is by far the largest grain crop grown in the CCOS II group of counties (California Agricultural Statistics Service, 2000a).  In 1999, rice production for the region was 1.96 million tons, or 97% of the state total (California Agricultural Statistics Service, 2000a).  From 1999 to 2000, California rice production grew 19% (California Agricultural Statistics Service, 2001).  The California Rice Commission (2001) verifies that more than 95 percent of California rice is grown in the Sacramento Valley.  In the CCOS II region, rice mills (NAICS code 311212) account for 83 to 97% of employment in the grain milling industry (US Census Bureau, 2001a).  In fact, at least 75% (75-92%) of California’s rice mill employees work in Yolo, Colusa, and Butte counties alone (US Census Bureau, 2001a).

Productions of corn and wheat in the CCOS II region, the second and third largest grain crops, were around 0.6 and 0.3 million tons (of 9.7 and 1.6 million tons statewide) (California Agricultural Statistics Service, 2000a).  Productions of other grain crops were negligible.  Employment in the CCOS II counties for flour milling, a common use of wheat, is around 14% (5-23%) of the statewide total.

### Emission Factors for Rice Processing

Processes at rice mills and grain elevators are primarily sources of particulate matter.  Processes include loading/unloading, conveying or transferring, cleaning, pressure soaking, steaming, drying, shelling (remove the husks), pearling (removing the bran and germ for white rice), sizing (for width), sorting (for color), grading (for brokenness), polishing (coating with glucose or talc), cooking, cooling, storage, flour milling, weighing, and packaging.  PM10 emission factors for these processes, which were acquired from the US EPA’s Compilation of Air Pollutant Emission Factors (AP-42) Chapter 9.9, are listed in Table 1.

Table 1.   Emission factors for rice processing facilities.

| Process | Emission factors (lb/ton of rice handled) |  |
| --- | --- | --- |
|  | PM | PM10 |
| Grain loading and unloading (hopper truck) | 0.035 | 0.0078 |
| Rice drying | 0.063 | 0.032 |
| Rice cleaning | 0.0031 | 0.0031 |
| Rice millhouse operations (inclusive of shelling, pearling, sizing, sorting, and grading) | 0.27 | 0.27 |
| Rice bran handling | 0.017 | 0.017 |
| Wheat flour milling – uncontrolled | 70 | 35 |

(Source: EPA, 2000)

The emission factor for wheat flour milling will be applied in lieu of a rice-specific factor.  Generally, flourmills do not operate in an uncontrolled mode, which corresponds to the emission factor of 70 lb PM10/ton shown in Table 1.  Therefore, it is assumed that mills operate with cyclones and/or baghouses, which usually achieve 98% or better control efficiencies.  In addition, it is assumed that control measures are applied to other processes with an arbitrarily selected efficiency of 85%.  Lastly, it is assumed that conveyance or transfer points generate emissions at rates similar to that of the loading/unloading process; and that the polishing process is similar in magnitude to other individual processes (0.05 lb/ton, as a conservatively high estimate).

Comprehensive emission factors for general rice milling and handling were calculated by considering typical process rates for rice mills and grain elevators.  The expression below represents a typical sequence of processes performed at a rice grain elevator.

Unload truck  transfer to initial storage  transfer to dryer  dry   transfer to primary cleaning  primary cleaning (1% impurities lost) transfer to storage  load for shipping to rice mill

The following equations calculate the cumulative PM emission factors for the sequence of grain elevator processes shown above.  Note that 1% of harvested rice is lost during the primary cleaning stage.  Because activity data is available in terms of rice harvested, proportions of 99% (0.99) are applied to correct for the reduced throughput at subsequent processes.

[0.0078 + 0.0078 + 0.0078 + 0.032 + 0.0078 + 0.0031 + (0.99  0.0078) + (0.99  0.0078)]  (1 – 0.85) = 0.012 lb PM10/ton

[0.035 + 0.035 + 0.035 + 0.063 + 0.035 + 0.0031 + (0.99  0.035) + (0.99  0.035)]  (1 – 0.85) = 0.041 lb PM/ton

The expressions below represent PM-emitting processes that are performed at a typical rice mill.  Corresponding emission factors are calculated below each process expression.  According to anecdotal information, hulls comprise about 20% by weight of the rice harvest; impurities (which are eliminated through primary and secondary cleaning) comprise about 3%, and the remaining 77% of the harvest becomes rice products.  According to the US Census Bureau, rice mills produced the following breakdown of products in 1997 (percent by weight):  “head rice,” or unbroken whole grains, 69%; “second head,” or slightly broken and/or damaged grains, 3.9%; screenings and brewers’ rice, or finely broken grains, 4.5%; rice flour, 8.0%; rice bran, 8.9%; and residuals/byproducts 5.5% (US Census Bureau, 1999a).  These process fractions are used to determine the proportions of the total rice harvest that typically follow each process at rice mills.

Receiving and millhouse processes:

Truck unloading  transfer to secondary cleaning  clean (3% impurities lost)  transfer to millhouse  millhouse processes  (20% husks, 77% millhouse product)

([0.99  (0.0078 + 0.0078 + 0.0031)] + [0.97  (0.0078 + 0.27)])  (1 – 0.85) = 0.043 lb PM10/ton

([0.99  (0.035 + 0.035 + 0.0031)] + [0.97  (0.035 + 0.27)])  (1 – 0.85) = 0.055 lb PM/ton

Rice husks (20% of harvested weight):

(From millhouse)  husks transfer to storage pile  load on trucks

[0.2   (0.0078 + 0.0078)]  (1 – 0.85) = 0.0005 lb PM10/ton

[0.2  (0.035 + 0.035)]  (1 – 0.85) = 0.002 lb PM/ton

Brokens/screenings/brewer’s rice (8.4% of millhouse product)

(From millhouse)  transfer brokens/screenings/brewer’s to storage  transfer for packing or truck loading

[0.084   0.77  (0.0078 + 0.0078)]  (1 – 0.85) = 0.0002 lb PM10/ton

[0.084   0.77  (0.035 + 0.035)]  (1 – 0.85)  = 0.0007 lb PM/ton

Finished “head rice” (69% of millhouse product)

(From millhouse)  transfer to polisher  polish  transfer to storage  transfer for packing or truck loading

[0.69  0.77  (0.0078 + 0.05 + 0.0078 + 0.0078)]  (1 – 0.85) = 0.006 lb PM10/ton

[0.69  0.77  (0.035 + 0.05 + 0.035 + 0.035)]  (1 – 0.85) = 0.012 lb PM/ton

Rice flour (8.0% of millhouse product)

(From millhouse)  transfer to flour grinding  flour processing (assumed 98% control efficiency)  transfer to packing

0.08  0.77  ([(0.0078 + 0.0078)  (1 – 0.85)] + (35  [1 – 0.98]) = 0.043 lb PM10/ton

0.08  0.77  ([(0.035 + 0.035)  (1 – 0.85)] + (70  [1 – 0.98]) = 0.087 lb PM/ton

Rice bran and byproducts (15.5% of millhouse products)

(From millhouse)  transfer bran to storage  transfer for packing or truck loading

[0.155  0.77  (0.017 + 0.017)]  (1 – 0.85) = 0.0006 lb PM10/ton

= 0.0006 lb PM/ton

Sum total for rice milling

0.043 + 0.0005 + 0.0002 + 0.006 + 0.043 + 0.0006 = 0.094 PM10/ton

0.055 + 0.002 + 0.0007 + 0.012 + 0.087 + 0.0006 = 0.158 PM/ton,

### Activity Data and Emissions for Rice Processing

In 1999, farms in the CCOS II region produced 1.96 million tons of rice (California Agricultural Statistics Service, 2000a).  In addition, statewide production increased 19% from 1999 to 2000 (California Agricultural Statistics Service, 2001b).  The USDA reported that California’s stores of rough rice increased by 0.39 million tons from 1999 to 2000, which represented around 16% of the state’s total rice production in 2000 (California Agricultural Statistics Service, 2001b).  It is assumed that the remainder, 84%, was milled during 2000.

The quantity of rice produced in the CCOS II region for year 2000, all of which is assumed processed in the region, is estimated to be 2.34 million tons (or 1.96 million tons  1.19).  The quantity of rice milled in the CCOS II region is estimated to be 1.96 million tons (or 2.34 million tons  0.84).  Corresponding PM emissions are estimated below.

2.34 million tons  0.012 lb PM10/ton  2000 lb/ton = 14.3 tons PM10 for grain elevators

2.34 million tons  0.041 lb PM/ton  2000 lb/ton = 48.3 tons PM for grain elevators

1.96 million tons  0.0935 lb PM10/ton  2000 lb/ton = 91.8 tons PM10 for rice mills

1.96 million tons  0.1579 lb PM/ton  2000 lb/ton = 154.9 tons PM for rice mills

These emissions are assigned proportionally to the CCOS II counties according to rice production and employment in the grain warehousing and rice milling industrial sectors (see Tables 2 and 3).  For grain storage and elevators, arbitrary weighting factors of 0.25 and 0.75 were applied to the employment and rice production distributions, respectively.  As an example, the disaggregations are shown below for Colusa County.

14.3 tons PM10  [(0.34  0.25) + (0.26  0.75)] = 4.1 tons PM10 for grain elevators

48.3 tons PM  [(0.34  0.25) + (0.26  0.75)] = 13.7 tons PM for grain elevators

91.8 tons PM10   0.37 = 34 tons PM10 for rice mills

154.9 tons PM  0.37 = 58 tons PM for rice mills

Table 2.   Estimated employment (first quarter, 1999) for grain storage and

estimated emissions (2000) for rice grain elevators (NAICS code 493130).

|  |  |  |  |  |  | Emissions |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| County | No. of Employees Range | No. of Employees Midpointb | Proportion of Total | Rice Production (1000 tons) | Proportion of Total | PM10 (tons) | PM (tons) |
| Butte | 1-20 | 10 | 6% | 431.9 | 18% | 2.21 | 7.43 |
| Colusa | 20-99 | 54 | 34% | 620.0 | 26% | 4.06 | 13.69 |
| Glenn | - | - | 0% | 390.5 | 17% | 1.79 | 6.03 |
| Placer | - | - | 0% | 58.1 | 2% | 0.27 | 0.90 |
| Sacramento | 20-99 | 54 | 34% | 39.9 | 2% | 1.41 | 4.74 |
| Shasta | - | - | 0% | 1.9 | 0% | 0.01 | 0.03 |
| Solano | 1-20 | 10 | 6% | 0.0 | 0% | 0.23 | 0.76 |
| Sutter | 1-20 | 10 | 6% | 516.6 | 22% | 2.59 | 8.74 |
| Tehama | - | - | 0% | 2.3 | 0% | 0.01 | 0.03 |
| Yolo | 1-20 | 10 | 6% | 130.1 | 6% | 0.82 | 2.77 |
| Yuba | 1-20 | 10 | 6% | 153.8 | 7% | 0.93 | 3.14 |
| Total | 45-157a | 157 | 100% | 2345 | 100% | 14 | 48 |

Source:  http://tier2.census.gov/cbp_naics/index.html, http://www.nass.usda.gov/ca/bul/agcom/indexcac.htm

aEmployment in non-CCOSII counties is known to be at minimum 274 employees, which limits the share of CCOS II employees to 157 out of the statewide total of 431.

bAdjusted so that the total will not exceed 157.

Table 3.   Estimated employment (first quarter, 1999) and

estimated emissions (2000) for rice milling (NAICS code 311212).

|  |  |  |  | Emissions |  |
| --- | --- | --- | --- | --- | --- |
| County/Area | No. of Employees Range | No. of Employees Midpointb | Proportion of Total | PM10 (tons) | PM (tons) |
| Butte | 100-249 | 175 | 17% | 15.9 | 26.9 |
| Colusa | 250-499 | 375 | 37% | 34.1 | 57.6 |
| El Dorado | 20-99 | 60 | 6% | 5.46 | 9.22 |
| Yolo | 500-518 | 509 | 50% | 46.3 | 78.2 |
| CCOS II Total | 906-1109 | 1008 | 100% | 92 | 155 |

Source:  http://tier2.census.gov/cbp_naics/index.html

### Other Grains Processing:  Emission Factors and Activity Data

Processes for wheat and corn grain elevators are similar to those for rice elevators, but AP-42 gives slightly different emission factors for cleaning: 0.012-0.075 lb/ton.  Recall that the California Agricultural Commissioners’ Reports mentioned that the 1999 corn and wheat harvests were 0.6 and 0.3 million tons for the CCOS II region.  The year-2000 harvests were estimated to be 18% larger in wheat and 17% smaller in corn than the 1999 harvests, respectively (California Agricultural Statistics Service, 2000b; California Agricultural Statistics Service, 2000c).  The process expression, comprehensive emission factor, and estimated emissions for wheat and corn grain elevators in the CCOS II region are shown below.  Emissions are disaggregated to the CCOS II counties proportionally according to corn and wheat productions and employment in the grain warehousing industrial sector (see Table 4).  For grain storage and elevators, arbitrary weighting factors of 0.25 and 0.75 were applied to the employment and grain production distributions, respectively.

Truck unloading  transfer to initial storage  transfer to primary cleaning  clean (1% impurities lost) transfer to storage  load for shipping to grain mill

[0.0078 + 0.0078 + 0.0078 + 0.075 + 0.99  (0.0078 + 0.0078)]  (1 – 0.85)

= 0.017 lb PM10/ton

[0.035 + 0.035 + 0.035 + 0.075 + 0.99  (0.035 + 0.035)]  (1 – 0.85) = 0.037 lb PM/ton

0.017 lb/ton  [(0.6 million tons  1.18) + (0.3 million tons  0.83)]  2000 lb/ton

= 8 tons PM10 per year for grain elevators

0.037 lb/ton  [(0.6 million tons  1.18) + (0.3 million tons  0.83)]  2000 lb/ton

= 18 tons PM per year for grain elevators

Colusa county:

8 tons PM10  [(0.35  0.25) + (0.05  0.75)] = 1 ton PM10 per year for grain elevators

18 tons PM  [(0.35  0.25) + (0.05  0.75)] = 2 ton PM per year for grain elevators

About 50% of produced corn is used for animal feed.  The comprehensive AP-42 emission factors for animal feed milling processes (receiving, transfer/conveyance, milling, etc.) are around 0.3 lb PM10/ton and 0.6 lb PM/ton.  Emissions are disaggregated to the CCOS II counties proportionally according to employment in the animal food processing industrial sector (see Table 4).

0.50  (0.6 million tons  1.18)  0.3 lb PM10/ton  (1 – 0.85)  2000 lb/ton

= 8 tons PM10 for animal feed processing mills

0.50  (0.6 million tons  1.18)  0.6 lb PM/ton  (1 – 0.85)  2000 lb/ton

= 16 tons PM for animal feed processing mills

Glenn county:

8 tons PM10  0.13 = 1 ton PM10 per year for animal feed processing mills

16 tons PM  0.13 = 2 ton PM per year for animal feed processing mills

About 60% of harvested wheat is processed at wheat flourmills.  The comprehensive AP-42 emission factors for wheat flour milling processes (receiving, transfer conveyance, milling, etc.) are around 0.76 lb PM10/ton and 1.7 lb PM/ton.  These emissions are disaggregated to the CCOS II counties proportionally according to employment in the flour milling industrial sector (see Table 4).

0.6  (0.3 million tons  0.83)  0.76 lb/ton  (1 – 0.85)  2000

= 9 tons PM10 for wheat flourmills

0.6  (0.3 million tons  0.83)  1.7 lb/ton  (1 – 0.85)  2000

= 19 tons PM for wheat flourmills

Solano county:

9 tons PM10  0.05 = 0.4 ton PM10 per year for wheat flourmills

19 tons PM  0.05 = 1 ton PM per year for wheat flourmills

Table 4.   Estimated employment proportions (1st quarter 1999) for

grain storage (NAICS code 493130), animal feed milling (NAICS 311119), and flour milling (NAICS 311210) with estimated emissions for corn and wheat processing (2000).

|  | Proportion of Total Grain Production | Proportion of Total Employmenta |  |  | Estimated Emissions (tons) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NAICS | NAICS | NAICS | Grain Elevators |  | Animal Feed Processing |  | Flour Milling |  |
| County/Area |  | 493130 | 311119 | 311210 | PM10 | PM | PM10 | PM | PM10 | PM |
| Amador | 0% | 0% | 0% | 0% | 0.01 | 0.02 | 0 | 0 | 1.22 | 2.65 |
| Butte | 0% | 6% | 0% | 14% | 0.15 | 0.33 | 0 | 0 | 1.22 | 2.65 |
| Colusa | 5% | 35% | 0% | 31% | 1.05 | 2.29 | 0 | 0 | 2.61 | 5.69 |
| El Dorado | 0% | 0% | 0% | 5% | 0.00 | 0.00 | 0 | 0 | 0.42 | 0.91 |
| Glenn | 13% | 0% | 13% | 0% | 0.77 | 1.68 | 1.02 | 2.06 | 0 | 0 |
| Sacramento | 37% | 35% | 75% | 1% | 2.97 | 6.51 | 6.09 | 12.37 | 0.07 | 0.15 |
| Shasta | 0% | 0% | 0% | 1% | 0.00 | 0.01 | 0 | 0 | 0.07 | 0.15 |
| Solano | 16% | 6% | 0% | 5% | 1.08 | 2.38 | 0 | 0 | 0.42 | 0.91 |
| Sutter | 5% | 6% | 0% | 0% | 0.42 | 0.92 | 0 | 0 | 0 | 0 |
| Tehama | 1% | 0% | 13% | 0% | 0.04 | 0.09 | 1.02 | 2.06 | 0 | 0 |
| Yolo | 21% | 6% | 0% | 44% | 1.40 | 3.08 | 0 | 0 | 3.75 | 8.16 |
| Yuba | 2% | 6% | 0% | 0% | 0.27 | 0.58 | 0 | 0 | 0 | 0 |
| Ccos ii Total | 100% | 100% | 100% | 100% | 8 | 18 | 8 | 16 | 9 | 19 |

Source:  Census Bureau (2001a)

aFrom midpoints of employment ranges.

## Nuts Processing and Roasting

This emissions category includes activities associated with the processing, handling, and roasting of nuts and seeds.  Nuts and seeds processing operations include loading and unloading, hulling, cleaning, roasting, etc.  Table 5 summarizes recently estimated quantities of nuts and seeds produced by farmers in the US, California, and the CCOS II region alone.

Table 5.   Nut and seed crops harvested in 1999 and/or 2000.

| Crop name | 1999 CCOS II Productiona (tons) | 1999 California Productiona (tons) | CCOSII Proportion of California Totala | 1999; 2000 US Productionb (tons) |
| --- | --- | --- | --- | --- |
| Walnuts | 125,531 | 309,924 | 40.5% | 283,000; 239,000 (CA only) |
| Almonds | 57,287 | 458,697 | 12% | 416,500; 351,500 (CA only) |
| Sunflower seed | 5,652 | 6,852 | 82% | 2,170,950; 1,772,200 |
| Pistachios | 1,487 | 65,892 | 2% | 61,500; 121,500 (CA only) |
| Pecans | 0 | 700 | 0% | 203,500 ; 104,925 |
| Macadamia nuts | 0 | 224 | 0% | 23,250; 25,000 (HI only) |
| Hazelnuts | 0 | 0 | 0% | 40,000; 24,000 (OR & WA only) |
| Peanuts | 0 | 0 | 0% | 1,914,745; 1,632,753 (Southeastern US only) |
| Total | 189,957 | 842,289 | 23% | 4,802,525 |
| Fruits & nuts (% of total unspecified) | 16% | 2% |  |  |

Note:  There are some discrepancies when data that were acquired from different sources of information are compared.  However, the differences are reasonably small (10% or less).

a Source: California Agricultural Statistics Service, 2000a

b Source: USDA, 2001a; and USDA, 2001b

### Nuts Processing: Activity Data

Different types of nuts undergo different processes and different process rates.  Below are explained the information and assumptions that were used to forecast year-2000 production levels from 1999 data; and to estimate the quantities of nuts that underwent specific processing steps (e.g, hulling, cleaning, etc.).  (Note that the year-2000 data that are presented in Table 5 were added during a late revision of this memorandum, and were not available at the time of first writing.)

#### Walnuts Processing

According to USDA California Agricultural Statistics Service, California’s 1999 walnut crop reached 283,000 tons.   However, the 2000 walnut crop was estimated to be 15.5% smaller than the 1999 crop (California Agricultural Statistics Service, 2001a).  For the past several years, California handlers processed approximately 100% of the annual quantities walnuts produced by California farmers (California Agricultural Statistics Service, 1997-2000).  Thus, it is estimated that the quantity of walnuts processed in the CCOS II region for year 2000 was roughly equivalent to the year-2000 harvest, or around 106,073 tons (125,531 tons  0.845).

Based on the California Walnut Inventory Report released by the Walnut Marketing Board on August 22, 2000, producers and handlers held stores of 9,500 tons of inshell walnuts out of a total inventory of 73,777 tons (calculated on an inshell equivalent weight basis) (International Tree Nut Council, 2000).  Thus, it is estimated that approximately 13% of harvested walnuts are not hulled, but are left inshell after processing (9,500 tons  73,777 tons).

#### Almonds Processing

According to the Almond Board’s Year-2000 Almanac, CCOS II counties produced 51,600 tons of almonds during the 1999-2000 crop year.  The Board also forecasted that the 2000-2001 harvest would be 23% smaller than that from 1999-2000.  Thus, it is estimated that year-2000 production of almonds in the CCOS II regions is 44,111 tons (or 0.77  57,287 tons).  No information regarding inshell vs. shelled production was available for almonds.  The same percentage that was calculated above for walnuts (13%) will be assumed.

#### Pistachio Processing

Over the past several years, USDA figures show that pistachio production fluctuates a great deal from year to year – as much as 100%.  According to the California Pistachio Board (2000), total shipments in 2000 were 70,631 tons, which is 7% larger than the 1999 harvest shown in Table 5.  Thus, the year-2000 harvest for CCOS II counties is forecast to be 7% larger than the 1999 harvest, or 1591 tons (1487 tons  1.07).  In addition, the Board suggests that 88% of stocks are shipped as open inshell, 5% as closed shell, 3% as shelling stock (with dark stain, adhering hull, closed shells or shell damage), and 5% as loose kernels.

#### Sunflower Seed Processing

According to the National Sunflower Association, 17.3% of the US sunflower harvest is comprised of non-oilseeds, which are used to make snack and whole-kernel food products (National Sunflower Association, 2000a; National Sunflower Association, 2000b).  California is a minor producer of sunflowers, so the USDA provides no California-specific harvest estimates.  Therefore, the County Agricultural Commissioners’ Reports (California Agricultural Statistics Service, 2000a) will be used instead of USDA data to estimate the production of whole-kernel sunflower seeds.  Whole-kernel sunflower seed production for the CCOS II region is estimated to be 17.3% of total production, or 978 tons (or 5,652 tons  0.173).  No forecast for year 2000 is available, therefore the 1999 production level is used instead.  No information was available regarding manufactured quantities of shelled vs. inshell sunflower seeds.  Therefore, a 50-50% split is assumed.

### Nuts Processing: Emissions

Little information was found regarding emission factors for processing nuts and seeds.  AP-42, Chapter 9.10, presents PM emission factors for almond processing.  Comprehensive emission factors, which include loading/unloading/transfer points, precleaning (controlled), and shelling (controlled), are estimated to be approximately 1.4 to 1.7 lb PM10/ton and 1.8 to 2 lb PM/ton of nuts processed, depending upon the proportion hulled.  In the absence of nut-specific emission factors, the emission factors for almond processing were applied to estimate year-2000 emissions for all types of nut processing in the CCOS II region.  It is assumed that initial handling processes (cleaning, drying, grading, hulling, etc.) occur in the counties where nuts and seeds are grown.  Therefore, year-2000 emissions are disaggregated according to the relative sizes of the 1999 county nut harvests (see Table 6).  An emissions control efficiency of 85% is arbitrarily assumed.

Walnut processing: 	106,073 tons   1.7 lb PM10/ton  2000  (1 – 0.85) = 14 tons PM10

106,073 tons   2 lb PM/ton  2000   (1 – 0.85) = 16 tons PM

Butte County only:	14 tons PM10  0.26 = 3.5 tons PM10

16 tons PM  0.26 = 4.1 tons PM

Almond processing:	44,111 tons   1.7 lb PM10/ton  2000  (1 – 0.85) = 5.7 tons PM10

44,111 tons   2 lb PM/ton  2000  (1 – 0.85) = 6.6 tons PM

Tehama County only:	5.7 tons PM10  0.051 = 0.29 tons PM10

6.6 tons PM  0.051 = 0.33 tons PM

Pistachio processing:	1591 tons   1.4 lb PM10/ton  2000  (1 – 0.85) = 0.17 tons PM10

1591 tons   1.6 lb PM/ton  2000  (1 – 0.85) = 0.19 tons PM

Glenn County only:	0.17 tons PM10  0.599 = 0.1 tons PM10

0.19 tons PM  0.599 = 0.1 tons PM

Sunflower seed processing:	978 tons  1.6 lb PM10/ton  2000  (1 – 0.85) = 0.11 tons PM10

978 tons   1.8 lb PM/ton  2000  (1 – 0.85) = 0.13 tons PM

Glenn County only:	0.11 tons PM10  0.929 = 0.11 tons PM10

0.13 tons PM  0.929 = 0.12 tons PM

### Nuts Roasting

Although no emission factors for nut roasters were identified, activity data are provided should an emission factor become available at a future date.  For a recent year (1997), it was estimated that approximately 40% to 60% of the total California nut-and-seed production was roasted.  It is assumed that California nut roasters exclusively use California-harvested nuts and seeds and that peanut butter production is negligible.  In the CCOS II region, employment for the industrial sector, “Roasted Nuts and Peanut Butter Processing” (NAICS code 311911) was approximately 2358 employees, or 47% of the state total (U.S. Census Bureau, 2001a).  Thus, for the year 1999, it is estimated that of the total California nut and seed harvest, approximately 158 to 237 thousand tons of nuts and seeds were roasted in the CCOS II region (or, 40% to 60% of 842,289  47%).  Below are presented similar year-2000 estimates for the CCOSII region, which were calculated from the data presented in Table 5.

0.47  0.40  (239,000 tons walnuts + 351,500 tons almonds + 6,852 tons sunflower seeds + 121,500 tons pistachios)

= 135 thousand tons of nuts and seeds roasted (low-end estimate)

0.47  0.60  (239,000 tons walnuts + 351,500 tons almonds + 6,852 tons sunflower seeds + 121,500 tons pistachios)

= 203 thousand tons of nuts and seeds roasted (high-end estimate)

This activity data may be disaggregated to counties according to the proportion of employment in each county relative to the total employment for NAICS code 311911, available from the US Census Bureau and summarized in Table 7 for 1999.

Table 6.   Estimated 1999 production of nuts and seeds in the CCOS II region and year-2000 estimated PM and PM10 emissions for processing.

|  | Almonds |  |  |  |  | Pistachios |  |  |  |  |  | Sunflower Seeds |  |  |  |  | Walnuts |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| County |  | Production (1000 tons) | Percent of Total | PM10 (tons) | PM (tons) |  | Production (1000 tons) | Percent of Total | PM10 (tons) | PM (tons) | Production (1000 tons) |  | Percent of Total | PM10 (tons) | PM (tons) | Production (1000 tons) |  | Percent of Total | PM10 (tons) | PM (tons) |
| Amador |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 382 |  | 0.3% | 0.04 | 0.05 |
| Butte |  | 22,696 | 39.6% | 2.25 | 2.62 |  | 530 | 35.6% | 0.06 | 0.07 | 0 |  | 0.0% | 0 | 0 | 32,600 |  | 26.0% | 3.54 | 4.13 |
| Calaveras |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 420 |  | 0.3% | 0.05 | 0.05 |
| Colusa |  | 12,125 | 21.2% | 1.20 | 1.40 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 6,357 |  | 5.1% | 0.69 | 0.81 |
| El dorado |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 132 |  | 0.1% | 0.01 | 0.02 |
| Glenn |  | 11,073 | 19.3% | 1.10 | 1.28 |  | 890 | 59.9% | 0.10 | 0.11 | 5,251 |  | 92.9% | 0.11 | 0.12 | 8,477 |  | 6.8% | 0.92 | 1.07 |
| Mariposa |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 |
| Mendocino |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 |
| Nevada |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 |
| Placer |  | 0 | 0.0% | 0 | 0 |  | 67 | 4.5% | 0.01 | 0.01 | 0 |  | 0.0% | 0 | 0 | 1,668 |  | 1.3% | 0.18 | 0.21 |
| Plumas |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 |
| Sac’to |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 352 |  | 0.3% | 0.04 | 0.04 |
| Shasta |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 962 |  | 0.8% | 0.10 | 0.12 |
| Sierra |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 |
| Solano |  | 609 | 1.1% | 0.06 | 0.07 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 4,893 |  | 3.9% | 0.53 | 0.62 |
| Sutter |  | 3,894 | 6.8% | 0.39 | 0.45 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 28,448 |  | 22.7% | 3.09 | 3.60 |
| Tehama |  | 2,900 | 5.1% | 0.29 | 0.33 |  | 0 | 0.0% | 0 | 0 | 401 |  | 7.1% | 0.01 | 0.01 | 16,470 |  | 13.1% | 1.79 | 2.09 |
| Tuolumne |  | 0 | 0.0% | 0 | 0 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 |
| Yolo |  | 3,423 | 6.0% | 0.34 | 0.40 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 11,605 |  | 9.2% | 1.26 | 1.47 |
| Yuba |  | 567 | 1.0% | 0.06 | 0.07 |  | 0 | 0.0% | 0 | 0 | 0 |  | 0.0% | 0 | 0 | 12,765 |  | 10.2% | 1.39 | 1.62 |
| Total |  | 57,287 | 100.0% | 5.7 | 6.6 |  | 1,487 | 100.0% | 0.17 | 0.19 | 5,652 |  | 100.0% | 0.11 | 0.13 | 125,531 |  | 100.0% | 13.6 | 15.9 |

Table 7.   Employment in the roasted nuts and peanut butter

manufacturing sector for 1999.

| County | No. of Employees (midpoint of range) | Proportion of State Total | Proportion of CCOSII Total |
| --- | --- | --- | --- |
| Sacramento | 1750 | 35% | 74% |
| Yolo | 375 | 8% | 16% |
| Glenn | 175 | 3% | 7% |
| Butte | 60 | 1% | 3% |
| CCOS II Total | 2360 | 47% | 100% |
| California Total | 4991 | 100% | nc |

Source: US Census Bureau, 2001a

## Breweries

Brewery processes include handling of malts and grains, milling of malts and grains (which generally only occurs at larger breweries), brewing, fermenting, and containerizing.  Based on the following information from the Beer Institute and the US Census Bureau, Table 8 lists estimated year-2000 brewery activity levels for California.

- California beer production was consistently 11-12% of US production from 1989 to 1997 (Beer Institute, 1999).

- The US brewing industry experienced 2.7% growth from 1997 to 2000 (Beer Institute, 2000a; US Census Bureau, 2001b).

- US production of beer was estimated to be 205,626,349 barrels in 1997 (Beer Institute, 1999).

- US brewery consumption of malts and grains in 1997 was 3,177,641 tons (US Census Bureau, 1999c).

- Brewery production of dried spent brewers’ grain in 1997 was 282,738 tons (US Census Bureau, 1999c).

- Large breweries held a 96.7% market share in 2000 (Institute for Brewing Studies, 2001).

- Beer production is split 51%-39%-10% by volume between canned, bottled, and keg beers (Beer Institute, 1999).

Table 8.   Activity parameters for beer brewing in California, year 2000.

| Activity | Magnitude |
| --- | --- |
| Grain handling | 379,957 tons of grain |
| Grain milling at brewery | 367,264 tons of grain |
| Brewers' grain drying | 33,808 tons |
| Fermentation/Brewing | 24,587,188 barrels of beer |
| Bottling | 12,943,780 barrels of beer |
| Canning | 10,288,494 barrels of beer |
| Keg filling | 2,457,550 barrels of beer |

(One barrel, or bbl = 31 gallons.)

Emission factors for brewery processes are available from AP-42, Chapters 9.9 and 9.12.  Table 9 summarizes emission factors selected for use from AP-42.  These emission factors were combined with assumed control efficiencies and the activity parameters listed in Table 8 in order to estimate year-2000 emissions for the State of California (see Table 10).  (Control efficiencies of 85% for grain handling and 98% for milling were assumed.)  These emissions were disaggregated to the county level according to employment in the sector for malt beverage breweries (NAICS 312120).  Results are shown in Table 11.  Emissions for the CCOS II group of counties alone total 32.8 tons PM10, 66.5 tons PM, and 60.3 tons VOC.

Grain handling at brewery:  (0.014 + 0.014 + 0.015 + 0.014) lb PM10/ton malt  (1-0.85)  379,957 tons malt  2000 lbs/ton= 1.6 tons PM10

Grain milling at brewery:  [0.7 + 0.014  (1-0.85)] lb PM10/ton malt  367,264 tons  2000 lb/ton = 128.9 tons PM10

Brewers' grain drying:  0.73 lb VOC/ton dried  33,808 tons = 12 tons VOC

Brewing:  0.001054 lb VOC/bbl beer  24,587,188 bbl = 13 tons VOC

Fermentation (assume closed):  0.002605 lb VOC/bbl beer  24,587,188 bbl = 32 tons VOC

Bottling:   0.017 lb VOC/bbl beer  12,943,780 bbl = 110 tons VOC

Canning:  0.014 lb VOC/bbl beer 10,288,494 bbl = 72 tons VOC

Keg filling:  0.00069 lb VOC/bbl beer  2,457,550 bbl = 0.8 tons VOC

Total VOC:  12 + 13 + 32 + 110 + 72 + 0.8 = 240 tons VOC

Mendocino county only:  240 tons VOC  4.3% = 10 tons VOC

Total PM10:  1.6 + 128.9 = 130.6 tons PM10

Solano county only:  130.6 tons PM10  9.2% = 12.03 tons PM10

Table 9.   Emission factors for brewing processes.

| Process | Emission Factors |
| --- | --- |
| Grain Handling |  |
| Unloading/transferring malt or grain | 0.014 lb PM10/ton of grain processed 0.064 lb PM/ton of grain processed |
| Hopper operationa | 0.015 lb PM10/ton of grain processed 0.015 lb PM/ton of grain processed |
| Composite total, with controlsb | 0.00855 lb PM10/ton of grain processed 0.03105 lb PM/ton of grain processed |
| Grain Milling |  |
| Unloading/transferring malt or grain | 0.014 lb PM10/ton of grain processed 0.064 lb PM/ton of grain processed |
| Millingc | 0.70 lb PM10/ton of grain processed 1.4 lb PM/ton of grain processed |
| Composite total, with controlsd | 0.7021 lb PM10/ton of grain processed 1.4096 lb PM/ton of grain processed |
| Brewing Operations |  |
| Cereal cooking | 0.0075 lb VOC/1,000 bbl of beer |
| Mashing | 0.054 lb VOC/1,000 bbl of beer |
| Brewing | 0.64 lb VOC/1,000 bbl of beer |
| Settling | 0.075 lb VOC/1,000 bbl of beer |
| Transferring spent grain | 0.25 lb VOC/1,000 bbl of beer |
| Cooling | 0.022 lb VOC/1,000 bbl of beer |
| Drying spent brewers’ grain | 0.73 lb VOC/ton of dried brewers’ grain |
| Totals | 1.054 lb VOC/1,000 bbl of beer 0.73 lb VOC/ton of dried brewers’ grain |
| Fermentation |  |
| Fermentation (closed) | 2 lb VOC/1,000 bbl of beer |
| Activated carbon recharge and venting | 0.035 lb VOC/1,000 bbl of beer |
| Filling aging tanks | 0.57 lb VOC/1,000 bbl of beer |
| Total | 2.605 lb VOC/1,000 bbl of beer |
| Containerizing |  |
| Canning | 14 lb VOC/1,000 bbl of beer |
| Bottling | 17 lb VOC/1,000 bbl of beer |
| Filling kegs | 0.69 lb VOC/1,000 bbl of beer |

Source: AP-42.

a Similar in magnitude to grain cleaning or rice bran handling.

b 3 transfer points are assumed.

c Assumed similar to wheat flour milling with 98% control.

d 1 transfer point is assumed

Table 10.  Year-2000 emissions for beer brewing in the State of California.

| Activity Description | Emission Factor | Activity |  | Emissions |
| --- | --- | --- | --- | --- |
| Grain handling | 0.00855 lb PM10/ton grain | 379,957 tons |  | 1.6 tons PM10 |
|  | 0.03105 lb PM/ton grain | 379,957 tons |  | 5.9 tons PM |
| Grain milling | 0.7021 lb PM10/ton grain | 367,264 tons |  | 128.9 tons PM10 |
|  | 1.4096 lb PM/ton grain | 367,264 tons |  | 258.8 tons PM |
| Brewers' grain drying | 0.73 lb VOC/ton dried b.g. | 33,808 tons |  | 12 tons VOC |
| Brewing | 0.001054 lb VOC/bbl beer | 24,587,188 bbl beer |  | 13 tons VOC |
| Fermentation (assume closed) | 0.002605 lb VOC/bbl beer | 24,587,188 bbl beer |  | 32 tons VOC |
| Bottling | 0.017 lb VOC/bbl beer | 12,943,780 bbl beer |  | 110 tons VOC |
| Canning | 0.014 lb VOC/bbl beer | 10,288,494 bbl beer |  | 72 tons VOC |
| Keg filling | 0.00069 lb VOC/bbl beer | 2,457,550 bbl beer |  | 0.8 tons VOC |
| Totals |  |  | 240 tons VOC |  |
|  |  |  | 130.6 tons PM10 |  |
|  |  |  | 264.7 tons PM |  |

Table 11.   Employment (1st quarter, 1999) and emissions (2000) for beer brewing by county.

|  | No. of Employees | Percent of | Emissions (tons/yr) |  |  |
| --- | --- | --- | --- | --- | --- |
| County Name | (Range Midpoint) | State Total | VOC | PM10 | PM |
| Alameda | 190 | 4.7% | 11.2 | 6.10 | 12.38 |
| Butte | 175 | 4.3% | 10.3 | 5.61 | 11.37 |
| Calaveras | 60 | 1.5% | 3.5 | 1.91 | 3.88 |
| Contra Costa | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| El Dorado | 34 | 0.8% | 2.0 | 1.09 | 2.22 |
| Fresno | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| Humboldt | 175 | 4.3% | 10.3 | 5.61 | 11.37 |
| Kern | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| Los Angeles | 1,981 | 48.8% | 117.1 | 63.65 | 129.07 |
| Mendocino | 175 | 4.3% | 10.3 | 5.61 | 11.37 |
| Napa | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| Nevada | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| Orange | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| Placer | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| Riverside | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| Sacramento | 175 | 4.3% | 10.3 | 5.61 | 11.37 |
| San Benito | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| San Bernardino | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| San Diego | 81 | 2.0% | 4.8 | 2.60 | 5.28 |
| San Francisco | 60 | 1.5% | 3.5 | 1.91 | 3.88 |
| San Luis Obispo | 60 | 1.5% | 3.5 | 1.91 | 3.88 |
| San Mateo | 60 | 1.5% | 3.5 | 1.91 | 3.88 |
| Santa Barbara | 60 | 1.5% | 3.5 | 1.91 | 3.88 |
| Santa Clara | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| Santa Cruz | 60 | 1.5% | 3.5 | 1.91 | 3.88 |
| Solano | 375 | 9.2% | 22.1 | 12.03 | 24.40 |
| Sonoma | 175 | 4.3% | 10.3 | 5.61 | 11.37 |
| Stanislaus | 60 | 1.5% | 3.5 | 1.91 | 3.88 |
| Sutter | 10 | 0.2% | 0.6 | 0.31 | 0.62 |
| State Total | 4,064 |  | 240.2 | 130.6 | 264.7 |

## Temporal Distributions

### Seasonal Distributions

News bulletins from the National Agricultural Statistics Service (1999-2000) and other anecdotal information sources were reviewed to determine harvest seasons, listed below.

- Wheat:  May through July

- Corn: July through October

- Almonds: August through October

- Rice and sunflowers: September through November

- Walnuts and pistachios: October through December

Seasonal emissions adjustments for PM10, shown in Tables 12 and 13, are based on 4 assumptions: (1) grain elevators ship farm products to buyers at a constant rate over the year, (2) grain elevators process farm products during the harvest season and for one month following the harvest season, (3) grain milling processes are constant over the course of the year, and (4) seasonal beer sales correlate well with production rates.

- 7% of emissions from rice and other grain elevators, which are associated with shipping or transferring products to buyers, are distributed evenly over 12 months.  The remaining emissions from rice grain elevators are distributed in September through December.  Of the remaining 93% of emissions for other grain elevators, 71% are associated with corn and 29% are associated with wheat.  The corn processing emissions are distributed from July through November and the wheat processing emissions are distributed from May through August.

- Emissions from rice and other grain milling are distributed evenly over 12 months.

- Emissions from almond processing are distributed from August through November.  Emissions from walnuts and pistachios processing are distributed from October through January.  Emissions from sunflower processing are distributed from September through December.

- The Beer Institute reports the following monthly distribution of US beer sales (average for 1999-2000).  It is assumed that beer production roughly follows the same distribution.

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7.7% | 7.6% | 8.8% | 8.5% | 9.2% | 9.4% | 9.1% | 9.0% | 8.4% | 7.8% | 7.6% | 7.0% |

The US beer sales figures, shown above, may be used to allocate VOC emissions for this category.

### Diurnal Distributions

Arbitrarily, PM10 emissions are estimated to occur 6 days per week (Sundays off) during regular business hours.  VOC emissions from brewing and fermentation (19% of total VOC emissions) are estimated to occur 7 days per week, 24 hours per day.  All other VOC emissions from breweries are estimated to occur 6 days per week (Sundays off) during regular business hours.

Table 12.   Seasonal distribution of PM10 emissions (tons) for agricultural products processing in the CCOS II region.

| Process | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rice grain elevators – loading for shipment | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Rice grain elevators – receiving and processing |  |  |  |  |  |  |  |  | 3.3 | 3.3 | 3.3 | 3.3 |
| Rice milling | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 |
| Other grain elevators – loading for shipment | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Corn grain elevators – receiving and processing |  |  |  |  |  |  | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| Wheat grain elevators – receiving and processing |  |  |  |  | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |
| Animal feed mills and flourmills | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 |
| Almond processing |  |  |  |  |  |  |  | 1.4 | 1.4 | 1.4 | 1.4 |  |
| Walnut/pistachio processing | 3.5 |  |  |  |  |  |  |  |  | 3.5 | 3.5 | 3.5 |
| Sunflower processing |  |  |  |  |  |  |  |  | 0.0 | 0.0 | 0.0 | 0.0 |
| Beer brewing | 2.5 | 2.5 | 2.9 | 2.8 | 3.0 | 3.1 | 3.0 | 2.9 | 2.7 | 2.6 | 2.5 | 2.3 |
| TOTAL PM10 emissions | 15.1 | 11.6 | 12.0 | 12.0 | 12.7 | 12.8 | 13.6 | 15.0 | 17.6 | 20.9 | 20.8 | 19.2 |
| Monthly distribution | 8.26% | 6.35% | 6.57% | 6.52% | 6.95% | 6.99% | 7.42% | 8.17% | 9.60% | 11.38% | 11.34% | 10.45% |
| Seasonal |  | 21.2% |  |  | 20.5% |  |  | 25.2% |  |  | 33.2% |  |

Table 13.   Seasonal distribution of PM emissions (tons) for agricultural products processing in the CCOS II region.

| Process | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rice grain elevators – loading for shipment | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |
| Rice grain elevators – receiving and processing |  |  |  |  |  |  |  |  | 11.2 | 11.2 | 11.2 | 11.2 |
| Rice milling | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 |
| Other grain elevators – loading for shipment | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Corn grain elevators – receiving and processing |  |  |  |  |  |  | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 |
| Wheat grain elevators – receiving and processing |  |  |  |  | 1.2 | 1.2 | 1.2 | 1.2 |  |  |  |  |
| Animal feed mills and flourmills | 2.9 | 2.9 | 2.9 | 2.9 | 2.9 | 2.9 | 2.9 | 2.9 | 2.9 | 2.9 | 2.9 | 2.9 |
| Almond processing |  |  |  |  |  |  |  | 1.7 | 1.7 | 1.7 | 1.7 |  |
| Walnut/pistachio processing | 4.0 |  |  |  |  |  |  |  |  | 4.0 | 4.0 | 4.0 |
| Sunflower processing |  |  |  |  |  |  |  |  | 0.0 | 0.0 | 0.0 | 0.0 |
| Beer brewing | 5.1 | 5.0 | 5.8 | 5.7 | 6.1 | 6.3 | 6.0 | 6.0 | 5.6 | 5.2 | 5.0 | 4.6 |
| TOTAL PM emissions | 25.4 | 21.2 | 22.0 | 21.9 | 23.6 | 23.7 | 25.4 | 27.0 | 36.7 | 40.3 | 40.2 | 38.1 |
| Monthly distribution | 7.3% | 6.2% | 6.4% | 6.3% | 6.8% | 6.9% | 7.4% | 7.8% | 10.6% | 11.7% | 11.6% | 11.0% |
| Seasonal |  | 19.9% |  |  | 20.0% |  |  | 25.8% |  |  | 34.3% |  |

## References and Useful Sources of Information

Beer Institute (2000) State of the Industry Report, prepared by the Beer Institute, Washington, D.C.  Accessed from web site http://www.beerinstitute.org/stateindreport.htm.

Beer Institute (1999) 1999 Brewers Almanac, prepared by the Beer Institute, Washington, D.C.  Accessed from web site http://www.beerinstitute.org/99almanacpgs1-15.pdf.

California Agricultural Statistics Service (2001a) Fruit and Nut Review for February 15, 2001, prepared by the California Agricultural Statistics Service of the US Department of Agriculture, Sacramento, California.  Accessed from web site http://www.nass.usda.gov/ca/rev/frt&nut/indexfn.htm.

California Agricultural Statistics Service (2001b) Field Crop Review for January 18, 2001, prepared by the California Agricultural Statistics Service of the US Department of Agriculture, Sacramento, California.  Accessed from web site http://www.nass.usda.gov/ca/rev/fldcrp/indexfc.htm.

California Agricultural Statistics Service (2000a) County Agricultural Commissioners' Data, accessed in June 2001 at web site http://www.nass.usda.gov/ca/bul/agcom/indexcac.htm.  Data are updated annually in August.

California Agricultural Statistics Service (2000b) Field Crop Review for October 17, 2000, prepared by the California Agricultural Statistics Service of the US Department of Agriculture, Sacramento, California.  Accessed from web site http://www.nass.usda.gov/ca/rev/fldcrp/indexfc.htm.

California Agricultural Statistics Service (2000c) Field Crop Review for June 16, 2000, prepared by the California Agricultural Statistics Service of the US Department of Agriculture, Sacramento, California.  Accessed from web site http://www.nass.usda.gov/ca/rev/fldcrp/indexfc.htm.

California Agricultural Statistics Service (1997-2000) Handlers’ Reports, prepared by the California Agricultural Statistics Service of the US Department of Agriculture, Sacramento, California.  Accessed from web site http://www.nass.usda.gov/ca/oth/indexhnd.htm.  Reports are released annually in August.

California Pistachio Board (2000) Pistachio Statistics, Microsoft Excel spreadsheet prepared by the California Pistachio Board, Fresno, California.  Accessed from web site http://www.pistachios.org/html/statsetz.html.

California Rice Commission (2001) Web site http://www.calrice.org.  Sacramento, California.

Institute for Brewing Studies (2001) Craft Brewing Industry Fact Sheet, prepared by the Institute for Brewing Studies, a division of the Association of Brewers, Boulder, Colorado.  Accessed from web site http://www.aob.org/IBS/00.2_craftbrewfs.htm.

International Tree Nut Council (2000) Global Statisticals Review, newsletter of the International Tree Nut Council, Reus, Spain, September.  Accessed from web site http://inc.treenuts.org/stats_walnut_sep00.html.

National Agricultural Statistics Service (1999-2000) Weekly Weather and Crop Bulletins – State Stories, prepared by the National Agricultural Statistics Service of the US Department of Agriculture, Washington, D.C.  Accessed from web site http://usda.mannlib.cornell.edu/reports/nassr/field/weather/.  Newsletters are released weekly throughout the year.

National Sunflower Association (2000a) 2000 US Sunflower Crop Quality Report, prepared by the National Sunflower Association, Bismark, North Dakota.  Accessed from web site http://www.sunflowernsa.com/pdfs/SunflowerReport2000.PDF.

National Sunflower Association (2000b) US Supply and Disappearance Tables, prepared by the National Sunflower Association, Bismark, North Dakota.  Accessed from web page http://www.sunflowernsa.com/producers/mark_stats/us_snd.html.

US Census Bureau, (2001a) CenStats Databases – County Business Patterns Data, database maintained by the US Census Bureau, Washington, D.C., and accessed June 2001 at web site http://tier2.census.gov/dbappweb.htm.

US Census Bureau (2001b) Annual Survey of Manufacturers  1999 Geographic Area Statistics, report prepared by the US Census Bureau, Washington, D.C., April.  Accessed from web site http://www.census.gov/prod/2001pubs/m99-as3.pdf.

US Census Bureau (1999a) Manufacturing Industry Series of the 1997 Economic Census – Rice Milling.  Report prepared by the US Census Bureau, Washington, D.C., November.  Accessed from web site http://www.census.gov/prod/ec97/97m3112b.pdf.

US Census Bureau (1999b) Manufacturing Industry Series of the 1997 Economic Census – Roasted Nuts and Peanut Butter Manufacturing.  Report prepared by the US Census Bureau, Washington, D.C., July.  Accessed from web site http://www.census.gov/prod/ec97/97m3119a.pdf.

US Census Bureau (1999c) Manufacturing Industry Series of the 1997 Economic Census – Breweries.  Report prepared by the US Census Bureau, Washington, D.C., December.  Accessed from web site http://www.census.gov/prod/ec97/97m3121d.pdf.

US Department of Agriculture (2001a) Fruit and Nut Yearbook Spreadsheet Files.  Tables maintained by the US Department of Agriculture on the USDA Economics and Statistics System, http://usda.mannlib.cornell.edu/, November update.

US Department of Agriculture (2001b) Oil Crop Yearbook Spreadsheet Files.  Tables maintained by the US Department of Agriculture on the USDA Economics and Statistics System, http://usda.mannlib.cornell.edu/, October update.

US Environmental Protection Agency (2000) Compilation of Air Pollutant Emission Factors, Volume 1: Stationary Point and Area Sources,” AP-42, Sections 9.9, 9.10, and 9.12. Office of Air Quality Planning and Standards, Research Triangle Park, NC.  5th edition (January 1996).  Supplements A-F (September 2000).  Accessed from web site http://www.epa.gov/ttn/chief/ap42/index.html.

Percent shown represents cumulative loss.

Note that this forecast appears to have been slightly in error according to information that became available from the USDA subsequent to the first writing of this memorandum.  The USDA (2001a) reported that the year-2000 harvest was only 16% smaller than the 1999 harvest.

Note that this forecast appears to have been largely in error according to information that became available from the USDA subsequent to the first writing of this memorandum.  The USDA (2001a) reported that the year-2000 harvest was nearly double the size of the 1999 harvest.  Because emissions from pistachio processing are estimated to be practically negligible – even if they are multiplied by a factor of two – we did not revise our earlier emissions estimates.

The low estimate, 40%, represents the 1997 nut harvest consumption rate that was reported by US roasters (US Census Bureau, 1999b; US Department of Agriculture, 2000).  The high estimate, 60%, is proportional to (a) the 1997 quantity of roasted nuts and seeds produced in the US; and (b) California’s share of the total 1997 US employment in the industrial sector, roasted nuts and peanut butter processing (US Census Bureau, 1999b).