# Attachment H:  Petroleum Marketing

This attachment discusses the emissions sources listed below, which are associated with storage and transportation of petroleum products and natural gas.  For the purposes of this memorandum, bulk storage and transfer of aviation fuels are assigned to the category, “bulk storage and transfer (unspecified).”

Description	EIC Code	CES Number

Bulk plants/terminals - breathing losses	33038211000000 	46466

Bulk plants/terminals  working losses	33038411000000 	46474

Tank cars and trucks  working losses	33039011000000 	46482

Bulk storage and transfer (unspecified)	33099511000000 	82248

Natural gas transmission pipelines	33031801100000	58685

## Bulk Storage and Transfer

Bulk facilities store and transfer petroleum products for distribution to retail gasoline outlets or for use by large-scale consumers (such as air, rail, or ship refuelers; or operators of large fleets).  Table 1 lists estimated annual average stocks of petroleum products held at bulk plants and terminals in California and average sales by prime suppliers in California for the year 2000 (estimated error within 15%).

Table 1.   Estimated annual average stocks held and average monthly sales of

petroleum products in California for the year 2000.

| Product Type | Stocks (1000 bblyr) | Sales (1000 gal/mo) |
| --- | --- | --- |
| Finished Motor Gasoline | 7,556 | 1,233,000 |
| Distillate Fuel Oil | 2,521 | 259,010 |
| Liquefied Petroleum Gases | 354 | 42,720 |
| Kerosene-Type Jet Fuel | 2,107 | 264,072 |
| Residual Fuel Oil | 1,431 | 86,177 |
| Other Hydrocarbons/Hydrogen/Oxygenates | 200 | - |
| Lubricants | 480 | 13,966 |
| Asphalt and Road Oil | 304 | 36,358 |
| Motor Gasoline Blending Components | 363 | - |
| Finished Aviation Gasoline | 96 | 1,664 |
| Miscellaneous Products | 81 | 3,829 |
| Kerosene | 96 | 1,733 |
| Pentanes Plus | 20 | 5,728 |

1 barrel (bbl) = 42 gallons (gal)

The elements of information and assumptions listed below were used to estimate the annual average distribution of petroleum products stocked by bulk terminals alone in California for 1997.  Growth in California’s total petroleum stocks from 1997 to 2000 was estimated from the relative ratio of 1997 to 2000 stocks held by bulk terminals in PAD District V (EIA, 1998a; EIA, 2001a).

- The Energy Information Administration (EIA) routinely tracks the following statistics.

- Stocks of petroleum products held separately by (1) refineries, (2) natural gas plants, and (3) bulk terminals in Petroleum Administration for Defense (PAD) District V in 1997 (EIA, 1998a)

- Total combined stocks of selected petroleum products held in 1997 by bulk terminals, refineries, and natural gas plants in California in 1997 (EIA, 1998a)

- 1997 California prime supplier sales of jet fuel and aviation gasoline (EIA, 1998b)

- The following assumptions were made.

- The relative distribution of petroleum stocks among bulk terminals, refineries, and natural gas plants in California is similar to that in PAD District V.

- California’s shares of PAD District V’s total combined petroleum stocks are similar to California’s shares of PAD District V’s bulk terminal stocks for those petroleum products that were reported (motor gasoline, kerosene, total distillate, residual, propane/propylene).

- California’s shares of PAD District V sales are similar to the California’s shares of PAD District V bulk terminal stocks for other petroleum products (kerosene-type jet fuel and finished aviation gasoline).

- An interpolation method was used to estimate stocks of products that could not be estimated otherwise due to lack of data (other hydrocarbons/hydrogen/oxygenates, lubricants, asphalt and road oil, motor gasoline blending components, finished aviation gasoline, miscellaneous products, kerosene, pentanes plus, naphtha-type jet fuel, special naphthas).  California’s shares of PAD District V bulk terminal stocks were interpolated from the known distribution of PAD District V bulk terminal stocks and the estimated distribution of California bulk terminal stocks.

The following elements of information and assumptions were used to estimate quantities of petroleum products stocked by bulk plants in California in 1997.

- The EIA documents US prices of petroleum products sold in 1997 (EIA, 1998c).

- The US Census Bureau reports the value of stocks held by bulk terminals in California and the value of stocks held by bulk plants in California (US Census Bureau, 2000)

- It was assumed that bulk plants stock petroleum products in the same distribution as do bulk terminals.

The following elements of information and assumptions were used to estimate the sales of petroleum products by bulk plants and terminals in California in 2000.

- The EIA and the California Board of Equalization routinely track the following statistics.

- Taxable sales of gasoline in California (Board of Equalization, 2001).

- Year-2000 sales of selected petroleum products by California prime suppliers (EIA, 2001b).

- Year-2000 sales of petroleum products by PAD District V prime suppliers (EIA, 2001a).

- The following assumptions were made.

- Product sales that are unreported for California can be interpolated from the reported distribution of product sales for PAD District V.

- Bulk plants sell the same distribution of petroleum products as bulk terminals.

- The California Board of Equalization publishes a more representative figure for gasoline throughput at bulk plants and terminals.  The Board of Equalization data represent all sales of gasoline.  However, the EIA excludes sales by prime suppliers to other prime suppliers and traders.

The following elements of information and assumptions were used to estimate quantities of petroleum products transported to bulk plants and stations via truck or train (see Table 2).

- The US Census Bureau reports the numbers of bulk terminals and bulk stations in California according to the transportation modes by which they receive petroleum products (truck, railcar, pipeline, barge, and tanker) (US Census Bureau, 2001a).

- It was assumed that the quantities of products transported to bulk stations/terminals via a transportation mode are proportional to the numbers of bulk stations/terminals that receive products via that mode.  (Most likely, this leads to an underestimate of the quantity transported via pipeline and an overestimate of the quantities transported via truck or railcar.)

Table 2.   Estimated quantities of petroleum products shipped to bulk terminals and stations in

California during the year 2000 according to the mode of transportation.

|  | Quantity Shipped by Mode of Transportation (1000 gal/mo) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Product Type | Truck | Railcar | Pipeline | Barge | Tanker | All |
| Finished Motor Gasoline | 315,083 | 3,734 | 661,275 | 23,217 | 61,912 | 1,233,000 |
| Distillate Fuel Oil | 76,613 | 908 | 160,790 | 5,645 | 15,054 | 259,010 |
| Liquefied Petroleum Gases | 12,636 | 150 | 26,520 | 931 | 2,483 | 42,720 |
| Kerosene-Type Jet Fuel | 78,110 | 926 | 163,932 | 5,756 | 15,348 | 264,072 |
| Residual Fuel Oil | 25,490 | 302 | 53,498 | 1,878 | 5,009 | 86,177 |
| Other Hydrocarbons/Hydrogen/Oxygenates | 0 | 0 | 0 | 0 | 0 | 0 |
| Lubricants | 4,131 | 49 | 8,670 | 304 | 812 | 13,966 |
| Asphalt and Road Oil | 10,754 | 127 | 22,570 | 792 | 2,113 | 36,358 |
| Motor Gasoline Blending Components | 0 | 0 | 0 | 0 | 0 | 0 |
| Finished Aviation Gasoline | 492 | 6 | 1,033 | 36 | 97 | 1,664 |
| Miscellaneous Products | 1,132 | 13 | 2,377 | 83 | 223 | 3,829 |
| Kerosene | 513 | 6 | 1,076 | 38 | 101 | 1,733 |
| Pentanes Plus | 1,694 | 20 | 3,556 | 125 | 333 | 5,728 |

### Emissions

Emission factors were selected from guidance documents of the EPA and the Emission Inventory Improvement Program (EPA, 1995; EIIP, 2001) (see Table 3).  For jet fuel, emission factors were unavailable for truck transit.  However, jet fuel emission factors were available for tanker transit, loading, and refinery storage (see Table 4).  On average, these tanker emission factors were about 3.4 smaller than the corresponding tanker emission factors for gasoline.  Therefore, the truck transit emission factors for jet fuel were estimated by dividing the truck transit emission factors for gasoline by a factor of 3.4 (i.e., 0.005  3.4 = 0.0015 VOC per 1000 gal transferred).  No emission factors were available for petroleum products unlisted in Table 3.

Table 3.   Emission factors for transfer and storage of petroleum products.

|  | Emission Factors by Process Type (Range is shown in parenthesis) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product Type | Tank Breathing Losses (lb VOC per 1000 gal·year storage) | Tank Working Losses (lb VOC per 1000 gal transferred) | Truck/Railcar Loading (lb VOC per 1000 gal transferred) | Truck Transit – Full (lb VOC per 1000 gal transferred) | Truck Transit – Empty (lb VOC per 1000 gal transferred) |
| Finished Motor Gasoline (RVP = 7) | 16.5 | 5.7 | 8.2 (4 to 12.4) | 0.005 | 0.055 |
| Distillate Fuel Oil | 0.035 (0.29 to 0.4) | 0.02 | 0.022 (0.014 to 0.03) | 0 | 0 |
| Kerosene-Type Jet Fuel | 0.3 | 0.03 | 0.028 (0.016 to 0.040) | 0.0015a | 0.016 |
| Residual Fuel Oil | 0 | 0 | 0.0002 (0.0001 to 0.0003) | 0 | 0 |
| Kerosene | 0.44 | 0 | 0.017 | 0 | 0 |

aRoughly estimated from the information listed in Table 4.

Table 4.   Ratio comparisons of all available emission factors for transfer and storage of motor gasoline and jet fuel.

| Product Type | Tanker/Ship/Barge Loading–Lowest Emission Factor) (lb VOC per 1000 gal transferred) | Tanker/Ship/Barge Loading–Highest Emission Factor) (lb VOC per 1000 gal transferred) | Tanker Transit (lb VOC per 1000 gal transferred) | Storage at refineries (lb VOC per 1000 gal·year storage) | Transfer at refineries (lb VOC per 1000 gal transferred) |
| --- | --- | --- | --- | --- | --- |
| Finished Motor Gasoline (RVP = 7) | 0.7 | 3.9 | 140.4 | 30.5 | 16.5 |
| Kerosene-Type Jet Fuel | 0.5 | 1.2 | 57 | 8.8 | 2.5 |
| Ratio of Gasoline Emission Factor to Jet Fuel Emission Factor | 1.4 | 3.3 | 2.5 | 3.5 | 6.6 |
| Average Ratio: | 3.4 |  |  |  |  |

Table 4 illustrates the scheme that was used to pair emission factors from Table 3 with activity parameters from Tables 1 and 2.  In addition, Table 4 lists spatial surrogates that were used to disaggregate emissions to the county level.  Following are example calculations to estimate emissions due to storage and transfer of gasoline alone in California.  Tables 5 through 7 list the results of similar calculations carried out and summed for all types of petroleum products.  For CCOSII counties that are split between air basins, emissions should be disaggregated to air basins according to the distribution of petroleum wholesalers.

For gasoline, storage tank breathing losses are calculated from the average quantity of gasoline stored at bulk plants and terminals in California.  In year 2000 on average, 7556 thousand barrels of gasoline were held in storage at any point during the year.

7556 thousand bbl·yr  42 gal/bbl 16.5 lb VOC/thousand gal·yr  ton/2000

= 2,600 tons VOC of gasoline storage tank breathing losses

Storage tank working losses and tank car/truck loading losses are calculated from the average quantity of gasoline sold by prime suppliers in California.  On average during 2000, 1,233,000 thousand gallons of gasoline were sold per month.

1,233,000 thousand gal/mo  12 mo/yr  5.7 lb VOC/thousand gal  ton/2000 lb

= 42,200 tons VOC of gasoline storage tank working losses

1,233,000 thousand gal/mo  12 mo/yr  8.2 lb/thousand gal  ton/2000 lb

= 60,700 tons VOC of gasoline tank car/truck loading emissions

Inbound transit losses from tank cars and trucks are calculated from the estimated quantities of gasoline transported to bulk plants and terminals by those modes of transportation.  It was estimated that 315,083 thousand gallons of gasoline per month were transported to bulk plants and terminals by tank trucks and 3734 thousand gallons per month were transported by rail cars.  Because trucks and rail cars are fully loaded on route to bulk stations or terminals and are empty on route from the bulk station or terminal, emission factors for empty and full transit are both applied to account for both halves of the round trip.

(315,083 + 3734) thousand gal/mo  12 mo/yr  (0.005 + 0.055) lb VOC/thousand gal

 ton/2000 lb

= 110 tons VOC of inbound gasoline tank car/truck transit emissions

Outbound transit losses from trucks are calculated from the estimated quantity of gasoline transported from bulk plants and terminals, which is taken to be equal to the quantity of gasoline sold by prime suppliers in California (1,233,000 thousand gallons per month).  Because trucks and rail cars are empty on route to bulk stations or terminals and are fully loaded on route from the bulk station or terminal, emission factors for empty and full transit are both applied to account for both halves of the round trip.

1,233,000 thousand gal/mo  12 mo/yr  (0.005+0.055) lb/thousand gal  ton/2000 lb

= 440 tons VOC of outbound gasoline tank car/truck transit emissions

Table 5.   Emission factors, activity parameters, and spatial surrogates selected to estimate emissions

for petroleum storage and marketing.

| Emissions Source Category | Emission Factor | Activity Parameter | Spatial Surrogate |
| --- | --- | --- | --- |
| Bulk plants/terminals – tank breathing losses | Tank breathing losses | Estimated quantities of petroleum products stocked in California | Bulk terminal/plant employmenta |
| Bulk plants/terminals – tank working losses | Tank working losses | Estimated quantities of petroleum products sold in California | Bulk terminal/plant employment (NAICS 422710)a |
| Tank cars and trucks – working losses (loading at bulk terminal/station) | Truck/railcar loading | Estimated quantities of petroleum products sold in California | Bulk terminal/plant employment (NAICS 422710) a |
| Tank cars and trucks inbound transit to bulk terminal/station (arrives full, leaves empty) | Truck transit-full plus Truck transit-empty | Estimated quantities of petroleum products transported to bulk terminals/stations via truck or railcar in California | 50% bulk terminal/plant employment (NAICS 422710) a and 50% refinery employment (NAICS 32410)a |
| Tank cars and trucks outbound transit to retail outlet (arrives empty, leaves full) | Truck transit-full Plus Truck transit-empty | Estimated quantities of petroleum products sold in California | 50% bulk terminal/plant employment (NAICS 422710) a and 50% gasoline fuel sales by countyb |
| Bulk gasoline storage and transfer – jet fuel – tank working losses | Tank working losses – jet fuel | Estimated quantities of aviation and jet fuels stocked in California | Number of enplanements by countyc |
| Bulk gasoline storage and transfer – jet fuel – tank breathing losses | Tank breathing losses – jet fuel | Estimated quantities of aviation and jet fuels sold in California | Number of enplanements by countyc |

a US Census Bureau, 2001b

b California Energy Commission, 2001

c Federal Aviation Administration, 1997

Table 6.   Year-2000 VOC emissions due to petroleum storage and transfer at

bulk plants and terminals in the State of California.

| Source Category | Emissions (tons VOC per year) |
| --- | --- |
| Bulk plants/terminals-breathing losses | 2,637 |
| Bulk plants/terminals-working losses | 42,200 |
| Tank cars and trucks-working losses (loading) | 60,699 |
| Tank cars and trucks transit-to bulk terminal/plant | 133 |
| Tank cars and trucks transit-to retail outlet | 444 |
| Bulk storage and transfer-jet fuel-working | 48 |
| Bulk storage and transfer-jet fuel-breathing | 4 |

| Table 7. Year-2000 activity data for petroleum storage and transfer at bulk plants and terminals by California county. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| County | Jet Fuel Tanks –Storage (1000 bbl·yr) | Jet Fuel Tanks –Transfer (1000 gal/yr) | Tank Truck Transit  Inbound + Outbound (1000 gal/yr)* | Bulk Plants/ Terminals – Storage (1000 bbl·yr)* | Bulk Plants/ Terminals & Tank Cars – Transfer/ Loading (1000 gal/yr)* |
| Alameda | 5,317 | 190,353 | 475,765 | 1,531 | 59,542 |
| Alpine | 0 | 0 | 715 | 0 | 0 |
| Amador | 0 | 0 | 8,918 | 0 | 0 |
| Butte | 23 | 822 | 190,981 | 4,544 | 176,783 |
| Calaveras | 0 | 0 | 40,352 | 988 | 38,425 |
| Colusa | 0 | 0 | 115,376 | 3,496 | 136,016 |
| Contra Costa | 1 | 37 | 1,259,331 | 31,397 | 1,221,401 |
| Del Norte | 8 | 275 | 114,579 | 3,496 | 136,016 |
| El Dorado | 0 | 13 | 94,082 | 1,619 | 62,970 |
| Fresno | 622 | 22,252 | 407,324 | 7,047 | 274,142 |
| Glenn | 0 | 1 | 107,915 | 3,237 | 125,941 |
| Humboldt | 98 | 3,495 | 105,465 | 2,250 | 87,516 |
| Imperial | 29 | 1,051 | 69,567 | 1,205 | 46,887 |
| Inyo | 0 | 0 | 39,435 | 988 | 38,425 |
| Kern | 120 | 4,313 | 899,979 | 23,511 | 914,607 |
| Kings | 0 | 0 | 250,333 | 7,282 | 283,292 |
| Lake | 0 | 0 | 141,474 | 4,127 | 160,562 |
| Lassen | 0 | 0 | 58,424 | 1,619 | 62,970 |
| Los Angeles | 35,052 | 1,254,904 | 7,563,209 | 160,674 | 6,250,499 |
| Madera | 0 | 0 | 154,604 | 4,127 | 160,562 |
| Marin | 0 | 0 | 103,646 | 569 | 22,126 |
| Mariposa | 0 | 0 | 35,233 | 988 | 38,425 |
| Mendocino | 0 | 0 | 213,776 | 6,020 | 234,200 |
| Merced | 2 | 78 | 221,585 | 5,558 | 216,228 |
| Modoc | 0 | 0 | 226,347 | 7,184 | 279,475 |
| Mono | 0 | 0 | 36,324 | 988 | 38,425 |
| Monterey | 265 | 9,497 | 155,564 | 1,629 | 63,388 |
| Napa | 0 | 10 | 87,421 | 1,619 | 62,970 |
| Nevada | 0 | 11 | 77,503 | 1,619 | 62,970 |
| Orange | 4,064 | 145,488 | 1,859,467 | 31,679 | 1,232,354 |
| Placer | 0 | 0 | 105,114 | 863 | 33,557 |
| Plumas | 0 | 0 | 37,040 | 988 | 38,425 |
| Riverside | 616 | 22,049 | 453,269 | 2,235 | 86,931 |
| Sacramento | 1,948 | 69,756 | 688,231 | 11,641 | 452,869 |
| San Benito | 0 | 0 | 42,106 | 988 | 38,425 |
| San Bernardino | 3,539 | 126,691 | 1,042,886 | 19,541 | 760,165 |
| San Diego | 7,776 | 278,382 | 1,246,451 | 14,455 | 562,335 |
| San Francisco | 0 | 0 | 1,077,787 | 26,058 | 1,013,714 |
| San Joaquin | 1 | 25 | 488,155 | 11,140 | 433,350 |
| San Luis Obispo | 142 | 5,096 | 276,145 | 6,527 | 253,922 |
| San Mateo | 20,804 | 744,792 | 636,393 | 12,096 | 470,558 |
| Santa Barbara | 423 | 15,157 | 239,857 | 4,028 | 156,689 |
| Santa Clara | 5,535 | 198,141 | 965,985 | 12,659 | 492,452 |
| Santa Cruz | 0 | 2 | 102,186 | 988 | 38,425 |
| Shasta | 77 | 2,763 | 107,068 | 1,959 | 76,214 |
| Sierra | 0 | 0 | 1,148 | 0 | 0 |
| Siskiyou | 0 | 2 | 123,080 | 3,500 | 136,167 |
| Solano | 4 | 141 | 407,332 | 9,708 | 377,659 |
| Sonoma | 31 | 1,096 | 240,660 | 3,388 | 131,785 |
| Stanislaus | 25 | 905 | 182,144 | 2,389 | 92,941 |
| Sutter | 0 | 0 | 49,530 | 988 | 38,425 |
| Tehama | 0 | 1 | 65,556 | 1,619 | 62,970 |
| Trinity | 0 | 0 | 33,809 | 988 | 38,425 |
| Tulare | 6 | 205 | 145,641 | 2,228 | 86,670 |
| Tuolumne | 0 | 1 | 84,092 | 2,250 | 87,516 |
| Ventura | 37 | 1,320 | 284,412 | 2,483 | 96,589 |
| Yolo | 1,948 | 69,741 | 251,276 | 6,534 | 254,185 |
| Yuba | 0 | 1 | 141,371 | 4,127 | 160,562 |
| Total | 88,514 | 3,168,864 | 24,633,418 | 487,357 | 18,959,042 |

| Table 7. Year-2000 estimated emissions for petroleum storage and transfer at bulk plants and terminals by California county. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| County | Jet Fuel Tanks –Breathing | Jet Fuel Tanks –Working | Truck Transit | Bulk Plants/ Terminals – Breathing* | Bulk Plants/ Terminals – Working* | Tank Truck Loading* |
| Alameda | 0.2 | 3 | 13.93 | 8 | 133 | 191 |
| Alpine | 0.0 | 0 | 0.02 | 0 | 0 | 0 |
| Amador | 0.0 | 0 | 0.27 | 0 | 0 | 0 |
| Butte | 0.0 | 0 | 4.20 | 25 | 393 | 566 |
| Calaveras | 0.0 | 0 | 0.87 | 5 | 86 | 123 |
| Colusa | 0.0 | 0 | 2.27 | 19 | 303 | 435 |
| Contra Costa | 0.0 | 0 | 27.19 | 170 | 2,719 | 3,910 |
| Del Norte | 0.0 | 0 | 2.25 | 19 | 303 | 435 |
| El Dorado | 0.0 | 0 | 2.29 | 9 | 140 | 202 |
| Fresno | 0.0 | 0 | 9.90 | 38 | 610 | 878 |
| Glenn | 0.0 | 0 | 2.14 | 18 | 280 | 403 |
| Humboldt | 0.0 | 0 | 2.41 | 12 | 195 | 280 |
| Imperial | 0.0 | 0 | 1.69 | 7 | 104 | 150 |
| Inyo | 0.0 | 0 | 0.85 | 5 | 86 | 123 |
| Kern | 0.0 | 0 | 19.05 | 127 | 2,036 | 2,928 |
| Kings | 0.0 | 0 | 5.04 | 39 | 631 | 907 |
| Lake | 0.0 | 0 | 2.84 | 22 | 357 | 514 |
| Lassen | 0.0 | 0 | 1.21 | 9 | 140 | 202 |
| Los Angeles | 1.5 | 19 | 173.16 | 869 | 13,913 | 20,011 |
| Madera | 0.0 | 0 | 3.24 | 22 | 357 | 514 |
| Marin | 0.0 | 0 | 2.95 | 3 | 49 | 71 |
| Mariposa | 0.0 | 0 | 0.72 | 5 | 86 | 123 |
| Mendocino | 0.0 | 0 | 4.37 | 33 | 521 | 750 |
| Merced | 0.0 | 0 | 4.77 | 30 | 481 | 692 |
| Modoc | 0.0 | 0 | 4.34 | 39 | 622 | 895 |
| Mono | 0.0 | 0 | 0.75 | 5 | 86 | 123 |
| Monterey | 0.0 | 0 | 4.15 | 9 | 141 | 203 |
| Napa | 0.0 | 0 | 2.09 | 9 | 140 | 202 |
| Nevada | 0.0 | 0 | 1.79 | 9 | 140 | 202 |
| Orange | 0.2 | 2 | 45.34 | 171 | 2,743 | 3,945 |
| Placer | 0.0 | 0 | 2.89 | 5 | 75 | 107 |
| Plumas | 0.0 | 0 | 0.77 | 5 | 86 | 123 |
| Riverside | 0.0 | 0 | 12.99 | 12 | 193 | 278 |
| Sacramento | 0.1 | 1 | 16.81 | 63 | 1,008 | 1,450 |
| San Benito | 0.0 | 0 | 0.93 | 5 | 86 | 123 |
| San Bernardino | 0.2 | 2 | 24.80 | 106 | 1,692 | 2,434 |
| San Diego | 0.3 | 4 | 32.79 | 78 | 1,252 | 1,800 |
| San Francisco | 0.0 | 0 | 23.56 | 141 | 2,256 | 3,245 |
| San Joaquin | 0.0 | 0 | 10.90 | 60 | 965 | 1,387 |
| San Luis Obispo | 0.0 | 0 | 6.09 | 35 | 565 | 813 |
| San Mateo | 0.9 | 11 | 15.08 | 65 | 1,047 | 1,507 |
| Santa Barbara | 0.0 | 0 | 5.87 | 22 | 349 | 502 |
| Santa Clara | 0.2 | 3 | 24.90 | 68 | 1,096 | 1,577 |
| Santa Cruz | 0.0 | 0 | 2.75 | 5 | 86 | 123 |
| Shasta | 0.0 | 0 | 2.56 | 11 | 170 | 244 |
| Sierra | 0.0 | 0 | 0.03 | 0 | 0 | 0 |
| Siskiyou | 0.0 | 0 | 2.50 | 19 | 303 | 436 |
| Solano | 0.0 | 0 | 8.95 | 53 | 841 | 1,209 |
| Sonoma | 0.0 | 0 | 6.12 | 18 | 293 | 422 |
| Stanislaus | 0.0 | 0 | 4.69 | 13 | 207 | 298 |
| Sutter | 0.0 | 0 | 1.15 | 5 | 86 | 123 |
| Tehama | 0.0 | 0 | 1.42 | 9 | 140 | 202 |
| Trinity | 0.0 | 0 | 0.67 | 5 | 86 | 123 |
| Tulare | 0.0 | 0 | 3.64 | 12 | 193 | 277 |
| Tuolumne | 0.0 | 0 | 1.76 | 12 | 195 | 280 |
| Ventura | 0.0 | 0 | 7.77 | 13 | 215 | 309 |
| Yolo | 0.1 | 1 | 5.33 | 35 | 566 | 814 |
| Yuba | 0.0 | 0 | 2.84 | 22 | 357 | 514 |
| Total | 4 | 48 | 577 | 2,637 | 42,200 | 60,699 |

### Emissions Controls

We verified that many of the CCOSII counties have requirements for emissions controls on petroleum storage tanks and petroleum loading operations.  The statuses of these requirements are summarized in Table 8.  For these counties, we will arbitrarily assume a control efficiency of 85% on related breathing, working, and/or loading emissions (where applicable).

Table 8.   Statuses of emissions control technologies for bulk storage and transfer in

CCOSII participating counties.

| County | Vapor Recovery on Loading | Storage Tank Controls |
| --- | --- | --- |
| Amador |  | Rule 213 |
| Butte | Rule 214 | Rule 215 |
| Calaveras |  | Rule 2123 |
| Colusa |  |  |
| El Dorado | Rules 238, 244 |  |
| Glenn | Article IV, Section 83 |  |
| Mariposa |  | Rule 213 |
| Mendocino |  |  |
| Northern Sierra (Nevada, Plumas, and Sierra) | Rule 218 | Rule 219 |
| Placer | Rule 215 | Rule 212 |
| Sacramento | Rules 447 and 448 | Rule 446 |
| Shasta | Rule 3:3 | Rule 3:3 |
| Feather River (Sutter and Yuba) | Rule 3.8 | Rule 3.8 |
| Tehama | Rule 4:36 |  |
| Tuolumne |  | Rule 213 |
| Yolo-Solano | Rule 2.21 | Rule 2.21 |

### Temporal Allocation

Emissions from gasoline fuel are overwhelmingly larger than emissions from other types of petroleum products.  Therefore, the seasonal distributions of working losses and transit may be approximated from sales of gasoline (Table 9).

Table 9.   Monthly distribution of prime supplier sales of

gasoline and aviation fuels in California for the

year 2000 (EIA, 2000/2001).

|  | Proportion of Total Annual Sales |  |
| --- | --- | --- |
| Month | Gasoline | Aviation Fuels |
| January | 7.2% | 7.0% |
| Feb | 8.2% | 7.5% |
| March | 8.7% | 9.1% |
| April | 8.8% | 8.3% |
| May | 8.6% | 8.2% |
| June | 8.2% | 8.6% |
| July | 8.0% | 8.9% |
| August | 8.8% | 9.0% |
| September | 8.4% | 8.8% |
| Oct | 8.4% | 8.3% |
| Nov | 8.3% | 8.3% |
| Dec | 8.5% | 7.9% |

Breathing losses vary with weather conditions.  Average monthly climate variables for Sacramento are listed in Table 10.  These data were input to the EPA’s emissions estimation method for storage tanks (EPA,1998) to estimate breathing losses from a hypothetical fixed-roof gasoline storage tank (where Reid vapor pressure = 8.5 psi, and API gravity = 57).  The results were used to estimate the monthly distribution of breathing losses shown in Table 10.

Table 10.   Average climate data for Sacramento and seasonal distribution of

storage tank breathing losses.

|  | Temperature (degrees Fahrenheit) |  | Solar Insolation (Btu/ft2/day) Average Daily Maximum |  |
| --- | --- | --- | --- | --- |
| Month | Average Daily Maximum | Average Daily Minimum |  | Proportion of Annual Total Breathing Losses |
| Jan | 53.1 | 37.8 | 982.9 | 3.7% |
| Feb | 59.7 | 41.1 | 1363.4 | 5.3% |
| Mar | 64.4 | 42.9 | 1648.8 | 6.6% |
| Apr | 71.7 | 45.9 | 1870.7 | 8.3% |
| May | 79.8 | 50.5 | 1902.4 | 9.7% |
| Jun | 87.2 | 55.2 | 1902.4 | 11.1% |
| Jul | 92.8 | 58 | 1997.5 | 12.7% |
| Aug | 91.5 | 57.7 | 2060.9 | 12.6% |
| Sep | 87.6 | 55.8 | 2092.6 | 11.8% |
| Oct | 77.9 | 50.2 | 1839.0 | 9.1% |
| Nov | 63.6 | 42.6 | 1236.6 | 5.5% |
| Dec | 53.5 | 38 | 919.5 | 3.7% |

We were unable to identify data sources to estimate diurnal patterns of emissions for bulk storage and transport of petroleum fuels.

## Natural Gas Transmission

The natural gas transmission category includes fugitive losses from transmission pipelines, compressor stations, underground natural gas storage facilities, liquefied natural gas storage facilities, and local distribution systems. Populations of natural gas storage fields and compressor stations are available from the EIA and the California Department of Conservation (Tables 11 and 12).  (There were no liquefied natural gas storage facilities in the CCOSII area.)  Statewide mileages of transmission and distribution pipeline are reported by the California Public Utility Commission and the Department of Transportation’s Office of Pipeline Safety.  The Department of Transportation (2001) reported that 11,221 miles of transmission pipeline, 92,012 miles of distribution pipeline, and 11.1 million service connections were in operation in California in 2000.  In addition, the EIA (2001c) publishes a GIS system of intrastate transmission pipelines.  However, the EIA database only accounts for about 60% of California’s total statewide transmission pipeline length.  Therefore, the EIA database was used only to allocate statewide transmission pipeline lengths by county (Table 13).  Table 14 tabulates the distribution pipeline mileages and the numbers of service connections by construction material.  The statistics for distribution pipelines were disaggregated to the county level according to the number of housing units reported by the US Census (US Census, 2001c).

Table 11.  Underground natural gas storage fields in California (California Department of

Conservation, 2001; EIA 2001d).

| Storage Field Name | County Name |
| --- | --- |
| Los Medanos | Contra Costa |
| Lost Hills | Kern |
| Playa Del Rey | Los Angeles |
| Whittier | Los Angeles |
| Montebello | Los Angeles |
| Aliso Canyon | Los Angeles |
| Honor Rancho | Los Angeles |
| McDonald Island | San Joaquin |
| La Goleta | Santa Barbara |
| Kirby Hill | Solano |
| Pleasant Creek | Yolo |
| Wild Goose | Butte |

Table 12.   Natural gas compressor stations in California (EIA, 2001c).

| Station Name | Associated Pipeline Co. | City | County |
| --- | --- | --- | --- |
| Delevan | Pacific Gas & Electric Co. | Delevan | Colusa |
| Wheeler Ridge | Southern California Gas Co. | Wheeler Ridge | Kern |
| Kettleman | Pacific Gas & Electric Co. |  | Kings |
| Avenal | Southern California Gas Co. | Avenal | Kings |
| Aliso Canyon | Southern California Gas Co. | Los Angeles | Los Angeles |
| Honor Rancho | Southern California Gas Co. | Santa Clara | Los Angeles |
| Montebello | Southern California Gas Co. | Los Angeles | Los Angeles |
| Playa Del Rey | Southern California Gas Co. | Los Angeles | Los Angeles |
| Whittier | Southern California Gas Co. | Los Angeles | Los Angeles |
| Dana Point | Southern California Gas Co. | San Clemente | Orange |
| Blythe | Southern California Gas Co. | Blythe | Riverside |
| Cactus City | Southern California Gas Co. | Indio | Riverside |
| Desert Center | Southern California Gas Co. | Desert Center | Riverside |
| Moreno | Southern California Gas Co. | Beaumont | Riverside |
| Kelso | Southern California Gas Co. | Kelso | San Bernardino |
| Newberry Springs | Southern California Gas Co. | Newberry | San Bernardino |
| North Needles | Southern California Gas Co. | Needles | San Bernardino |
| South Needles | Southern California Gas Co. | Lobecks Pass | San Bernardino |
| Gaviota | Southern California Gas Co. | Goleta | Santa Barbara |
| Goleta | Southern California Gas Co. | Goleta | Santa Barbara |
| Ventura | Southern California Gas Co. | Ventura | Santa Barbara |
| Santa Rosa | Pacific Gas & Electric Co. |  | Sonoma |
| Gerber | Pacific Gas & Electric Co. | Gerber | Tehama |

Table 13. Mileage of transmission pipeline per county

for CCOS II participating counties.

| County | Proportion of State Mileage | Mileage |
| --- | --- | --- |
| Amador | 0.3% | 29 |
| Butte | 2.2% | 250 |
| Calaveras | 0.2% | 26 |
| Colusa | 2.6% | 296 |
| El Dorado | 0.0% | 3 |
| Glenn | 1.8% | 206 |
| Mendocino | 0.8% | 87 |
| Nevada | 0.2% | 23 |
| Placer | 0.7% | 78 |
| Sacramento | 2.3% | 259 |
| Shasta | 2.2% | 246 |
| Solano | 2.5% | 282 |
| Sutter | 1.4% | 153 |
| Tehama | 2.3% | 253 |
| Yolo | 1.9% | 217 |
| Yuba | 0.8% | 84 |
| California | 100% | 11,221 |

Table 14.   Distribution pipeline mileages and service connections by construction material

(Department of Transportation, 2001).

| Pipeline Mileage |  |  |  | Number of Service Connections |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Cast Iron | Unprotected Steel | Protected Steel | Plastic Main | Total | Unprotected Steel | Protected Steel |
| 359 | 11,212 | 42,077 | 38,364 | 7,530,457 | 235,064 | 3,175,142 |

### Emissions

Methane emissions were calculated according to emission factors that have been recommended by the Emission Inventory Improvement Program (Table 15).  TOC and VOC emissions were calculated in a manner that is consistent with analyses of natural gas compositions for San Francisco and Los Angeles (Perry’s Chemical Engineers’ Handbook, 1983) (Table 16).  VOCs were estimated to be non-methane, non-ethane hydrocarbons.

Table 15.   Methane mission factors for natural gas transmission and distribution systems

(Emission Inventor Improvement Program, 1999).

| Emissions Source | Emission Factor | units (annual) |
| --- | --- | --- |
| Compressor Stations | 982 | tons per station |
| Underground Gas Storage Fields | 1008 | tons per field |
| Transmission Pipeline | 0.75 | tons per mile |
| Cast Iron Pipeline | 5.10 | tons per mile |
| Unprotected Steel Pipeline | 2.38 | tons per mile |
| Protected Steel Pipeline | 0.12 | tons per mile |
| Plastic Main Pipeline | 0.46 | tons per mile |
| Services | 0.015 | tons per service |
| Unprotected Steel Services | 0.036 | tons per service |
| Protected Steel Services | 0.0039 | tons per service |

Table 16.  Natural gas analyses for San Francisco and Los Angeles

(Perry’s Chemical Engineers’ Handbook, 1983).

|  | Range |  |  |
| --- | --- | --- | --- |
| Species | Low | High | Mid-point |
| Methane | 86.50% | 88.69% | 87.60% |
| Ethane | 7.01% | 8.00% | 7.51% |
| Propane | 1.90% | 1.93% | 1.92% |
| Butane | 0.28% | 0.30% | 0.29% |
| Pentanes | 0.03% | 0.10% | 0.07% |
| Hexanes | 0.00% | 0.10% | 0.05% |
| CO2 | 0.50% | 0.62% | 0.56% |
| N2 | 1.43% | 2.60% | 2.02% |

Emissions for Tehama County were estimated as follows.  Tehama County has 1 compressor station, which is estimated to emit 982 tons per year, and no underground storage fields.  Emission factors were applied to mileages of transmission and distribution pipeline and numbers of service connectors as follows.

253 miles transmission pipeline  0.75 tons per mile = 190 tons per year

1 mile cast iron distribution pipeline  5.1 tons per mile = 5.1 tons per year

22 miles unprotected steel distribution pipeline  2.38 tons per mile = 52.4 tons per year

81 miles protected steel distribution pipeline  0.12 tons per mile = 9.7 tons per year

74 miles plastic pipeline  0.46 tons per mile = 34.0 tons per year

14,517 total service connectors  0.015 tons per service = 217.8 tons per year

453 unprotected steel service connectors  0.036 tons per service = 16.3 tons per year

6121 protected steel service connectors  0.0039 tons per service = 23.9 tons per year

Methane emissions are estimated as the sum total of emissions from the compressor station, pipelines, and service connections (1535 tons per year).  VOC and TOG emissions were estimated according to the speciation profile for natural gas (see Table 14).

1535 tons CH4 per year   (1.92% + 0.29% + 0.07% + 0.05%)  87.6%

= 41 VOC tons per year

1535 tons CH4 per year  100%  [1  (0.56% + 2.02%)]= 1576 tons TOG per year

Emission estimates for natural gas transmission are presented in Tables 17 and 18.  For the CCOSII counties that are split between air basins – El Dorado, Placer, and Solano – emissions from distribution pipelines should be geographically disaggregated by housing units; emissions from transmission pipelines should be disaggregated by pipeline length; and emissions from compressor stations or storage fields should be assigned to the locations of these facilities.

### Temporal Allocation

Emissions from natural gas transmission and distribution systems are due to leaking pipelines and equipment.  Although natural gas consumption varies over the course of the year, continuous pressure must be maintained in natural gas lines.  Therefore, it is recommended that emissions are distributed evenly throughout the year.

Table 17. Year-2000 estimated emissions from natural gas transmission

for CCOSII participating counties.

| County | Pipe-line Mileage | No. Compressors | No. Storage Fields | Emissions (tpy) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Methane (CH4) |  |  |  | VOC | TOC |
|  |  |  |  | From Pipeline | From Compressor | From Storage Fields | Total | Total | Total |
| Amador | 29 |  |  | 22 |  |  | 22 | 1 | 22 |
| Butte | 250 |  | 1 | 187 |  | 1,008 | 1,195 | 32 | 1,227 |
| Calaveras | 26 |  |  | 20 |  |  | 20 | 1 | 20 |
| Colusa | 296 | 1 |  | 222 | 982 |  | 1,204 | 32 | 1,236 |
| El Dorado | 3 |  |  | 2 |  |  | 2 | 0 | 2 |
| Glenn | 206 |  |  | 154 |  |  | 154 | 4 | 158 |
| Mendocino | 87 |  |  | 65 |  |  | 65 | 2 | 67 |
| Nevada | 23 |  |  | 18 |  |  | 18 | 0 | 18 |
| Placer | 78 |  |  | 58 |  |  | 58 | 2 | 60 |
| Sacramento | 259 |  |  | 194 |  |  | 194 | 5 | 199 |
| Shasta | 246 |  |  | 184 |  |  | 184 | 5 | 189 |
| Solano | 282 |  | 1 | 212 |  | 1,008 | 1,220 | 32 | 1,252 |
| Sutter | 153 |  |  | 115 |  |  | 115 | 3 | 118 |
| Tehama | 253 | 1 |  | 190 | 982 |  | 1,172 | 31 | 1,203 |
| Yolo | 217 |  | 1 | 163 |  | 1,008 | 1,171 | 31 | 1,202 |
| Yuba | 84 |  |  | 63 |  |  | 63 | 2 | 65 |
| State Total | 11,221 | 2 | 3 | 1,869 | 1,964 | 4,032 | 6,857 | 182 | 7,039 |

| Table 18. Activity and annual emissions data for natural gas distribution pipelines in California. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mileage of Distribution Pipeline |  |  |  | No. Servicesd |  |  | Emissions (tpy) |  |  |  |
| County | No. Housing Units | Cast Iron | Un-protected Steel | Protected Steel | Plastic Main | Total | Un-protected Steel | Protected Steel | CH4 | VOC | TOC |  |
| Alameda | 540,183 | 16 | 496 | 1,861 | 1,697 | 333,031 | 10,396 | 140,419 | 8,334 | 221 | 8,554 |  |
| Alpine | 1,514 | 0 | 1 | 5 | 5 | 933 | 29 | 394 | 23 | 1 | 24 |  |
| Amador | 15,035 | 0 | 14 | 52 | 47 | 9,269 | 289 | 3,908 | 232 | 6 | 238 |  |
| Butte | 85,523 | 3 | 79 | 295 | 269 | 52,726 | 1,646 | 22,231 | 1,319 | 35 | 1,354 |  |
| Calaveras | 22,946 | 1 | 21 | 79 | 72 | 14,147 | 442 | 5,965 | 354 | 9 | 363 |  |
| Colusa | 6,774 | 0 | 6 | 23 | 21 | 4,176 | 130 | 1,761 | 105 | 3 | 107 |  |
| Contra Costa | 354,577 | 10 | 325 | 1,221 | 1,114 | 218,602 | 6,824 | 92,171 | 5,470 | 145 | 5,615 |  |
| Del Norte | 10,434 | 0 | 10 | 36 | 33 | 6,433 | 201 | 2,712 | 161 | 4 | 165 |  |
| El Dorado | 71,278 | 2 | 65 | 246 | 224 | 43,944 | 1,372 | 18,529 | 1,100 | 29 | 1,129 |  |
| Fresno | 270,767 | 8 | 249 | 933 | 850 | 166,932 | 5,211 | 70,385 | 4,177 | 111 | 4,288 |  |
| Glenn | 9,982 | 0 | 9 | 34 | 31 | 6,154 | 192 | 2,595 | 154 | 4 | 158 |  |
| Humboldt | 55,912 | 2 | 51 | 193 | 176 | 34,471 | 1,076 | 14,534 | 863 | 23 | 885 |  |
| Imperial | 43,891 | 1 | 40 | 151 | 138 | 27,059 | 845 | 11,409 | 677 | 18 | 695 |  |
| Inyo | 9,042 | 0 | 8 | 31 | 28 | 5,575 | 174 | 2,350 | 139 | 4 | 143 |  |
| Kern | 231,564 | 7 | 213 | 798 | 727 | 142,763 | 4,456 | 60,194 | 3,572 | 95 | 3,667 |  |
| Kings | 36,563 | 1 | 34 | 126 | 115 | 22,542 | 704 | 9,504 | 564 | 15 | 579 |  |
| Lake | 32,528 | 1 | 30 | 112 | 102 | 20,054 | 626 | 8,456 | 502 | 13 | 515 |  |
| Lassen | 12,000 | 0 | 11 | 41 | 38 | 7,398 | 231 | 3,119 | 185 | 5 | 190 |  |
| Los Angeles | 3,270,909 | 96 | 3,002 | 11,268 | 10,273 | 2,016,566 | 62,947 | 850,265 | 50,461 | 1,336 | 51,798 |  |
| Madera | 40,387 | 1 | 37 | 139 | 127 | 24,899 | 777 | 10,499 | 623 | 17 | 640 |  |
| Marin | 104,990 | 3 | 96 | 362 | 330 | 64,728 | 2,020 | 27,292 | 1,620 | 43 | 1,663 |  |
| Mariposa | 8,826 | 0 | 8 | 30 | 28 | 5,441 | 170 | 2,294 | 136 | 4 | 140 |  |
| Mendocino | 36,937 | 1 | 34 | 127 | 116 | 22,772 | 711 | 9,602 | 570 | 15 | 585 |  |
| Merced | 68,373 | 2 | 63 | 236 | 215 | 42,153 | 1,316 | 17,773 | 1,055 | 28 | 1,083 |  |
| Modoc | 4,807 | 0 | 4 | 17 | 15 | 2,964 | 93 | 1,250 | 74 | 2 | 76 |  |
| Mono | 11,757 | 0 | 11 | 41 | 37 | 7,248 | 226 | 3,056 | 181 | 5 | 186 |  |
| Monterey | 131,708 | 4 | 121 | 454 | 414 | 81,200 | 2,535 | 34,237 | 2,032 | 54 | 2,086 |  |
| Napa | 48,554 | 1 | 45 | 167 | 153 | 29,934 | 934 | 12,621 | 749 | 20 | 769 |  |
| Nevada | 44,282 | 1 | 41 | 153 | 139 | 27,301 | 852 | 11,511 | 683 | 18 | 701 |  |
| Orange | 969,484 | 28 | 890 | 3,340 | 3,045 | 597,702 | 18,657 | 252,015 | 14,956 | 396 | 15,353 |  |
| Placer | 107,302 | 3 | 98 | 370 | 337 | 66,153 | 2,065 | 27,893 | 1,655 | 44 | - |  |
| Plumas | 13,386 | 0 | 12 | 46 | 42 | 8,253 | 258 | 3,480 | 207 | 5 | - |  |
| Riverside | 584,674 | 17 | 537 | 2,014 | 1,836 | 360,460 | 11,252 | 151,985 | 9,020 | 239 | - |  |
| Sacramento | 474,814 | 14 | 436 | 1,636 | 1,491 | 292,730 | 9,138 | 123,427 | 7,325 | 194 | 7,519 |  |
| San Benito | 16,499 | 0 | 15 | 57 | 52 | 10,172 | 318 | 4,289 | 255 | 7 | 261 |  |
| San Bernardino | 601,369 | 18 | 552 | 2,072 | 1,889 | 370,753 | 11,573 | 156,324 | 9,277 | 246 | 9,523 |  |
| San Diego | 1,040,149 | 31 | 955 | 3,583 | 3,267 | 641,268 | 20,017 | 270,384 | 16,047 | 425 | 16,472 |  |
| San Francisco | 346,527 | 10 | 318 | 1,194 | 1,088 | 213,639 | 6,669 | 90,079 | 5,346 | 142 | 5,488 |  |
| San Joaquin | 189,160 | 6 | 174 | 652 | 594 | 116,620 | 3,640 | 49,172 | 2,918 | 77 | 2,996 |  |
| San Luis Obispo | 102,275 | 3 | 94 | 352 | 321 | 63,054 | 1,968 | 26,586 | 1,578 | 42 | 1,620 |  |
| San Mateo | 260,576 | 8 | 239 | 898 | 818 | 160,649 | 5,015 | 67,736 | 4,020 | 106 | 4,126 |  |
| Santa Barbara | 142,901 | 4 | 131 | 492 | 449 | 88,101 | 2,750 | 37,147 | 2,205 | 58 | 2,263 |  |
| Santa Clara | 579,329 | 17 | 532 | 1,996 | 1,820 | 357,165 | 11,149 | 150,595 | 8,937 | 237 | 9,174 |  |
| Santa Cruz | 98,873 | 3 | 91 | 341 | 311 | 60,957 | 1,903 | 25,702 | 1,525 | 40 | 1,566 |  |
| Shasta | 68,810 | 2 | 63 | 237 | 216 | 42,422 | 1,324 | 17,887 | 1,062 | 28 | 1,090 |  |
| Sierra | 2,202 | 0 | 2 | 8 | 7 | 1,358 | 42 | 572 | 34 | 1 | 35 |  |
| Siskiyou | 21,947 | 1 | 20 | 76 | 69 | 13,531 | 422 | 5,705 | 339 | 9 | 348 |  |
| Solano | 134,513 | 4 | 123 | 463 | 422 | 82,929 | 2,589 | 34,966 | 2,075 | 55 | 2,130 |  |
| Sonoma | 183,153 | 5 | 168 | 631 | 575 | 112,917 | 3,525 | 47,610 | 2,826 | 75 | 2,900 |  |
| Stanislaus | 150,807 | 4 | 138 | 520 | 474 | 92,975 | 2,902 | 39,202 | 2,327 | 62 | 2,388 |  |
| Sutter | 28,319 | 1 | 26 | 98 | 89 | 17,459 | 545 | 7,361 | 437 | 12 | 448 |  |
| Tehama | 23,547 | 1 | 22 | 81 | 74 | 14,517 | 453 | 6,121 | 363 | 10 | 373 |  |
| Trinity | 7,980 | 0 | 7 | 27 | 25 | 4,920 | 154 | 2,074 | 123 | 3 | 126 |  |
| Tulare | 119,639 | 4 | 110 | 412 | 376 | 73,759 | 2,302 | 31,100 | 1,846 | 49 | 1,895 |  |
| Tuolumne | 28,336 | 1 | 26 | 98 | 89 | 17,470 | 545 | 7,366 | 437 | 12 | 449 |  |
| Ventura | 251,712 | 7 | 231 | 867 | 791 | 155,184 | 4,844 | 65,432 | 3,883 | 103 | 3,986 |  |
| Yolo | 61,587 | 2 | 57 | 212 | 193 | 37,969 | 1,185 | 16,009 | 950 | 25 | 975 |  |
| Yuba | 22,636 | 1 | 21 | 78 | 71 | 13,955 | 436 | 5,884 | 349 | 9 | 358 |  |
| State Total | 2,214,549 | 359 | 11,212 | 42,077 | 38,364 | 7,530,457 | 235,064 | 3,175,142 | 188,437 | 4,991 | 193,427 |  |

## References

California Board of Equalization (2001) Fuel taxes.  Published in the 2000-2001 Annual Report of the Board of Equalization.  Accessed from website http://www.boe.ca.gov/annual/statindex0001.htm.

California Department of Conservation (2001) Special query request from the database of oil and gas field operations, maintained by the Division of Oil and Gas of the California Department, Sacramento, California.  Technical services contact telephone (916) 323-1779.

California Energy Commission (2001) California retail service stations, fleet fueling facilities and private storage tank sites and other statistical data by county.  Website accessed June 2001 at http://www.energy.ca.gov/statistics/gasoline_stations/index.html.  Page last updated March 21, 2000.

Department of Transportation (2001) Distribution and transmission annual, 2000.  Office of Pipeline Safety.  http://ops.dot.gov/DT98.htm

Emission Inventory Improvement Program (2001) Technical report series, volume ii  point sources. chapter 14  uncontrolled emission factor listing for criteria air pollutants.  Chapter prepared by Eastern Research Group for the Point Sources Committee of the Emission Inventory Improvement Program.  Published by the US EPA, RTP.

Emission Inventory Improvement Program (1999) Technical report series, volume viii  greenhouse gases. chapter 3  methods for estimating methane emissions from natural gas and oil systems.  Chapter prepared by ICF Consulting for the Greenhouse Gas Committee of the Emission Inventory Improvement Program.  Published by the US EPA, RTP.

EIA (2001a)  Petroleum Supply Annual 2000, Volume 1. Stocks of crude oil and petroleum products by pad district, 2000. table 12. pad district vsupply, disposition, and ending stocks of crude oil and petroleum products, 2000 (table 30).  Report prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, June.  http://www.eia.doe.gov/oil_gas/petroleum/data_publications/petroleum_supply_annual/psa_volume1/psa_volume1.html

EIA (2001b)  Prime Supplier Report, January 2001. Prime supplier sales volumes of petroleum products by grade, pad district, and state. Report prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, January. http://www.eia.doe.gov/pub/oil_gas/petroleum/data_publications/prime_supplier_report/historical/2001/2001_01/pdf/c007.pdf

EIA (2001c) EIA's specialized natural gas geographic information system (EIAGIS-NG). Software and databases prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, January (most recent update).  To order, contact the U.S. Department of Energy, Energy Information Administration, 1000 Independence Ave S.W., Washington, DC, 20585-0644, attention Jim Tobin EI-442 (e-mail jtobin@eia.doe.gov, telephone 202-586-4835).  http://www.eia.doe.gov/pub/oil_gas/natural_gas/applications/eia_specialized_natural_gas_information_system_gis/html/contents.html

EIA (2001d) Oil and gas field code master list – 2000. Report prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, January. http://www.eia.doe.gov/oil_gas/natural_gas/data_publications/field_code_master_list/fcml.html

EIA (2000/2001) Petroleum marketing monthly. Monthly report nos. DOE/EIA-0380(2000/04) through DOE/EIA-0380(2001/03) prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, April 2000 through March 2001.

EIA (1998a) Petroleum Supply Annual 1997, Volume 1. Table 30. Stocks of crude oil and petroleum products by pad district, 1997 (table 31). In refinery, bulk terminal, and natural gas plant stocks of selected petroleum products by pad district and state, 1997.  Report prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, June.  http://www.eia.doe.gov/pub/oil_gas/petroleum/data_publications/petroleum_supply_annual/psa_volume1/historical/1997/psa_volume1_1997.html

EIA (1998b) Petroleum Marketing Annual, 1997.  Table 49.  Prime supplier sales volumes of aviation fuels, propane, and residual fuel oil by pad district and state. Report prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, December.  http://www.eia.doe.gov/pub/oil_gas/petroleum/data_publications/petroleum_marketing_annual/historical/1997/pma_1997.html

EIA (1998c)  Petroleum Marketing Monthly, March 1998.  Table 4. US refiner prices of petroleum products for resale (table 2).  In US refiner prices of petroleum products to end users. Report prepared by the Energy Information Administration Office of Oil and Gas, U.S. Department of Energy, Washington, DC, March. http://www.eia.doe.gov/pub/oil_gas/petroleum/data_publications/petroleum_marketing_monthly/historical/1998/1998_03/pmm_1998_03.html

EPA (1998) Organic liquid storage tanks.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Supplement D (August 1998), Chapter 7.1.  Report prepared by Office of Air Quality Planning and Standards of the EPA,  Research Triangle Park, NC.

EPA (1995) Transportation and marketing of petroleum liquids.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Chapter 5.2.  Report prepared by Office of Air Quality Planning and Standards of the EPA,  Research Triangle Park, NC.

Federal Aviation Administration (1997) Airport activity and development statistics  1996 enplanements by state and airport, December 18.  Accessed from http://ntl.bts.gov/DOCS/l-aads/.  (More recent data is available from http://www.faa.gov/arp/AandD-stat.htm; however, there was little difference in the relative distribution of enplanements from 1997 to 1999.)

National Climatic Data Center (1992) National solar radiation data base (NSRDB), 1961-1990, vol. l (1992).  Database maintained by the National Climatic Data Center, Asheville, NC.

Perry’s chemical engineers’ handbook, 6th edition.  (1984)  Table 9-14, Analyses of Natural Gas.  Edited by Perry R.H., Green D.W., and Maloney, J.O.  McGraw-Hill: New York.

US Census Bureau. (2000) 1997 Economic census of wholesale trade, geographic area series: California. Report no. EC97W42A-CA(RV) by the US Census Bureau, Washington, D.C., March.

US Census Bureau (2001a). 1997 economic census of wholesale trade, miscellaneous subjects. Report no. EC97W42S-SB by the US Census Bureau, Washington, D.C., February.

US Census Bureau, (2001b) CenStats Databases – County business patterns data, database maintained by the US Census Bureau, Washington, D.C., and accessed June 2001 at web site http://tier2.census.gov/dbappweb.htm.

US Census Bureau, (2001c) Census 2000 housing units, database maintained by the US Census Bureau, Washington, D.C., and accessed September 2001 at web site http://quickfacts.census.gov/hunits/.

Western Regional Climate Center (2001) Monthly climate summary for Sacramento FAA airport, average for the period 1/1/1941 through 12/31/2000.  Database maintained by the Western Regional Climate Center, Reno, Nevada, and accessed October 2001 at web site http://www.wrcc.dri.edu/summary/climsmnca.html.

* Represents the sum total of all fuel types.

* Represents the sum total of all fuel types.

Western Regional Climate Center, 2001

National Climate Data Center, 1992

Department of Transportation (2001)

EIA (2001c)

California Department of Conservation (2001)

Department of Transportation (2001)

U.S. Census Bureau (2001c) 2001 Housing Units