# Attachment I:  Miscellaneous Industrial Chemical Use and Chemical Manufacturing

This memorandum discusses the following emissions source categories.  These source categories emit VOCs through the evaporation losses of process raw materials or solvents, which are consumed in manufacturing processes.

Description	EIC Code	CES Number

Industrial Coatings (Unspecified)	230-995-9000-0000 	46748

Miscellaneous Industrial Solvent Use*	299-995-8000-0000	46862

Printing	240-995-8000-0000	66829

Fiberglass Impregnation and Fabrication	410-403-5018-0000	74674

Synthetic Rubber Manufacturing	410-402-5062-0000 	46938

General Plastics Manufacturing 	410-404-5000-0000	46946

*associated with cleaning or surface coating operations

## Miscellaneous and Unspecified Industrial Coatings and Solvents

The “Industrial Coatings–Unspecified” and the “Miscellaneous Industrial Solvent Use” categories refer to industrial coatings and associated solvents that are used by miscellaneous, undefined industries.  These categories are otherwise open-ended, but should not cover industries that are included in other ARB emissions source categories (architectural coating, auto refinishing, marine coatings, paper coatings, fabric coatings, metal furniture and fixture coatings, can and coil coatings, metal parts and products coatings, wood furniture and fixture coatings, nor printing).

Industrial coatings and associated solvents include varnishes, lacquers, paints, thinners, and surface cleaners.  We analyzed data from the US Census Bureau (1999/2000; 2001a) for industries that were reported to purchase adhesives, sealants, glues, paints, varnishes, lacquers, stains, shellacs, japans, enamels, plastics resins, printing inks, and allied products.  We estimate that within the CCOS II participating counties, the industries listed in Table 1 account for most manufacturing expenditures for the indicated product categories. Of the industries listed in Table 1, the following are recommended for inclusion in the “Industrial Coatings–Unspecified” and/or the “Miscellaneous Industrial Solvent Use” categories because they are not necessarily inventoried in other ARB area source categories.

NAICS	Manufacturing Industry

326130	Laminated plastics plate, sheet and shape manufacturing

334417	Electronic connector manufacturing

334613	Magnetic and optical recording media manufacturing

336211	Motor vehicle body manufacturing (not refinishing)

336213	Motor home manufacturing (not refinishing)

336612	Boat manufacturing

We consulted business directories to identify and locate businesses of the types listed above in Northern California (see Table 2).  We randomly contacted five of these business to get an initial idea of coating and solvent use habits.  Three of the businesses were sales offices and had no local manufacturing plants and no use of solvents, coatings, or adhesives.  One business was a permitted manufacturer of laminate countertops and reports consumptions of solvents and coatings to the local District.  The last contact was a bus manufacturer that uses adhesives (approximately 4 pounds per year), but avoids use of solvents nor paints by contracting paint work to a shop with a paint booth.  On the basis of these few exploratory contacts, it appears that emissions for facilities that are exempt from reporting their use of surface coatings and solvents are relatively small and negligible.

Table 1.   Top consumers of industrial coatings, solvents, and adhesives within

the CCOS II participating counties (US Census Bureau, 2001a).

|  | Resp. | No. | No. |  | Product Category |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAICS | Agy. | Emp. | Est. | Manufacturing Industry | 1 | 2 | 3 | 4 | 5 | 6 |
| 322222 | ARB | 831 | 5 | Coated and laminated paper manufacturing |  | x |  |  | x |  |
| 323110 | Dist. | 916 | 139 | Commercial lithographic printing |  | x |  |  | x |  |
| 323111 | Dist. | 258 | 3 | Commercial gravure printing |  | x |  |  | x |  |
| 323112 | Dist. | 174 | 7 | Commercial flexographic printing |  | x |  |  | x |  |
| 323113 | Dist. | 66 | 30 | Commercial screen printing |  | x |  |  | x |  |
| 325211 | Dist. | 125 | 7 | Plastics material and resin manufacturing |  |  |  | x |  |  |
| 326130 | ? | 92 | 2 | Laminated plastics plate, sheet and shape manufacturing |  |  |  | x |  | x |
| 332312 | ARB | 84 | 27 | Fabricated structural metal manufacturing |  |  | x |  |  |  |
| 332322 | ARB | 27 | 45 | Sheet metal work manufacturing |  |  | x |  |  |  |
| 332431 | ARB | 246 | 6 | Metal can manufacturing | x |  | x |  | x |  |
| 332439 | ARB | 889 | 5 | Other metal container manufacturing | x |  | x |  | x |  |
| 332812 | ARB | 328 | 9 | Metal coating, engraving and allied services | x |  | x |  | x |  |
| 334417 | ? | 758 | 3 | Electronic connector manufacturing |  |  |  | x |  |  |
| 334613 | ? | 512 | 6 | Magnetic and optical recording media manufacturing |  |  |  | x |  |  |
| 336211 | ? | 66 | 11 | Motor vehicle body manufacturing (not refinishing) |  |  | x |  |  |  |
| 336213 | ? | 27 | 2 | Motor home manufacturing (not refinishing) |  |  | x | x |  |  |
| 337110 | ARB | 2276 | 121 | Wood kitchen cabinet and countertop manufacturing (ARB) | x |  |  |  |  |  |
| 337122 | ARB | 27 | 29 | Nonupholstered wood household furniture manufacturing (ARB) | x |  | x |  |  |  |

Resp. Agy. = responsible agency; No. Emp. = number of employees; No. Est. = number of establishments; Dist. = district; ? = undefined

Product categories:

1 Adhesives and sealants	4 Plastics resins

2 Glues and adhesives	5 Printing inks

3 Paints, varnishes, lacquers, stains, shellacs, japans, enamels, and allied products	6 Plasticizers

| Table 2. Business listings for northern California for businesses that use miscellaneous and unspecified industrial coatings and solvents. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Business Name | Telephone | Address | City | Zip |
| 326130 Laminated Plastics Plate, Sheet and Shape Manufacturing US Census Bureau (2001a) indicates facilities are located in Placer and Shasta Counties. |  |  |  |  |
| Farallon Plastics Ltd | 916-371-3162 | 1941 Walters Ct | Fairfield | 94533 |
| Macro Plastics Inc | 707-437-1200 | 2250 Huntington Dr | Fairfield | 94533 |
| Wrights Plastics | 530-743-3476 | 7479 Doc Adams Rd | Marysville | 95901 |
| Spence Danl D Laminated Plastics Res | 916-988-3536 | 8957 Eden Oaks Ave | Orangevale | 95662 |
| Dependable Plastics and Pattern | 707-863-4900 | 4900 Fulton Dr | Suisun City | 94585 |
| Bay Valley Plastics | 707-451-4138 | 440 Mcclellan St | Vacaville | 95688 |
| Master Plastics Inc | 707-451-3168 | 820 Eubanks Dr | Vacaville | 95688 |
| U S A Scientific Plastics | 916-666-7186 | 1242 Commerce Ave | Woodland | 95776 |
| 334417 Electronic Connector Manufacturing US Census Bureau (2001a) indicates facilities are located in Butte, El Dorado and Yolo Counties. |  |  |  |  |
| Fisher Connectors | 916-922-3823 | 4208 N Freeway Blvd | Sacramento | 95834 |
| Electric Connection | 916-725-4422 | 9447 Greenback Ln # 607 | Orangevale | 95662 |
| Amp Inc | 530-274-8597 | 10926 Bonanza Way | Rough And Ready | 95975 |
| 334613 Magnetic and Optical Recording Media Manufacturing US Census Bureau (2001a) indicates facilities are located in Mendocino, Placer, Sacramento and Yolo Counties. |  |  |  |  |
| Seltek Co | 707-746-6399 | 110 E D St | Benicia | 94510 |
| Bunting Magnetics Co | 916-424-5164 | 6355 Riverside Blvd | Sacramento | 95831 |
| Inland Business Machines | 916-448-3221 | 1346w N Market Blvd | Sacramento | 95834 |
| 336211 Motor Vehicle Body Manufacturing US Census Bureau (2001a) indicates facilities are located in Nevada, Placer, Sacramento, Solano, Sutter and Yolo Counties |  |  |  |  |
| Tecon Enterprises | 530-621-0571 | 3581 China Garden Rd | Diamond Springs | 95619 |
| Fair Oaks Auto and Truck Center Repair | 916-966-3131 | 4115 Crestline Ave | Fair Oaks | 95628 |
| Bus Manufacturing USA Inc | 916-925-6680 | 5926 Patrol Rd | McClellan |  |
| Pacific Cylinders Inc | 916-621-0111 | 3581 China Garden Rd | Placerville | 95667 |
| DO Fabrication | 916-689-7836 | 8550 Tiogawoods Dr | Sacramento |  |
| River City Truck Equipment Inc | 916-921-2639 | 631 N Market Blvd # M | Sacramento |  |
| Rosies Hitch Systems Inc | 916-349-8510 | 5770 Roseville Rd | Sacramento |  |
| Sacramento Towing Equipment | 916-395-8641 | 1415 47th Ave # A | Sacramento |  |
| T & N Mfg & Sales | 916-423-2290 | 8550 Tiogawoods Dr | Sacramento |  |
| Industrial Friction Supply Co | 916-443-6676 | 255 Dos Rios St | Sacramento | 95814 |
| Raley Auto and Truck Center | 916-925-5411 | 4350 Raley Bl | Sacramento | 95838 |
| A-Z Bus Sales Inc | 916-372-8710 | 3117 Coke St | West Sacramento | 95691 |
| Port Truck Supply Inc | 916-372-5130 | 3939 W Capitol Ave | West Sacramento | 95691 |
| 336213 Motor Home Manufacturing US Census Bureau (2001a) indicates facilities are located in Butte and Mendocino Counties. no listings found |  |  |  |  |
| 336612 Boat manufacturing US Census Bureau (2001a) indicates facilities are located in Butte, Mendocino, Nevada, Sacramento, Shasta, and Solano Counties. |  |  |  |  |
| American River Mfg | 916-638-0900 | 3265 Monier Cir | Rancho Cordova | 95742 |
| Ark Works | 916-893-2171 | 11088 Midway | Chico | 95928 |
| Betts James Enterpr | 916-582-9588 | 1043 River Park Pl | Truckee | 96160 |
| Brendel Racing | 916-887-9560 | 12122 Dry Creek Rd | Auburn | 95602 |
| Classic Craft Boats | 916-366-1220 | 10275 Old Placerville Rd | Sacramento | 95827 |
| Design Concepts Boat Dlrs | 916-343-2232 | 265 Boeing Ave | Chico | 95973 |
| Feather River Boat Co | 916-671-2600 | 727 Sutter St | Yuba City | 95991 |
| Keaton Boats | 916-383-2150 | 8509 Folsom Blvd | Sacramento | 95826 |
| Lawman Boat Co | 916-674-3453 | 1570 Starr Dr | Yuba City | 95993 |
| Lightning Performance Boats | 916-387-0651 | 8838 Elder Creek Rd | Sacramento | 95828 |
| Modern Marine Composites | 916-638-7824 | 3295 Monier Cir | Rancho Cordova | 95742 |
| Pacific Boats | 916-241-1846 | 4311 Caterpillar Rd | Redding | 96003 |
| Sealight Boats | 916-795-4816 | PO Box 962 | Winters | 95694 |
| Sestern Boats | 916-243-4777 | 1054 Twin View Blvd | Redding | 96003 |
| Sheffields Boat Works | 916-371-3824 | 526 Galveston St | West Sacramento | 95691 |
| Sport Auto Marine | 916-685-5017 | 9730 Tralee Wy | Elk Grove | 95624 |
| Sub Sea Systems Inc | 916-386-7550 | 8830 Fruitridge Rd | Sacramento | 95826 |
| Tsunami Ocean Kayak Design | 916-275-4313 | 13732 Bear Mountain Rd | Redding | 96003 |
| Downunder Boat Works | 707-279-2628 | 3590bigvalleyrd | Kelseyville | 95451 |

## Printing

Printing processes include web offset lithography, web letterpress, rotogravure, flexography, screen printing, manual-fed printing, and sheet-fed printing.  The preferred techniques for estimating emissions from printing require estimates of the quantities of inks used by ink type and VOC content.  These data are not available, except through a direct poll of commercial printers.  Therefore, we consulted a business directory to locate northern California printers of the following types (listings are attached).

NAICS	Manufacturing Industry

323110	Commercial lithographic printing

323111	Commercial gravure printing

323112	Commercial flexographic printing

323113	Commercial screen printing

We suggest that in the future, counties should individually inventory the limited number of gravure and lithographic shops that were identified (Table 3).  We noted that the US Census Bureau (2001a) indicated numerous lithographic printing facilities in the CCOS II area (see Table 1).  However, we found that business listings tend to include far more screen printing establishments.  Among smaller facilities, we suspect a great deal of crossover between the lithographic, flexographic, and screen printing shops.  These small shops may be inventoried by extrapolating the results of a random survey or by applying a per capita emission factor that is offered for use by the EPA (1995a).  For the area surveyed, a sample of 30 to 40 randomly selected non-permitted commercial printers should be sufficient to assess emissions from these small operations.

The San Diego Air Pollution Control District (1989) noted that a “laborious survey of printing material suppliers yielded an emission estimate only 25% lower than simply multiplying the per capita emission factor times the county population.”  Therefore, because a survey is beyond the scope of this project and because the per capita factors have yielded comparable results to those of a survey of a survey in the past, we have employed top-down techniques to estimate emissions.

The Emission Inventory Improvement Program (EIIP, 1996) recommends the process-specific emission factors listed in Table 3.  These emission factors were matched with US Census Bureau (1999/2000) estimates for 1997 US consumptions of printing inks in order to estimate emissions as shown in Table 3.  It is assumed that 1997 consumption rates are roughly similar to 2000 consumption rates.  A weighted average emission factor was estimated for offset lithography by assuming that newspaper printing accounts for 86% of this type of printing and periodicals printing account for the remainder, which is distributed evenly between the other types of lithographic printing.  This distribution is proportional to the values of year-2000 shipments by the newspaper and periodical printing industries, $32 billion and $5 billion, respectively.

Table 3.   Emission factors for printing processes, US ink consumption, and

related emissions estimates for the year 2000.

| Printing Process | Emission Factor (lb VOC/lb ink) | US Ink Consumption (million lb) | Emissions (tons VOC) |
| --- | --- | --- | --- |
| Rotogravure | 0.73 | 329 | 120,085 |
| Flexography | 0.64 | 203 | 64,960 |
| Offset Lithography | 0.38a | 1,078 | 205,731 |
| Heatset | 1.25 |  |  |
| Nonheatset Web | 0.58 |  |  |
| Nonheatset Sheet | 3.37 |  |  |
| Newspaper | 0.16 |  |  |
| Letterpress | 0.31 | 100 | 15,500 |
| Screen | Unavailable | 26 | 4 to 88b |
| Other | Unavailable | 5.6 | 0.9 to 19b |
| Total |  | 1,742 | 406,332c |

a Weighted average value.

b Range calculated from min/max emission factors for other types of processes.

c Mid-range sum.

In 2000, 10.2% of total US employment for the printing industry (NAICS 32311) was located in California (US Census Bureau, 2002).  Therefore, California’s year-2000 VOC emissions are estimated as follows.

406,332  10.21% = 41,500 tons

California statewide emissions are disaggregated to counties according to employment in the printing industry (NAICS 32311).  It was verified through contacts with local county air districts that El Dorado, Placer, Sacramento, Solano, and Yolo counties require the use of printing inks with low VOC contents. (Very small facilities, which emit below the regulatory threshold, are exempt.)   Emissions reductions of 70% were assumed for these counties.  Emissions results are tabulated in Table 4.

Table 4.   Year-2000 emissions estimates for the printing industry.

| County | Proportion of Statewide Printing Industry Employment | Uncontrolled VOC Emissions (tons) | Controlled VOC Emissions (tons) |
| --- | --- | --- | --- |
| Amador | 0.013% | 5 | 5 |
| Butte | 0.231% | 96 | 96 |
| Calaveras | 0.041% | 17 | 17 |
| Colusa | 0.013% | 5 | 5 |
| El Dorado | 0.079% | 33 | 10 |
| Glenn | 0.079% | 33 | 33 |
| Mariposa | 0.000% | - | - |
| Mendocino | 0.079% | 33 | 33 |
| Nevada | 0.079% | 33 | 33 |
| Placer | 0.231% | 96 | 29 |
| Plumas | 0.000% | - | - |
| Sacramento | 3.100% | 1,287 | 386 |
| Shasta | 0.140% | 58 | 58 |
| Sierra | 0.000% | - | - |
| Solano | 0.176% | 73 | 22 |
| Sutter | 0.013% | 5 | 5 |
| Tehama | 0.013% | 5 | 5 |
| Tuolumne | 0.013% | 5 | 5 |
| Yolo | 0.496% | 206 | 62 |
| Yuba | 0.013% | 5 | 5 |

## Fiberglass Impregnation and Fabrication

Fiberglass is made of a fiber-reinforced matrix of plastics resin.  Other terms for fiberglass are “reinforced plastics” or “composites”.  Also included the range of composite products are synthetic marble products (such as counter tops or bowling balls), which have similar production techniques (except that no fiber reinforcements are used).  Production processes include those listed below.  Only 5% of fiberglass composites are produced via methods other than those listed below (CFA, 2002).

- Open mold (hand lay-up or spray lay-up):  An open mold is prepared with a non-stick material, a gel coat, and/or a small amount of plastic resin.  Fiber substrate is laid into the open resin-wetted mold.  Then, the fiber is saturated with plastic resin by hand or with a mechanical spray gun.  This process is repeated until the desired thickness is reached.  Hand lay-up is used for low-volume production, while spray lay-up is used for intermediate-volume production.  Approximately 47% of composites are produced via open-mold processes (CFA, 2002).

- Closed mold:  In this case, a closed mold is filled with plastics resin by injection, vacuum, or compression pressure.  These processes are used for high-performance products (such as aircraft parts) or high-volume production lines.  Approximately 21% of composites are produced via closed-mold processes (CFA, 2002).

- Filament wind:  Resin-coated fiber filaments are gradually wound around a form.  Filament winding results in a product with a smooth, finished interior; high fiber density; and high tensile strength. Products include chemical and fuel storage tanks, pressure vessels, and rocket motor cases.  Approximately 21% of composites are produced via filament winding (CFA, 2002).

- Pultrusion: Continuous fiber is pulled through a resin bath and then consolidated in a heated die. Products include objects that have a uniform cross-section along their length, such as ladder rails, channels, and structural composite beams.  Approximately 5% of composites are produced via pultrusion (CFA, 2002).

The U.S. EPA and the CFA are currently working to develop emissions estimation guidance for the composite fabrication industry.  Their latest research is summarized by Nuñez et al. (1999) and Haberlein (1999 and 2001).  We used the CFA’s equations to estimate styrene emissions as follows.

For spray lay-up (open molding):  EF = (0. 714  %styrene) –18%

For filament winding:  EF = (0.2746  %styrene) – 19.5%

Where:	EF	= 	Styrene emission factor (percent by weight of composite production)

%styrene 	=	Styrene content of the plastic resin (percent by weight)

In addition, we applied the assumptions that

- The emission rate for closed-mold processes is 85% less than that for open-mold processes.  This assumption is based on the information from the Composite Manufacturers Association and the EPA that closed-mold processes offer much less opportunity for emissions from material surfaces than do open-mold processes and that once hardened, VOCs are trapped inside the finished fiberglass body.

- The emission factor for pultrusion is similar to that for filament winding.

- The emission factor for other processes is similar to that for open-mold processes.

- Emissions from gel coating are negligible when compared to other sources. This assumption is based on information that, as a surface coating, gelcoat material use (by mass) is almost negligible compared to material use of resins in fiberglass production.  However, its mass-based emission factor according to Harcourt is only about times that of resins.

Typical styrene contents were adopted from Nuñez et al (1999).  The resultant emission factors are listed in Table 5.

Table 5.   Styrene contents and estimated emission factors for fiberglass

composite fabrication processes.

| Process | Resin Styrene Content (% by weight) | Styrene Emission Factors |  |
| --- | --- | --- | --- |
|  |  | (% by weight of composite product) | (lb styrene per ton product) |
| Open mold | 43% | 12.7% | 254 |
| Closed mold | 35% | 1.9% | 38 |
| Filament wind | 40% | 8.0% | 160 |
| Pultrusion | 40% | 8.0% | 160 |
| Other | 40% | 12.7% | 254 |

According to the Composite Fabricators Association (CFA, 2002), U.S. production of composites reached 3.9 billion pounds of material in the year 2000.  U.S. production was consumed for the following industrial end uses (CFA, 2002):  automobile, truck, or recreational vehicle manufacturing (33%); construction materials, such as bathroom fixtures, countertops, or architectural components (19%); corrosion-resistent products, such as petroleum storage tanks (12%); boat building (11%); electrical products, such as circuit boards, insulators, switching equipment (10%); consumer products, such as sporting goods (6%); appliances (5%); and aircraft construction (1%).  All other end uses comprised only 3% of U.S. consumption.

Productions of fiberglass composites for the year 2000 in California and in the CCOSII Region was estimated to be 186,600 tons and 6,400 tons by apportioning U.S. end uses according to employment in selected industrial sectors as reported by the U.S. Census Bureau (2001a; 2002).  The apportionment factors are listed in Table 6.

Table 6.  Apportionment factors for fiberglass composite production rates.

| End Use | Industrial Sectors (NAICS) | California Factor (% of total US employment in sectors) | CCOSII Factor (% of total Calif. employment in sectors) |
| --- | --- | --- | --- |
| Auto, truck, and RV manufacturing | Motor vehicle mfg (3361); Motor vehicle body & trailer mfg (3362); Motor vehicle parts mfg (3363) | 4.3% | 2.7% |
| Marine boats | Boat building (336612) | 4.3% | 4.8% |
| Electrical products | Wiring device mfg (33593) | 6.5% | 4.1% |
| Corrosion-resistent products | All other plastics product mfg (326199) | 8.9% | 2.9% |
| Consumer products | Sporting & athletic goods mfg (33992) | 19.0% | 3.7% |
| Construction products (bathtubs, countertops, etc.) | Plastics plumbing fixture mfg (326191) | 22.3% | 3.7% |
| Appliances | Household appliance mfg (3352) | 2.2% | 0.4% |
| Aircraft | Aerospace product & parts mfg (3364) | 19.5% | 1.0% |
| Other | All other plastics product mfg (326199) | 8.9% | 2.9% |

The CCOSII production of composite materials was assigned proportionally to process types according to the distribution reported by the CFA (2002) (see Table 7).  These production levels were multiplied by the emission factors from Table 4 to estimate total styrene emissions for the CCOSII study region, 545 tons.  Emissions were then disaggregated to individual counties according to the total county-wide employment levels for the industrial sectors previously listed Table 5.  The Bay Area Air Quality Management District and the Sacramento Metropolitan Air Quality Management District regulate the production processes for fiberglass manufacturers by restricting the VOC content of resins used, requiring the use of a vapor suppressant, or requiring the employment of a closed mold process.  Judging from the emission factors presented by Haberlein (1999), either of the restrictions on open mold processes can be expected to result in emissions reductions 25% to 60%.  (We assumed a 40% reduction in emissions.)  Results are shown in Table 8.  An example calculation follows below.

US Production was reported to be 3.90 billion pounds for year 2000 (CFA, 2002).  The production for the industrial end use of automobile, truck, or recreational vehicle manufacturing is estimated according to the CFA’s reported distribution by end use.

3.90 billion pounds  2000 lb/ton  33% = 6,435 tons

The production in California for this end use is proportional to California’s employment relative to total US employment in the related industrial sectors:  motor vehicle manufacturing, motor vehicle body and trailer manufacturing, and motor vehicle parts manufacturing (NAIC codes 3361, 3362, and 3363).

643,500 tons  4.3% = 27,670 tons

Similarly, the production in the CCOSII region for this end use is proportional to CCOSII employment relative to total California employment in the related industrial sectors:  motor vehicle manufacturing, motor vehicle body and trailer manufacturing, and motor vehicle parts manufacturing (NAIC codes 3361, 3362, and 3363).

27,670 tons  2.7% = 747 tons

Similar calculations are carried out to estimate production rates for other end uses (construction materials, corrosion-resistant products, electrical products, consumer products, appliances, aircraft, and other).  The sums across industrial categories for California and CCOSII are 186,605 tons and 6,370 tons, respectively.

The quantity of fiberglass manufactured in the CCOS II Region is proportional to the distribution by process type reported by the CFA (2002).

6,370  47% = 2,994 tons

This production rate is multiplied by its estimated emission factor, taken from Table 5, to estimate emissions for the CCOS II Region.

2,994  254 lb/ton  2000 lb/ton = 380 tons styrene

Similar calculations are carried out to estimate production rates for other processes (closed mold, filament wind, pultrusion, and other).  The sum of emissions for all processes in the CCOSII Region is 545 tons styrene.

Emissions for Shasta County are proportional to the county’s total employment relative to CCOSII employment in the related industrial sectors: motor vehicle manufacturing, motor vehicle body and trailer manufacturing, and motor vehicle parts manufacturing (NAIC codes 3361, 3362, and 3363); boat building (336612); wiring device manufacturing (33593); all other plastics product manufacturing (326199); sporting and athletic goods manufacturing (33992); plastics plumbing fixture manufacturing (326191); household appliance manufacturing (3352); aerospace product and parts manufacturing (3364); all other plastics product manufacturing (326199).

545 tons  3.2% = 17 tons

Table 7.   Year-2000 production rates by process and total styrene emissions for fiberglass

composites manufacturing in the CCOSII Region.

| Process | Proportion of Total Production | CCOSII Production (tons) | Emission Factor (lb/ton) | CCOSII Emissions (tons) |
| --- | --- | --- | --- | --- |
| Open mold | 47% | 2,994 | 254 | 380 |
| Closed mold | 31% | 1,975 | 38 | 38 |
| Filament wind | 12% | 764 | 160 | 61 |
| Pultrusion | 5% | 318 | 160 | 25 |
| Other | 5% | 318 | 254 | 40 |
| Total | 100% | 6,369 |  | 545 |

Table 8.   Year-2000 emissions from fiberglass composites manufacturing

for the CCOSII counties.

| County | Emissions (tons) |
| --- | --- |
| Amador | 2 |
| Butte | 46 |
| Calaveras | 7 |
| Colusa | 0.8 |
| El Dorado | 29 |
| Glenn | 1.4 |
| Mariposa | 0.8 |
| Mendocino | 13 |
| Nevada | 20 |
| Placer | 66 |
| Plumas | 3 |
| Sacramento | 138 |
| Shasta | 17 |
| Sierra | 0.2 |
| Solano | 54 |
| Sutter | 7 |
| Tehama | 2 |
| Tuolumne | 10 |
| Yolo | 34 |
| Yuba | 2 |
| Total | 545 |

## Plastic and Rubber Manufacturing

Plastics and synthetic rubbers are made of polymers, which are formed from styrene, butadiene, propylene, vinyl chloride, or ethylene terephthalate.  EPA (1995b; 1995c) presents VOC emission factors for the plastics industry that range from 0.3 to 1.4 lb VOC per hundred pounds of produced plastics and rubber.  In plastics and rubber manufacturing industries, nearly all of the raw materials are converted to product.  US Census Bureau (1999/2000) estimates the quantities of raw materials consumed in the US by the plastics and synthetic rubber manufacturing industries (see Table 9).  A factor of 1% is applied to each of the VOC-rich raw materials to estimate total US emissions.

Table 9.   Annual 1997 material consumption by US plastics and synthetic rubber

manufacturers and estimated emissions.

|  | Material Consumption (million lbs) |  | VOC Emissions |
| --- | --- | --- | --- |
| Material | Plastics | Rubber | (tpy) |
| Styrene | 7,185 | 788 | 39,864 |
| Phenol | 1,608 | nd | 8,042 |
| Synthetic organic dyes, pigments, lakes, and toners | 9 | nd | nVOC |
| Acrylates and methacrylates, monomers | 2,405 | 116 | 12,606 |
| Alcohols, except ethyl | 1,946 | 8 | 9,772 |
| Formaldehyde | 1,366 | nd | 6,830 |
| Vinyl acetate, monomer | 1,067 | nd | 5,333 |
| Vinyl chloride | 6,822 | nd | 34,112 |
| Ethane | 4,926 | nd | 24,632 |
| Ethylene | 20,077 | 364 | 102,204 |
| Propane | 4,163 | nd | 20,813 |
| Propylene | 9,793 | 355 | 50,740 |
| Butadiene | 596 | 3,128 | 18,621 |
| Other refined petroleum (C4+) | 2,058 | 671 | 13,641 |
| Plastics resins | 3,102 | 228 | nVOC |

nd = not disclosed; nVOC = VOC content low or unknown

According to the US Census Bureau (2001a), California’s only synthetic rubber manufacturing plants are located in southern California and San Francisco.  Emissions for the plastics industry are allocated to California according to California’s share (2.3%) of the total US plastics manufacturing market.  Emissions are distributed to Mendocino, Sacramento, Shasta, and Solano Counties according to employment in the industry (Table 10).  In addition, emissions were estimated for all facilities and for small facilities (<20 employees) according to numbers of employees and establishments per county.

Table 10.  Estimated annual VOC emissions for plastics manufacturing, 1997.

| County | Proportion of State Total | All Facilities VOC Emissions (tons/year) | Small Facilities VOC Emissions (tons/year) |
| --- | --- | --- | --- |
| Mendocino | 2.63% | 195 | 0 |
| Sacramento | 2.63% | 195 | 0 |
| Shasta | 0.61% | 45 | 45 |
| Solano | 2.56% | 190 | 62 |

According to the Society of the Plastics Industry (2000), California’s plastics manufacturing industry grew at an annual rate of 6% from 1996 through 1999 (or 20.5% over 3 years).  Thus, year-2000 emissions for small facilities in Shasta and Solano Counties are estimated to be 54 and 75 tons VOC per year, respectively.

In addition, we consulted a business directory for manufacturers of plastics and resins in northern California.  Only 3 facilities were identified, all of which were located in Sacramento.  We were unable to contact these facilities because none appeared to have valid telephone contact information.

## References

CFA (2002) “What are composites? (web site),” “Composites industry overview (presentation),” and “An introduction to the composites industry (brochure)”.  Accessible on-line from http://www.cfa-hq.org/composites.shtml.

Emission Inventory Improvement Program (1996) Technical report series, volume iiI  area sources. chapter 7  graphic arts.  Chapter prepared by Eastern Research Group for the Point Sources Committee of the Emission Inventory Improvement Program.  Published by the US EPA, RTP.

EPA (1999) Manufacture of rubber products.  Review draft chapter for compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Chapter 4.12.

EPA (1995a) General graphical printing.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), chapter 4.9.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

EPA (1995b) Plastics.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), chapter 6.6.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

EPA (1995b) Synthetic rubber.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), chapter 6.10.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

Haberlein, RA (2001) Emission factors for non-atomized application of gel coats used in the open molding of composites.  Report prepared by Engineering Environmental, Annapolis, Maryland, for the Composites Fabricators Association, Washington, D.C., July 17.

Haberlein, RA (1999) Technical discussion of the unified emission factors for open molding of composites.  Report prepared by Engineering Environmental, Annapolis, Maryland, for the Composites Fabricators Association, Washington, D.C. April 7.

Haberlein, RA (1999) Emission factors for liquid organic peroxide catalysts used in the open molding of composites.  Report prepared by Engineering Environmental, Annapolis, Maryland, for the Composites Fabricators Association, Washington, D.C. March 24.

Haberlein, RA (1999) Annotated CFA emission models for the reinforced plastics industries.  Report prepared by Engineering Environmental, Annapolis, Maryland, for the Composites Fabricators Association, Washington, D.C. (February 28, 1998) and annotated by the CFA (April 7, 1999).

Nuñez C.M., Geddes H.R., Bahner M.A., Clayton C.A. (1999) An empirical model to predict styrene emissions from fiber-reinforced plastics fabrication processes.  In Journal of the Air and Waste Management Association, Vol 49, pp 1168-1178, October.

San Diego Air Pollution Control District (1989) Area source methodology – printing.  Summary emissions methodology report prepared by Carl Selnick of the San Diego Air Pollution Control District for submittal to the ARB, September.

Society of the Plastics Industry (2000) Plastics in California.  In the SPI economic report, 2000.  Report prepared by the Society of the Plastics Industry, Washington, D.C.

US Census Bureau (2002) Censtats databases – county business patterns data.  Database maintained by the US Census Bureau, Washington, D.C., and accessed August 2002 at web site http://tier2.census.gov/dbappweb.htm.

US Census Bureau (2001a) County business patterns database, 1998-1999.  Database no. CD-CBP-98/99 published by the US Census Bureau, Washington, DC, May.

US Census Bureau (2001b) Manufacturing industry series of the 1997 economic census –. product summary.  Report no. EC97-M31S-PS prepared by the US Census Bureau, Washington, D.C, October.

US Census Bureau (1999/2000) Manufacturing industry series of the 1997 economic census – various manufacturing types.  Reports prepared by the US Census Bureau, Washington, D.C.  Accessed from the Internet, http://www.census.gov/prod/www/abs/97ecmani.html.

| Business listings for northern California printers. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Business Name | Telephone | Address | City | Zip |
| 323110 Commercial Lithographic Printing (Yolo, Sacramento, Butte) |  |  |  |  |
| Exclamation Inc | 707-747-2921 | 5000 E 2nd St | Benicia | 94510 |
| Mannmade | 916-583-3383 | PO Box 875 | Carnelian Bay | 96140 |
| Tess Gift-Homewares and Card Shop | 916-273-6997 | 145 Mill St | Grass Valley | 95945 |
| Scarlata Sam and Sierra | 209-742-6824 | 5397 Tip Top Rd | Mariposa | 95338 |
| Cardco Distributing | 916-392-9306 | 6898 Park Riviera Way | Sacramento | 95831 |
| 323111 Commercial Gravure printing (El Dorado, Mendocino, Tuolumne) |  |  |  |  |
| Pankey & Assoc | 916-428-6555 | 3526 La Grande Blvd | Sacramento |  |
| Zelinko Printing Company | 707-425-0361 | 1820 Walters Ct | Fairfield | 94533 |
| North Star Printing Company | 916-272-6636 | 12400 Loma Rica Dr | Grass Valley | 95945 |
| 323112 Commercial Flexographic Printing (El Dorado, Nevada, Placer, Sacramento) no listings |  |  |  |  |

| Business listings for northern California screen printing businesses. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Business Name | Telephone | Address | City | Zip |  |
| Free Time Sportsware | 916-378-1528 | 3351 Marmac Rd | Anderson | 96007 |  |
| Schwartz C M Printing & Graphics | 916-878-6760 | PO Box 208 | Applegate | 95703 |  |
| Baby Dear | 916-889-2341 | 425 Lincoln Way | Auburn | 95603 |  |
| Laughing Horse Design | 916-888-8200 | 12205 Locksley Ln | Auburn | 95602 |  |
| Gold Country Impressions | 916-888-0219 | 11735 Atwood Rd | Auburn | 95603 |  |
| G P Wholesale Printing | 916-887-8638 | 2975 Richardson Dr | Auburn | 95603 |  |
| Sun Sign Co | 916-885-8771 | 164 Teamtrack Rd | Auburn | 95603 |  |
| Compucheck Printing | 916-885-8615 | 2280 Grass Valley Hwy | Auburn | 95603 |  |
| American Speedy Printing Centers-Fax | 916-885-8007 | 11702 Enterprise Dr | Auburn | 95603 |  |
| Auburn Printers Fax Line | 916-885-6517 | 142 Hoffman Ave | Auburn | 95603 |  |
| Dancing Dog Productions | 916-885-3692 | 1466 Canal St | Auburn | 95603 |  |
| Baby Ts | 916-885-1508 | 1466 Canal St | Auburn | 95603 |  |
| Timberline Sign Center | 916-823-3388 | 140 Borland Ave | Auburn | 95603 |  |
| Cpc Press | 916-823-0289 | 9850 Hillview Rd | Auburn | 95602 |  |
| All Design Printing Co | 916-371-0391 | 313 D St | Broderick | 95605 |  |
| Galleria Graphic | 916-974-3939 | 4126 Manzanita Ave | Carmichael | 95608 |  |
| Print & Design Professionals | 916-973-0439 | PO Box 439 | Carmichael | 95609 |  |
| Graphic Graffitti | 916-972-0118 | 5934 Don Way | Carmichael | 95608 |  |
| Western Screenprinting | 916-966-6933 | 6630 Madison Ave | Carmichael | 95608 |  |
| Colors Printing-Graphics-Design | 916-944-7044 | 7743 Fair Oaks Blvd | Carmichael | 95608 |  |
| Nutter Printing | 916-944-1124 | 7637 Fair Oaks Blvd | Carmichael | 95608 |  |
| Knox David | 916-944-0790 | PO Box 628 | Carmichael | 95609 |  |
| Accent Bridal | 916-489-7913 | 4005 Manzanita Ave | Carmichael | 95608 |  |
| Terra Firma Company Usa | 916-489-3481 | Carmichael Professional Plaza | Carmichael | 95821 |  |
| Ink Spot Print Shop | 916-488-5656 | 6828 Fair Oaks Blvd | Carmichael | 95608 |  |
| Print Media Plus | 916-485-8895 | 4800 Manzanita Ave | Carmichael | 95608 |  |
| Alpha Commercial Printing | 916-482-9890 | 5109 Gibbons Dr | Carmichael | 95608 |  |
| Creative Printing | 916-332-1166 | 5232 Manzanita Ave | Carmichael | 95608 |  |
| Shirts Required | 916-343-8321 | 730 W-9th Chico | Chicago Park | 95712 |  |
| Signs-R-Us | 916-342-9257 | 21391/2 Esplanade | Chicago Park | 95712 |  |
| Softwear Printing | 916-894-8551 | 333 W 12th Ave | Chico | 95926 |  |
| Quadco Too | 916-894-7790 | 2535 Zanella Way | Chico | 95928 |  |
| Karma Kreations | 916-894-7117 | 140 Broadway St | Chico | 95928 |  |
| A B Dick Products Co Of Chico Service Dept | 916-893-8714 | 702 Nord Ave | Chico | 95926 |  |
| Paper & Ink | 916-893-2877 | 3023 Esplanade | Chico | 95973 |  |
| Media Screenprinting | 916-893-0545 | 320 Broadway St | Chico | 95928 |  |
| Chico Screenprint | 916-891-5029 | 4331 Hedstrom Way | Chico | 95973 |  |
| Preservation Press | 916-891-5020 | 848 W 9th St | Chico | 95928 |  |
| Sms Screen Printers | 916-345-6783 | 839 Main St | Chico | 95928 |  |
| Positive Print | 916-345-6168 | 1360 Mangrove Ave | Chico | 95926 |  |
| West Coast Lettering & Athletic Inc | 916-345-6088 | 337 Nord Ave | Chico | 95926 |  |
| Jacksons Printing | 916-342-7461 | 732 Cherry St | Chico | 95928 |  |
| Heidelberg Graphics | 916-342-6582 | 2 Stansbury Ct | Chico | 95928 |  |
| Western Printing | 916-969-4015 | 7520 Cedar Dr | Citrus Heights | 95610 |  |
| Lion Print | 916-969-0860 | 8071 Greenback Ln | Citrus Heights | 95610 |  |
| Video Pals | 916-729-3278 | 8300a Auburn Blvd | Citrus Heights | 95610 |  |
| A-1 Colors Plus Silk Screening And Embroidery | 916-726-4604 | 7600 Away Way | Citrus Heights | 95610 |  |
| Lion Print Fax | 916-725-2027 | 8071 Greenback Ln | Citrus Heights | 95610 |  |
| J D Printing | 916-723-8193 | 8108 Garry Oak Dr | Citrus Heights | 95610 |  |
| B S Printing | 916-721-5557 | 7637 Auburn Blvd | Citrus Heights | 95610 |  |
| About Printing | 916-338-2800 | 5990 Devecchi Ave | Citrus Heights | 95621 |  |
| Foothill Printers | 916-675-0703 | 12140 La Porte Rd | Clipper Mills | 95930 |  |
| Holcroft Letterpress | 209-683-3645 | PO Box 1137 | Coarsegold | 93614 |  |
| Gkm Corp | 916-823-7070 | 896 S Canyon Wy | Colfax | 95713 |  |
| Halton Joseph Printing | 916-346-2526 | 14 N Main St | Colfax | 95713 |  |
| Johnson Printing & Design | 916-458-7701 | 631 Market St | Colusa | 95932 |  |
| Olive Knot Products | 916-824-5280 | 4750 Hall Rd | Corning | 96021 |  |
| Dons Barber & Hobby Shop | 916-824-4836 | 1214 Solano St | Corning | 96021 |  |
| California Art And Printing Co | 916-758-0430 | 425 L St | Davis | 95616 |  |
| Recycle Records | 916-756-4943 | 205 F St | Davis | 95616 |  |
| Screaming Squeegee Screen Printing | 916-756-1477 | 17 Arboretum Dr | Davis | 95616 |  |
| Davis Sport Shop | 916-753-2572 | 206 F St | Davis | 95616 |  |
| Design-Yr-Sweats | 916-753-2548 | 231 G St | Davis | 95616 |  |
| Eagle Press | 916-622-6259 | 333 Industrial Dr 2 2 | Diamond Springs | 95619 |  |
| California Custom Tees | 916-621-4623 | 694 Pleasant Valley Rd | Diamond Springs | 95619 |  |
| Dixon Printers See Printing Shop The | 916-678-2383 | 125 E A St | Dixon | 95620 |  |
| Native Arts Gallery | 916-235-0882 | 5855 Dunsmuir Ave | Dunsmuir | 96025 |  |
| Native Ltd | 916-235-0778 | 5859 Sacramento Ave | Dunsmuir | 96025 |  |
| Shasta Design | 916-235-0450 | 5200 Florence Loop | Dunsmuir | 96025 |  |
| Argonne Printing Co | 209-264-2392 | 5624 S Elm Ave | Easton | 93706 |  |
| Impact Posters | 916-939-9333 | 4961 Windplay Dr | El Dorado Hills | 95762 |  |
| Lawson & Mardon Postcard | 916-939-4006 | 1084 Uplands Dr | El Dorado Hills | 95762 |  |
| Press Connection The Ofc | 916-933-6042 | 1746 Canberra Pl # 1 | El Dorado Hills | 95762 |  |
| Impact Images | 916-933-4700 | 4961 Windplay Dr | El Dorado Hills | 95762 |  |
| Flynn & Flynn | 916-933-0887 | 2001 Vista Mar Dr | El Dorado Hills | 95762 |  |
| Graphic Solutions | 916-686-4415 | 9750 Dino Dr | Elk Grove | 95624 |  |
| Flash Printing-Flash Ads | 916-685-8380 | 9255 Survey Rd Unit 11 | Elk Grove | 95624 |  |
| Fair Oaks Graphics | 916-967-6644 | 10422 Fair Oaks Blvd | Fair Oaks | 95628 |  |
| Mather Card & Copy | 916-966-0900 | 5229 Hazel Ave | Fair Oaks | 95628 |  |
| Storybook Weddings | 916-965-5566 | PO Box 542 | Fair Oaks | 95628 |  |
| Minuteman Press | 916-962-3358 | 8116 Madison Ave | Fair Oaks | 95628 |  |
| Clark & Christopher Apparel | 916-961-3626 | 7940 California Ave | Fair Oaks | 95628 |  |
| Fall River Arts | 916-336-6524 | 43514 Hwy 299 E | Fall River Mills | 96028 |  |
| Craftech | 916-983-7768 | 177 Bittercreek Dr | Folsom | 95630 |  |
| Western Advertising Specialities | 916-929-1817 | 1125 E Hillsdale Blvd # Bl | Foster City | 94404 |  |
| Eye-Method Publishing Co | 916-687-6611 | 10310 Tavernor Rd | Galt | 95624 |  |
| Argo Mountain Express | 916-333-4420 | 6101 Front | Georgetown | 95634 |  |
| Ink Mine The | 916-791-2886 | 4060 Douglas Blvd | Granite Bay | 95746 |  |
| Arrow Mountain Silkscreen Printing | 916-273-9029 | 110 Spring Hill Dr | Grass Valley | 95945 |  |
| Mcdowell Printing | 916-846-5067 | 837 Hazel St | Gridley | 95948 |  |
| Shirt Shop The | 209-223-3884 | 577 S State Highway 49 | Jackson | 95642 |  |
| Tahoe Commercial Press | 916-546-3111 | 8353 Cutthroat Ave | Kings Beach | 96143 |  |
| Cool World Printing & Publishing | 916-743-8656 | 5174 Lindhurst Ave | Marysville | 95901 |  |
| Bluett Printing Co | 916-743-7562 | 1031 Ramirez St | Marysville | 95901 |  |
| Twin Cities Printing Co | 916-742-8865 | 5995 Lindhurst Ave | Marysville | 95901 |  |
| Custom Designs | 916-926-6667 | 1008 Ream Ave | Mount Shasta | 96067 |  |
| Encore Press | 916-645-8076 | 630 Fowler Rd | Newcastle | 95658 |  |
| Genes Pre Press | 916-483-0938 | 3487 Orange Grove Ave | North Highlands | 95660 |  |
| Transnational Printing Services Inc | 916-482-5888 | 3335 Longview Dr | North Highlands | 95660 |  |
| Arcade Printing | 916-344-1302 | 4708 Roseville Rd | North Highlands | 95660 |  |
| American Wholesale Thermographers | 916-344-1298 | 4704 Roseville Rd | North Highlands | 95660 |  |
| Creative Image | 916-332-1005 | 6525-32d | North Highlands | 95652 |  |
| Copyright | 209-847-7558 | 335 N-3d Ave | Oakdale | 95361 |  |
| Lindsey Printing | 209-847-1298 | 137 N-1st Ave | Oakdale | 95361 |  |
| Silkscreen Impressions | 209-683-7446 | 40359 Hwy 41 Ste 10 | Oakhurst | 93644 |  |
| Firesign | 209-683-4944 | 49774 Road 426 | Oakhurst | 93644 |  |
| Suncrest Printing & Bindery | 916-988-3264 | 6216 Main Ave | Orangevale | 95662 |  |
| Screen Express | 916-987-7111 | 6236 Main Ave | Orangevale | 95662 |  |
| Precision Printing | 916-965-3277 | 8709 Baxter Way | Orangevale | 95662 |  |
| Petrarch Press The | 916-692-0828 | PO Box 488 | Oregon House | 95962 |  |
| Oscars Signs & Sportswear | 916-865-5851 | 730-4th Orld | Orland | 95963 |  |
| Kits Screen Pinting | 916-533-9095 | 2055 Bird St | Oroville | 95965 |  |
| Comers Print Shop | 916-533-5717 | 2008 Lincoln St | Oroville | 95966 |  |
| Ourtlaw Grphics | 916-532-9647 | 2598 5th Ave | Oroville | 95965 |  |
| Palo Cedro Printing | 916-547-5458 | Deschutes Rd & Old 44 Dr | Palo Cedro | 96073 |  |
| Chaney Signs | 916-877-7273 | 5991 Clark Rd | Paradise | 95969 |  |
| Cordells Quick Printing | 916-877-0593 | 1132 Elliott Rd | Paradise | 95969 |  |
| Jinguji Graphics | 916-663-9401 | 2210 Taylor Rd | Penryn | 95663 |  |
| Crystal Graphics | 916-642-9580 | 175 Placerville Dr | Placerville | 95667 |  |
| Flying Ace T-Shirts | 916-622-8656 | 3871 Missouri Flat Rd | Placerville | 95667 |  |
| Line & Letter Shop | 916-622-2253 | 254 Main St | Placerville | 95667 |  |
| Whos Who Silk Screen & Graphics | 916-622-1714 | 5694 Mother Lode Dr | Placerville | 95667 |  |
| Pine Tree Printing | 916-832-5911 | 81 E Sierra St | Portola | 96122 |  |
| Signtech | 916-853-2200 | 2301 Vehicle Dr | Rancho Cordova | 95670 |  |
| Owens Printing Co Inc | 916-851-9700 | 3358 Monier Cir | Rancho Cordova | 95742 |  |
| A Super Flyer & Resumes | 916-851-3651 | 11082 Coloma Rd | Rancho Cordova | 95670 |  |
| D & K Specialtees | 916-851-0282 | 11440 Sunrise Gold Cir | Rancho Cordova | 95742 |  |
| Press The | 916-638-7737 | 2951 Sunrise Blvd | Rancho Cordova | 95742 |  |
| All Pro Athletic | 916-638-5068 | 11241 Coloma Rd | Rancho Cordova | 95670 |  |
| Darkroom Productions | 916-638-4260 | 11265 Sunrise Gold Cir | Rancho Cordova | 95742 |  |
| Express Printing Center | 916-638-0322 | 11255 Sunrise Gold Cir | Rancho Cordova | 95742 |  |
| American River Press | 916-635-9524 | 2583 Mercantile Dr | Rancho Cordova | 95742 |  |
| Image Industries International | 916-635-7820 | 2704 Mercantile Dr | Rancho Cordova | 95742 |  |
| Colour Press | 916-635-1748 | 11368 Monier Park Pl | Rancho Cordova | 95742 |  |
| Jp Printing | 916-631-8604 | 3333 Sunrise Blvd | Rancho Cordova | 95742 |  |
| A-1 Embroidery & Screen Printing | 916-631-8470 | 11327 Trade Center Dr | Rancho Cordova | 95742 |  |
| Alpha Graphics | 916-484-1000 | 11015 Olson Dr | Rancho Cordova | 95670 |  |
| T-Shirt Productions | 916-368-1177 | 10089 Folsom Blvd | Rancho Cordova | 95670 |  |
| Kims Hallmark Wedding Boutique | 916-366-7212 | 10361 Folsom Blvd | Rancho Cordova | 95670 |  |
| Kims Hallmark | 916-366-6511 | 10361 Folsom Blvd | Rancho Cordova | 95670 |  |
| Aaa Garments & Lettering | 916-363-4590 | 10119 Folsom Bl | Rancho Cordova | 95827 |  |
| Ts Custom Shirts-N-Etc | 916-529-0498 | 835 Lassen Ave | Red Bluff | 96080 |  |
| Ct Publishing & Copy Center | 916-378-0511 | 3711 Meadow View Dr | Redding | 96002 |  |
| Lyro Printing | 916-365-2788 | 3707 Meadow View Dr | Redding | 96002 |  |
| Bones Motor Sport Deisng | 916-246-8897 | 1450 Branstetter Ln | Redding | 96001 |  |
| Dare To Wear Inc From Redding Telephones Call | 916-246-5678 | 8865 Airport Rd | Redding | 96002 |  |
| Walker Lithograph Co | 916-244-6036 | 1135 Pine St | Redding | 96001 |  |
| Look Images | 916-244-4286 | 4770 Caterpillar Rd | Redding | 96003 |  |
| Grecos | 916-244-3551 | 7013 Danyeur Rd | Redding | 96001 |  |
| Print Boy | 916-244-2019 | 2708 S Market St | Redding | 96001 |  |
| Ball Boy | 916-244-2016 | 2655 Park Marina Dr | Redding | 96001 |  |
| Cal-State Design | 916-241-1112 | 12059 Randolph Rd | Redding | 96003 |  |
| Bobbies Poster Graphics | 916-224-1276 | 1438 Hartnell Ave | Redding | 96002 |  |
| Press Room Printing | 916-223-0664 | 1455 South St | Redding | 96001 |  |
| Signs Plus | 916-222-2708 | 1244 Hartnell Ave | Redding | 96002 |  |
| Shasta Silly Shirt & Silkscreen | 916-222-0202 | 3073 Bechelli Ln | Redding | 96002 |  |
| Total Printing | 916-221-7249 | 2339 Old Alturas Rd | Redding | 96003 |  |
| Applejacks | 916-221-1707 | 3336 Bechelli Ln | Redding | 96002 |  |
| Unique Designs X 2 | 916-221-1375 | PO Box 492284 | Redding | 96049 |  |
| D C-Mags | 916-677-6758 | PO Box 160 | Rescue | 95672 |  |
| J & M Printing | 916-652-4600 | 4321 Anthony Ct | Rocklin | 95677 |  |
| Distributor Printing | 916-632-7515 | 2224 Sierra Meadows Dr | Rocklin | 95677 |  |
| Action Laser Charge | 916-971-3333 | 658 Commerce Dr Ste A | Roseville | 95678 |  |
| Paul Baker Printing Inc | 916-969-8317 | 220 Riverside Ave | Roseville | 95678 |  |
| T L C Printing | 916-961-1400 | 105 Ascot Dr | Roseville | 95661 |  |
| Monas Art Cards Gifts & Weddings | 916-783-9093 | 1079 Sunrise Ave Ste G | Roseville | 95661 |  |
| Paul Baker Printing Inc | 916-783-8317 | 220 Riverside Ave | Roseville | 95678 |  |
| S C S Enterprises | 916-773-5057 | 209 Harding Blvd | Roseville | 95678 |  |
| Top Quality Productions | 916-773-2735 | 502 Giuseppe Ct | Roseville | 95678 |  |
| Pro-Tech Printing Services | 916-928-9415 | 1342 N Market Blvd # W | Sacramento | 95834 |  |
| Paper-Pros Wholesale Printing Inc | 916-928-1140 | 1342 N Market Blvd | Sacramento | 95834 |  |
| Printer On Retainer | 916-928-0507 | 1435 N Market Blvd # W | Sacramento | 95834 |  |
| Private Printing | 916-928-0206 | 1435 N Market Blvd # W | Sacramento | 95834 |  |
| Creative Ts N Things | 916-927-8858 | 1500 El Camino Ave | Sacramento | 95815 |  |
| Printery The | 916-927-4483 | 2378 Winnipeg St | Sacramento | 95815 |  |
| Designers Press | 916-927-2506 | 201 Lathrop Way | Sacramento | 95815 |  |
| Mansion Printers | 916-925-8010 | 1805 Tribute Rd | Sacramento | 95815 |  |
| Acme Printing Co | 916-925-7447 | 2003 El Camino Ave | Sacramento | 95821 |  |
| Precision Printing | 916-925-6600 | 1525 El Camino Ave | Sacramento | 95815 |  |
| Modern Impressions Printing | 916-925-5582 | 2224 Beaumont St | Sacramento | 95815 |  |
| Complete Printing Services | 916-925-4412 | 4247 S Market Ct | Sacramento | 95834 |  |
| Ellis & Ellis Signs-Displays | 916-924-1936 | 1111 Joellis Way | Sacramento | 95815 |  |
| Mediarite Press | 916-923-5542 | PO Box 191832 | Sacramento | 95819 |  |
| Forest Trade Printing Company | 916-923-2111 | 2541 Del Paso Bl | Sacramento | 95815 |  |
| Souza Printing | 916-923-1720 | 2452 Del Paso Blvd | Sacramento | 95815 |  |
| Bid-Rite Printing Service Inc | 916-922-3990 | 2003 El Gamino Av | Sacramento | 95815 |  |
| Libby & Co Inc | 916-922-2880 | 96 Arden Way | Sacramento | 95815 |  |
| Designer Signs By Len Austin Enterprises | 916-922-1234 | 2450 Del Paso Blvd | Sacramento | 95815 |  |
| B & L Printing Inc | 916-922-0294 | 1760 Challenge Way | Sacramento | 95815 |  |
| Impressions West | 916-921-6136 | 1241 Blumenfeld Dr | Sacramento | 95815 |  |
| By Invitation Only | 916-920-3324 | 2105 Marconi Ave | Sacramento | 95821 |  |
| Preston Tarver Creative Signs | 916-920-2881 | 1717 Kathleen Ave | Sacramento | 95815 |  |
| Printing Plus | 916-920-2322 | 4247 S Market Ct | Sacramento | 95834 |  |
| Form Source | 916-920-1491 | 2105 Marconi Ave | Sacramento | 95821 |  |
| Two Rivers Printing | 916-920-1322 | 1828 Tribute Rd | Sacramento | 95815 |  |
| Bindery Resources | 916-739-1500 | 8155 Belvedere Ave | Sacramento | 95826 |  |
| Pacific Edits Plus | 916-737-2430 | 2827-43d | Sacramento | 95815 |  |
| Dan-D-Signs | 916-689-2202 | 7205 Willowriver Ct | Sacramento | 95828 |  |
| Express Wholesale Printing | 916-646-9709 | 4590 Pell Dr | Sacramento | 95838 |  |
| Acme Signs | 916-646-3636 | 2003 El Camino Ave | Sacramento | 95821 |  |
| Faria Printing & Graphics | 916-638-3231 | 1820 Tribute Rd | Sacramento | 95815 |  |
| Faria Printing & Graphics | 916-638-2308 | 1820 Tribute Rd | Sacramento | 95815 |  |
| Ink Well The | 916-568-0420 | 1500 El Camino Ave | Sacramento | 95815 |  |
| Ernie The Printer | 916-489-9058 | 3525 Watt Ave | Sacramento | 95821 |  |
| Roundhouse Press | 916-488-8305 | 4339 Auburn Blvd | Sacramento | 95841 |  |
| Image Print & Graphics | 916-488-4717 | 4670 Aldona Ln | Sacramento | 95841 |  |
| Point West Print Services | 916-487-8054 | 2100 Watt Ave | Sacramento | 95825 |  |
| Robertas Printing-Copying Fax | 916-486-2641 | 2546 Cottage Way | Sacramento | 95825 |  |
| Wintune Graphics Inc | 916-485-7500 | 4600 Pell Dr | Sacramento | 95838 |  |
| Capitol Foil & Embossing | 916-485-5828 | 2734 Auburn Blvd | Sacramento | 95821 |  |
| Rasmussens | 916-485-5075 | 2104 Porter Way | Sacramento | 95825 |  |
| Mcglone Copy Shop | 916-484-7009 | 3061 Fulton Ave | Sacramento | 95821 |  |
| Advance Printing & Signs | 916-484-0772 | 3510 Auburn Blvd | Sacramento | 95821 |  |
| Maranatha Printing | 916-483-3526 | 2550 Valley Rd | Sacramento | 95821 |  |
| American Mail-Well Envelope | 916-483-1712 | 3815 Marconi Ave | Sacramento | 95821 |  |
| Party Creators | 916-482-2288 | 4543 Orange Grove Ave | Sacramento | 95841 |  |
| Quality Color Express | 916-457-5350 | 1409-32d | Sacramento | 95815 |  |
| Lincks Sign-Art | 916-456-5550 | 912 56th St | Sacramento | 95819 |  |
| Js Fine Quality Printing | 916-456-5319 | 8179 Belvedere Ave | Sacramento | 95826 |  |
| Print Shop The | 916-455-6834 | 5010 Freeport Blvd | Sacramento | 95822 |  |
| Sfl Graphics And Printing Services | 916-455-5890 | 1749-36th | Sacramento | 95817 |  |
| Jacks T Shirts | 916-455-2897 | 5524 Elvas Ave | Sacramento | 95819 |  |
| Jam Graphics & Printing Services | 916-455-0342 | 2425-24th St | Sacramento | 95817 |  |
| Endless Rossibili-Tees | 916-454-4967 | 8220 Belvedere Ave | Sacramento | 95826 |  |
| Chiropractic Printing Service | 916-454-3061 | 3453 Ramona Ave | Sacramento | 95826 |  |
| Amos Gluing | 916-454-2794 | 8167 Belvedere Ave | Sacramento | 95826 |  |
| Sun Printing | 916-454-1818 | 5248 Fruitridge Rd | Sacramento | 95820 |  |
| Compubraille Inc | 916-452-6189 | 2791-24th | Sacramento | 95817 |  |
| Deason & Stirn Prntng | 916-452-4548 | 2810 O St | Sacramento | 95816 |  |
| Enterprise Printing | 916-452-4113 | 4255 Power Inn Rd | Sacramento | 95826 |  |
| Prassa J Printers | 916-452-0736 | 3655 J St | Sacramento | 95816 |  |
| Heaven High Enterprises | 916-451-9634 | 3400-3d Ave | Sacramento | 95817 |  |
| Kellys Bindery | 916-451-7725 | 3045-65th | Sacramento | 95817 |  |
| Pullmann Printing | 916-451-4539 | 2104 Alhambra Blvd | Sacramento | 95817 |  |
| Davenport Graphics | 916-451-2107 | 2413 27th St | Sacramento | 95818 |  |
| Anchor Group The | 916-448-6444 | 6012 N-10th | Sacramento | 95814 |  |
| Imperial Die Cutting | 916-448-5066 | 400 16th St | Sacramento | 95814 |  |
| Art Sign Company The | 916-447-6776 | 600 Sunbeam Ave | Sacramento | 95814 |  |
| Old Sacramento Merchants Great Movie And Anima | 916-447-3623 | 1023 Front St | Sacramento | 95814 |  |
| Crystal Printing | 916-447-1908 | 408 12th St | Sacramento | 95814 |  |
| Business Graphics | 916-447-0929 | 2220 K St | Sacramento | 95816 |  |
| Art Equals Motion | 916-446-8899 | 1400 E St | Sacramento | 95814 |  |
| Time Printing Inc | 916-446-6152 | 1614 D St | Sacramento | 95814 |  |
| Sierra Pacific Lithographics | 916-446-5200 | 2415 C St | Sacramento | 95816 |  |
| Alpha Graphics Printshops Of The Future | 916-446-0167 | 501 J St | Sacramento | 95814 |  |
| Future Film Inc | 916-444-9577 | 2311 C St | Sacramento | 95816 |  |
| Central Valley Press | 916-444-9383 | 1714 X St | Sacramento | 95818 |  |
| Toms Printing Inc | 916-444-7788 | 1819 E St | Sacramento | 95814 |  |
| Signature Press | 916-444-7154 | 430-17th | Sacramento | 95814 |  |
| Metro Printing & Copy | 916-444-6482 | 2314 J St | Sacramento | 95816 |  |
| Butler Susan A Print & Design Management | 916-444-2655 | 813 F St | Sacramento | 95814 |  |
| Tan Ron C & Company | 916-444-1664 | 2102-19th | Sacramento | 95814 |  |
| Clark Printing Co | 916-443-7665 | 2120 P St | Sacramento | 95816 |  |
| Echo Shirts & Printing | 916-443-3246 | 2114-19th | Sacramento | 95814 |  |
| Ips Printers Typographers Accounting Dept | 916-442-8963 | 2020 K St | Sacramento | 95814 |  |
| Ips Printers Typographers Sales Dept | 916-442-8962 | 2020 K St | Sacramento | 95814 |  |
| Commerce Printing Service | 916-442-8100 | 322 12th St | Sacramento | 95814 |  |
| Sacramento Quick Print | 916-442-5615 | 2612 J St | Sacramento | 95816 |  |
| Heatwaves Custom Silkscreening | 916-442-5391 | 600 Sunbeam Ave | Sacramento | 95814 |  |
| Copeland Printing Services | 916-442-5339 | 2220 K St | Sacramento | 95816 |  |
| Allied Printing Co | 916-442-1373 | 1912 O St | Sacramento | 95814 |  |
| Seleeper Doug Co | 916-441-1405 | 2700 J St | Sacramento | 95816 |  |
| A & S Embossing Service | 916-428-9321 | 3281-35th Ave | Sacramento | 95823 |  |
| Graphics Diversified | 916-427-8030 | 176 Otto Cir | Sacramento | 95822 |  |
| City Press Company | 916-422-6959 | 4917-47th Ave | Sacramento | 95823 |  |
| Graphic Arts Center | 916-421-3460 | 8036 Maybelline Way | Sacramento | 95823 |  |
| Vietnam Printing | 916-395-3646 | 6019 Stockton Blvd | Sacramento | 95824 |  |
| Florin Mall T-Shirts Plus | 916-392-8589 | 6117 Florin Rd | Sacramento | 95823 |  |
| Alpha Printing | 916-391-9500 | 5880 Stockton Blvd | Sacramento | 95824 |  |
| Graffi-Tees Screen Printing | 916-391-9096 | 3446 La Grande Blvd | Sacramento | 95823 |  |
| No Neg Press | 916-391-6797 | 109 Otto Cir | Sacramento | 95822 |  |
| Power Printing | 916-391-2345 | 7256 Franklin Blvd | Sacramento | 95823 |  |
| Express Signs Unlimited | 916-391-2323 | 4178 Florin Rd | Sacramento | 95823 |  |
| Capitol Discount Printing | 916-388-5618 | 8605 Folsom Blvd | Sacramento | 95826 |  |
| Norwest Trade Printers Inc | 916-388-1500 | 20 Blue Sky Ct | Sacramento | 95828 |  |
| River City Trade Bindery | 916-388-0665 | 20 Blue Sky Ct | Sacramento | 95828 |  |
| Western Type & Telecommunications | 916-387-5697 | 21 Blue Sky Ct | Sacramento | 95828 |  |
| Rags Sportswear | 916-387-5493 | 8101 Elder Creek Rd | Sacramento | 95824 |  |
| Letter Press Connection The | 916-387-2323 | 5451 Warehouse Way | Sacramento | 95826 |  |
| Eagle Press | 916-383-7850 | 8111-37 Th Ave | Sacramento | 95826 |  |
| Zuprinco Printing Co | 916-383-5757 | 5310 Power Inn Rd | Sacramento | 95820 |  |
| Wildman Custom Screen Printing | 916-383-5646 | 8121 Industrial Pkwy | Sacramento | 95824 |  |
| Print Mail 2000 Inc | 916-383-3820 | 5580 Power Inn Rd | Sacramento | 95820 |  |
| Design-Yr-Sweats | 916-381-5849 | 7670 La Riviera Dr | Sacramento | 95826 |  |
| Screen Printing Unlimited | 916-369-7876 | 10087 Mills Station Rd | Sacramento | 95827 |  |
| Sign Solutions | 916-369-0176 | 10087 Mills Station Rd | Sacramento | 95827 |  |
| Screen Graphics | 916-368-7778 | 9935 Horn Rd | Sacramento | 95827 |  |
| Apple Letterpress | 916-364-5254 | 9195 Kiefer Bl | Sacramento | 95827 |  |
| Comproforms | 916-364-0707 | 9221 Beatty Dr | Sacramento | 95826 |  |
| Thermcraft Inc | 916-363-9411 | 10388 Rockingham Dr | Sacramento | 95827 |  |
| Folsom Printing | 916-362-1338 | 10087 Mills Station Rd | Sacramento | 95827 |  |
| Print Tech | 916-362-0767 | 10265 Old Placerville Rd | Sacramento | 95827 |  |
| Georges & Shapiro Lithograph Inc | 916-361-7100 | 10321 Placer Ln | Sacramento | 95827 |  |
| Gorilla Graphics Printing | 916-361-1600 | 9912 Business Park Dr | Sacramento | 95827 |  |
| Four Color Press | 916-361-1103 | 9921 Horn Rd | Sacramento | 95827 |  |
| Nixon Douglas Printing | 916-339-2567 | 5041 College Oak Dr | Sacramento | 95841 |  |
| Advanced Graphic Design | 916-338-1238 | Missie Wy | Sacramento | 95842 |  |
| Artprint Usa | 916-334-7690 | 5920 Rosebud Ln | Sacramento | 95841 |  |
| Special Ts | 916-334-5151 | 5049 College Oak Dr | Sacramento | 95841 |  |
| Professional Printing | 916-332-8147 | 4755 Auburn Blvd | Sacramento | 95841 |  |
| Dirty Shirt Co The Sacramento Silk Screening | 916-331-4607 | 5165 Auburn Blvd | Sacramento | 95841 |  |
| Colortricity Inc | 916-325-9699 | 1005 N B St | Sacramento | 95814 |  |
| Awesome Images | 916-677-5730 | 4131 S Shingle Rd | Shingle Springs | 95682 |  |
| Losel Business Forms | 916-677-2158 | 3440 Meder Rd | Shingle Springs | 95682 |  |
| Blue Sky Screen Printing | 209-536-9679 | 11844 Campo Seco Rd | Sonora | 95370 |  |
| American Sign Products | 209-533-8800 | PO Box 4025 | Sonora | 95370 |  |
| Foothill Business Cards | 209-532-8210 | 48 S Washington St | Sonora | 95370 |  |
| Sonora Press | 209-532-4202 | 18974 Hess Ave | Sonora | 95370 |  |
| Clints Sports | 209-532-1942 | 67 S Washington St | Sonora | 95370 |  |
| Colo Rite | 209-532-1023 | 15440 Bear Cub Dr | Sonora | 95370 |  |
| Idlewild Printing & Graphics | 916-544-5720 | 854 Emerald Bay Rd | South Lake Tahoe | 96150 |  |
| A Stitch In Tim | 916-544-4876 | 1025 Emerald Bay Rd | South Lake Tahoe | 96150 |  |
| Jos Design | 916-544-2811 | 197 Shady Ln | South Lake Tahoe | 96157 |  |
| Pro Leisure | 916-542-1957 | 2140 Eloise Ave | South Lake Tahoe | 96150 |  |
| Insty Prints | 916-542-0687 | 2331 Lake Tahoe Blvd | South Lake Tahoe | 96150 |  |
| Sierra Shirts | 916-541-0888 | 1030 Industrial Ave | South Lake Tahoe | 96150 |  |
| Anchor Printing | 916-541-0378 | 854 Emerald Bay Rd | South Lake Tahoe | 96150 |  |
| Lasco | 916-257-7790 | 80 N Roop St | Susanville | 96130 |  |
| Cypress Apparel & Graphics | 916-583-7205 | 241 North Lake Bl | Tahoe City | 96145 |  |
| All Occasions | 916-581-5312 | 233 Talvista Dr | Tahoe City | 96145 |  |
| Pacific Sierra Screen Printing | 916-587-8960 | 11499 Glen Rd | Truckee | 96161 |  |
| Sierra Prints | 916-587-7910 | 12219 Business Park Dr | Truckee | 96161 |  |
| W & T Graphix | 916-587-0902 | 10095 W River St | Truckee | 96161 |  |
| Fine Line Producti | 916-587-0760 | PO Box 2452 | Truckee | 96160 |  |
| Joels Print Shop | 916-582-9387 | 12219 Business Park Dr | Truckee | 96161 |  |
| A G Manufacturing | 916-582-9169 | 10960 W River St | Truckee | 96161 |  |
| Community Ink | 916-582-9012 | 12716 Northwoods Blvd | Truckee | 96161 |  |
| Georgias Wild Creations | 209-586-3218 | 22510 Twain Harte Dr | Twain Harte | 95383 |  |
| Pip Printing West Area | 916-756-1777 | 1583 Enterprise Blvd | West Sacramento | 95691 |  |
| Delta Web Printing & Bindery | 916-375-0044 | 1871 Enterprise Blvd | West Sacramento | 95691 |  |
| Trade Rotary Corp | 916-374-8991 | 1310 Tradewinds Cir | West Sacramento | 95691 |  |
| Scott Bindery | 916-372-4165 | 1355 Tradewinds Cir | West Sacramento | 95691 |  |
| Treasure Chest Advertising Co Inc | 916-371-9500 | 1201 Shore St | West Sacramento | 95691 |  |
| J D Graphics | 916-371-4727 | 3834 Commerce Dr | West Sacramento | 95691 |  |
| Rams Printing | 916-371-2020 | 1309 Tradewinds Cir | West Sacramento | 95691 |  |
| C-Mac Printing | 916-371-1492 | 2651 Del Monte St | West Sacramento | 95691 |  |
| Ad Litho Press | 916-371-1331 | 1331 Terminal St | West Sacramento | 95691 |  |
| Ajs Olde Print Shoppe | 916-934-4579 | 134 S Butte St | Willows | 95988 |  |
| Shirt Creek | 916-666-7774 | County Fair Mall | Woodland | 95776 |  |
| Fine Arts Press | 916-662-7530 | 527 Main St | Woodland | 95695 |  |
| Gold Nugget Printing Co | 916-842-2387 | 317 W Miner St | Yreka | 96097 |  |
| Cliffs Trophies | 916-842-1706 | 111 S Oregon St | Yreka | 96097 |  |
| Illusion Studios | 916-755-4946 | 768 Chestnut St | Yuba City | 95991 |  |
| Marathon Business Forms | 916-674-5058 | PO Box 1746 | Yuba City | 95992 |  |
| Shirtworks | 916-674-1079 | 747 Sutter St | Yuba City | 95991 |  |
| Valley Rose Printing Inc Fax | 916-673-6683 | 421 Center St | Yuba City | 95991 |  |
| Fenrich Imprinting | 916-673-4754 | PO Box 786 | Yuba City | 95992 |  |
| Arrowsmith Ink | 916-673-4088 | 1135 Starlite Ln | Yuba City | 95991 |  |
| Sutte Valley Sign | 916-671-2989 | 1625 Alicia Dr | Yuba City | 95993 |  |