# Attachment K:  Wood Processing Losses

This memorandum discusses emissions from the wood processing industry, including other wood processing (EIC code 450-995-0230-0000).  (Rough emissions estimates are presented for sawmill/woodworking operations, EIC code 450-456-0230-0000, but these types of facilities are included with the point source inventory.)  Products of the industry include lumber, plywood, waferboard, medium-density fiberboard, hardboard, and engineered wood products.  Emissions include PM, THC, VOC, CO, and NOx from mechanical processes, drying, pressing, and other miscellaneous processes.

AP-42 provides emission factors for facility-specific emissions estimates (U.S. EPA, 2002).  Variables that must be known include heating energy type (natural gas, waste wood, radiowave, etc.); type of wood processed (hardwood, softwood, or a mixture); and moisture content of wood.  We identified specific wood processing facilities in the CCOSII domain and acquired as much activity data as possible for each (see attached list of facilities).  However, the available information was too incomplete to generate facility-specific emissions data.  Therefore, we gathered California statewide production data and disaggregated emissions as discussed below.

Warren (2002) reports that there was no softwood plywood nor softwood waferboard production in California from 1993 through 2000.  According to the Engineered Wood Association (Adaire, 2002), there is also no production in California of softwood plywood, oriented strandboard, and engineered wood products (which included glulam beams, wood I-joists, and laminiated veneer lumber).  The Timber Committee (2002) of the United Nations Economic Commission for Europe (UNECE) produces an annual market review of the worldwide forest products industry, including the United States.  For wood products that are produced in California, Table 1 summarizes the Timber Committee’s US production data.  In addition, the US Census Bureau (1999a-d) reports product shipments from California and the United States for various segments of the forest products manufacturing industry.  Table 2 summarizes the California and US product shipment data.  From the data in Tables 1 and 2, California’s productions of wood products are estimated as shown in Table 3.

Table 1.   Year-2000 production of wood products in the United States.

| Product | 2000 US Production (thousand board-feet) |
| --- | --- |
| Sawn softwood* | 34,681 |
| Sawn hardwood | 12,683 |
| Hardwood plywood | 765 |
| Particle board | 3,922 |
| Hardboard | 525 |
| Medium-density fiberboard | 1,060 |

*2001 production

Source:  Timber Committee of the UNECE, 2002

Table 2.   Wood product shipments for the US and California.

| Product | Value of Product Shipments ($1000) |  | California Share of US Total |
| --- | --- | --- | --- |
|  | US | California |  |
| Softwood lumber, not edge worked, manufactured from purchased lumber and edge worked (US Census 1999a; US Census 1999b) | 14,853,162 | 1,892,447 | 12.8% |
| Softwood cut stock and dimension (US Census, 1999b) | 1,005,658 | 145,076 |  |
| Hardwood lumber, not edge worked, manufactured from purchased lumber and edge worked (US Census, 1999a; US Census, 1999b) | 4,830,316 | 130,366 | 3.4% |
| Hardwood cut stock and dimension (US Census, 1999b) | 1,250,186 | 76,722 |  |
| Hardwood plywood, except prefinished hardwood plywood made from purchased hardwood plywood (US Census, 1999c) | 1,219,392 | 29,343 | 2.4% |
| Particleboard made from particleboard produced at this location (US Census, 1999d) | 1,263,554 | 69,773 | 5.5% |
| Hardboard made from purchased hardboard (US Census, 1999d) | 207,125 | 3,346 | 1.6% |
| Prefinished particleboard and medium density fiberboard (mdf) made from purchased particleboard and mdf (US Census, 1999d) | 696,867 | 96,679 | 14% |

Table 3.   Estimated production quantities of wood products for California.

| Product | Proportion of US total | 2000 CA Production (thousand board-feet) |
| --- | --- | --- |
| Sawn softwood | 12.8% | 4,456 |
| Sawn hardwood | 3.4% | 431 |
| Hardwood plywood | 2.4% | 18 |
| Particle board | 5.5% | 216 |
| Hardboard | 1.6% | 8 |
| Medium-density fiberboard | 14.0% | 148 |

The US Census Bureau (2001) reports county-specific employment data for sawmills (NAICS 321113), hardwood plywood manufacturing (NAICS 321211), and reconstituted wood product manufacturing (NAICS 321219).  As a group, the CCOSII counties have 32%, 4%, and 47% (respectively) of all California’s workers in these industries.  As shown in Table 4, these proportions were used with the statewide production quantities listed in Table 3 to estimate the production quantities of wood products for the CCOSII region.

Table 4.   Estimated production quantities of wood products for the CCOSII area.

| Product | Proportion of California total | 2000 CCOS Production (thousand board-ft) |
| --- | --- | --- |
| Sawn softwood | 32% | 1,442 |
| Sawn hardwood | 32% | 140 |
| Hardwood plywood | 4% | 0.7 |
| Particle board | 47% | 101 |
| Hardboard | 47% | 3.9 |
| Medium-density fiberboard | 47% | 69 |

Table 5 tabulates comprehensive emission factors for various wood products and industrial processes as reported by the U.S. EPA (U.S. EPA, 2002).  Conversion factors were applied to create a set of emission factors with consistent units of measure (listed in Table 6).  These emission factors were applied to the 2000 CCOSII wood product production quantities (listed in Table 4) in order to estimate emissions.  Other than those for sawn softwood and hardwood, the emission factors presented in Tables 5 and 6 do not account for emissions controls.  Therefore, the use of cyclones or baghouses with control efficiencies of 85% are assumed for PM and PM10.  An example calculation is provided below.  Table 7 summarizes the CCOSII emissions results for all types of wood products and pollutants.

CCOSII production of particleboard = 101 thousand board-feet

101 thousand bd-ft  (4.11 + 0.89) lb PM/bd-ft  2000 lb/ton  (1  0.85) = 38 tons PM

101 thousand bd-ft  (1.29 + 0.022) lb PM10/bd-ft  2000 lb/ton  (1  0.85) = 10 tons PM10

101 thousand bd-ft  (3.36 + 0.023) lb NOx/bd-ft  2000 lb/ton = 171 tons NOx

Table 5.   Comprehensive emission factors for wood products processing, as reported by EPA (US EPA, 2002).

| Product | Processes Included | Emission Factors |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PM | PM10 | NOx | CO | VOC | Emission Factor Units |
| Sawn softwood | Cyclone for truck chip bin at chip and saw operations (2); Cyclone receiving wood refuse from trim saw and planar at chip and saw operations; | 0.608 |  |  |  |  | pounds per oven-dried ton |
| Sawn hardwood | Cyclone for truck chip bin at chip and saw operations (x2); Cyclone receiving wood refuse from trim saw and planar at chip and saw operations; | 0.608 |  |  |  |  | pounds per oven-dried ton |
| Hardwood plywood | Press; wood-fired plywood dryer; plywood trim chipper; sander; saw | 0.443 | 0.11a | 0.17 | 0.23 | 0.402 | pounds of pollutant per thousand square feet of 3/8-inch thick panel |
| Particle board | Wood-fired rotary dryer; flaker/refiner/hammermill | 2.2 | 0.69 | 1.8 | 0.59 | 1.16 | pounds per oven-dried ton |
| Particle board | Press; cooler; sander | 0.657 | 0.016 | 0.017 | 0.37 | 1.27 | pounds of pollutant per thousand square feet of 3/4-inch thick panel |
| Hardboard | Wood-fired dryer, ovens, press preheaters; log chipper; refiner | 2.47 | 0.6a | 0.44 | 0.085 | 1.595 | pounds per oven-dried ton |
| Hardboard | Press; sander; humidification | 0.29 | 0.086 | 0.0028 | 0.16 | 1.28 | pounds of pollutant per thousand square feet of 1/8-inch thick panel |
| Medium-density fiberboard | Wood-fired dryer; chipper; former | 10.99 | 1.6 | 12a | 4 | 6.77 | pounds per oven-dried ton |
| Medium-density fiberboard | Press; cooler; sander; saw; hogger | 0.434 | 0.154 | 0.03 | 0.034 | 1.07 | pounds of pollutant per thousand square feet of 3/4-inch thick panel |

aNo emission factor data were available.  Represents an assumed value that was estimated from the CO:NOx or PM10:PM ratios for other wood-fired dryers.

Table 6.   Comprehensive emission factors for wood products processing (with units conversion).

| Product | Processes | Emission Factors (lb per board-foot) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PM | PM10 | NOx | CO | VOC | Conversion Factor |
| Sawn softwood | Cyclone for truck chip bin at chip and saw operations (2); Cyclone receiving wood refuse from trim saw and planar at chip and saw operations; | 0.78 |  |  |  |  | 31 lbs/ft3  82.99 ft3/bd-ft  2000 lbs/ton |
| Sawn hardwood | Cyclone for truck chip bin at chip and saw operations (x2); Cyclone receiving wood refuse from trim saw and planar at chip and saw operations; | 1.01 |  |  |  |  | 40 lbs/ft3  82.99 ft3/bd-ft  2000 lbs/ton |
| Hardwood plywood | Press; wood-fired plywood dryer; plywood trim chipper; sander; saw | 1.15 | 0.29 | 0.44 | 0.60 | 1.04 | 0.5 kg/m3  2.205 lb/kg  m3/35.3 ft3  82.99 ft3/bd-ft |
| Particle board | Wood-fired rotary dryer; flaker/refiner/hammermill | 4.11 | 1.29 | 3.36 | 1.10 | 2.16 | 45 lbs/ft3  82.99 ft3/bd-ft  2000 lbs/ton |
| Particle board | Press; cooler; sander | 0.89 | 0.022 | 0.023 | 0.50 | 1.71 | 0.26 kg/m3  2.205 lb/kg  m3/35.3 ft3  82.99 ft3/bd-ft |
| Hardboard | Wood-fired dryer, ovens, press preheaters; log chipper; refiner | 4.10 | 1.02 | 0.73 | 0.14 | 2.65 | 40 lbs/ft3  82.99 ft3/bd-ft  2000 lbs/ton |
| Hardboard | Press; sander; humidification | 2.32 | 0.687 | 0.0224 | 1.28 | 10.22 | 1.54 kg/m3  2.205 lb/kg  m3/35.3 ft3  82.99 ft3/bd-ft |
| Medium-density fiberboard | Wood-fired dryer; chipper; former | 18.2 | 2.66 | 20 | 6.6 | 11.2 | 40 lbs/ft3  82.99 ft3/bd-ft  2000 lbs/ton |
| Medium-density fiberboard | Press; cooler; sander; saw; hogger | 0.58 | 0.21 | 0.040 | 0.05 | 1.44 | 0.26 kg/m3  2.205 lb/kg  m3/35.3 ft3  82.99 ft3/bd-ft |

aNo emission factor data were available.  Represents an assumed value that was estimated from the CO:NOx or PM10:PM ratios for other wood-fired dryers.

Table 7.   Total emissions from wood processing in the CCOSII area.

| Product | CCOS Production (thousand board-ft) | Total PM-Controlled Emissions (tons per year) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PM | PM10 | NOx | CO | VOC |
| Sawn softwood | 1,442 | 564 |  |  |  |  |
| Sawn hardwood | 140 | 70 |  |  |  |  |
| Hardwood plywood | 0.7 | 0.1 | 0.01 | 0.1 | 0.2 | 0.4 |
| Particle board | 101 | 37.8 | 9.9 | 171 | 81 | 196 |
| Hardboard | 3.9 | 1.9 | 0.50 | 1.5 | 2.8 | 25 |
| Medium-density fiberboard | 69 | 98.1 | 14.9 | 693 | 232 | 440 |
| Total |  | 772 | 25 | 866 | 316 | 662 |

Emissions for each type of wood product are disaggregated to the county level according to employment in the associated industry.  For counties that are split between air basins (El Dorado, Placer, and Solano), emissions are further distributed to air basins proportionally according to total industrial employment in each area.  An example for Menodocino County follows.  Table 8 summarizes emissions data for the individual CCOSII counties.

CCOSII total PM emissions = 772 tons PM per year

CCOSII total PM emissions for sawmills = 564 + 70 = 634 tons PM per year

CCOSII total PM emissions for reconsitituted wood products = 37.8 + 1.9 + 98.1 = 137.8 tons PM per year

CCOSII total PM emissions for hardwood plywood = 0.1 tons PM per year

Mendocino county employment in sawmills, reconsitituted wood products, and hardwood plywood: 249; 74; and 0

CCOSII total employment in sawmills, reconsitituted wood products, and hardwood plywood: 1702; 355; and 18

634 tons/yr  (249/1702) + 137.8 tons/yr  (74/355) + 0.1 tons/yr  (0/18) = 94 + 29 + 0 = 120 tons PM/yr in Mendocino

Table 8.   Employment in the wood products industries and associated emissions for the CCOSII counties.

| County | Employment |  |  | Emissions (tpy) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Employment in Sawmills | Reconsitituted Wood Product Manufacturing | Hardwood Veneer and Plywood Manufacturing | PM | PM10 | NOx | CO | VOC |
| Amador | 39 | 74 |  | 43 | 5 | 180 | 65 | 133 |
| Butte | 9 | 74 |  | 32 | 5 | 180 | 65 | 133 |
| Calaveras | 9 |  |  | 3 | 0 | 0 | 0 | 0 |
| El Dorado | 124 |  |  | 46 | 0 | 0 | 0 | 0 |
| Mariposa | 9 |  | 9 | 4 | 0.26 | 0.8 | 1.5 | 12.8 |
| Mendocino | 249 | 74 |  | 121 | 5 | 180 | 65 | 133 |
| Nevada | 9 |  |  | 3 | 0 | 0 | 0 | 0 |
| Placer | 124 | 124 |  | 94 | 9 | 302 | 109 | 222 |
| Plumas | 249 |  |  | 93 | 0 | 0 | 0 | 0 |
| Sacramento | 9 |  |  | 3 | 0 | 0 | 0 | 0 |
| Shasta | 617 |  | 9 | 231 | 0.26 | 0.8 | 1.5 | 12.8 |
| Sierra | 74 |  |  | 28 | 0 | 0 | 0 | 0 |
| Solano |  | 9 |  | 3 | 0.6 | 22 | 8 | 16 |
| Tehama | 9 |  |  | 3 | 0 | 0 | 0 | 0 |
| Tuolumne | 124 |  |  | 46 | 0 | 0 | 0 | 0 |
| Yolo | 9 |  |  | 3 | 0 | 0 | 0 | 0 |
| Yuba | 39 |  |  | 15 | 0 | 0 | 0 | 0 |
| CCOSII Total | 1,702 | 355 | 18 | 772 | 25 | 866 | 316 | 662 |

## References

Adaire C. (2002) Personal communication with Craig Adaire market research specialist with the Engineered Wood Association, Tacoma, Washington, August.  Tel. 253-565-6600.

_____ (1999) Directory of the wood products industry.  Portland, Oregon: Miller Freeman Publications.

Timber Committee of the UNECE (2002) Forest Products Annual Market Review 2001-2002, Timber Bulletin, Vol. LIV, ECE/TIM/BULL/55/3, United Nations Publications ISSN 0259-4323, March.  Accessed from web site http://www.unece.org/trade/timber/docs/rev-02/rev02.htm.

US Census Bureau (2001) County business patterns database, 1998-1999.  Database no. CD-CBP-98/99 published by the US Census Bureau, Washington, DC, May.

US Census Bureau (1999a) Manufacturing Industry Series of the 1997 Economic Census – Sawmills.  Report EC97M-3211A prepared by the US Census Bureau, Washington, D.C., November.  Accessed from web site http://www.census.gov/prod/ec97/97m3211a.pdf.

US Census Bureau (1999b) Manufacturing Industry Series of the 1997 Economic Census – Cut Stock, Resawing Lumber, and Planing.  Report EC97M-3219B prepared by the US Census Bureau, Washington, D.C., October.  Accessed from web site http://www.census.gov/prod/ec97/97m3219b.pdf.

US Census Bureau (1999c) Manufacturing Industry Series of the 1997 Economic Census – Hardwood Veneer and Plywood Manufacturing.  Report EC97M-3212A prepared by the US Census Bureau, Washington, D.C., August.  Accessed from web site http://www.census.gov/prod/ec97/97m3212a.pdf.

US Census Bureau (1999d) Manufacturing Industry Series of the 1997 Economic Census – Hardwood Veneer and Plywood Manufacturing.  Report EC97M-3212A prepared by the US Census Bureau, Washington, D.C., August.  Accessed from web site http://www.census.gov/prod/ec97/97m3212a.pdf.

US Environmental Protection Agency (2002) Compilation of Air Pollutant Emission Factors, Volume 1: Stationary Point and Area Sources,” AP-42, Chapter 10: Wood Products Industry. Office of Air Quality Planning and Standards, Research Triangle Park, NC.  5th edition (January 1996) and updates through August, 2002.  Accessed from web site http://www.epa.gov/ttn/chief/ap42/index.html.

Warren D.D. (2002) Production, prices, employment, and trade in northwest forest industries, all quarters 2000.  Resource bulletin PNW-RB-236 prepared by the Pacific Northwest Research Station of the Forest Service of the US Department of Agriculture, January.  Accessed from web site http://www.fs.fed.us/pnw/pubs/.

_____ (1998) Wood technology. North American factbook.  San Francisco, California: Miller Freeman Publications.

Wood products processing facilities located in the CCOSII area.

(Sources: Directory of the Wood Products Industry, 1999; Wood Technology, 1998)

| Lumbermills |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Address | County | 1996 Production | 1996 Production | Daily Production Capacity | 8-Hr Shift Capacity | Employees |
|  |  |  | bd. ft. | bd. ft. | bd. ft. | bd. ft. |  |
| Big Valley Lumber Co. | Burney, CA | Shasta | 42,000,000 |  |  |  |  |
| Wetsel-Oviatt Lumber Co | Folsom, CA | Sacramento | 58,000,000 |  |  |  |  |
| Georga-Pacific Corp | Martell, CA | Amador | 137,000,000 |  |  |  |  |
| P&M Cedar Products Inc. | Anderson, CA | Shasta | All 3 CA sites + 1 OR site: 95,000,000 | NA | NA | NA | 150 |
| Sierra Pacific Industries | Anderson, CA | Shasta |  | NA |  |  | NA |
| Sound Stud | Anderson, CA | Shasta |  | NA |  |  | NA |
| Harwood Products | Branscomb, CA | Mendocino | 90,000,000 | 75,000,000 | 500,000 | 250,000 | 230 |
| Sierra Pacific Industries | Burney, CA | Shasta |  | NA | 480,000 | 240,000 | 175 |
| Sierra Pacific Industries | Camino, CA | El Dorado |  | NA | NA |  | NA |
| Collins Pine Co | Chester, CA | Plumas |  | 75,000,000 | 300,000 | 150,000 | 200 |
| Fiberboard Corp | Chinese Camp, CA | Tuolomne |  | NA | NA | NA | NA |
| Sierra Pacific Industries | Chinese Camp, CA | Tuolomne |  | 48,000,000 | NA | NA | 125 |
| Georga-Pacific Corp | Fort Bragg, CA | Mendocino | 177,000,000 | 177,000,000 | 650,000 | NA | 350 |
| Louisiana pacific Corp. | Fort Bragg, CA | Mendocino |  | NA | NA | NA | NA |
| Sierra Pacific Industries | Lincoln, CA | Placer |  | NA | NA | 325,000 | 169 |
| Sierra Pacific Industries | Loyalton, CA | Sierra |  | NA | NA | NA | 145 |
| Cedar Creek Horselogging & Milling Inc. | Nevada City, CA | Nevada |  | 5,000,000 | 4,000 | 4,000 | NA |
| J. W. Bamford Inc. | Oroville, CA | Butte |  | NA | 8,000 | NA | NA |
| Cal Oak Products | Oroville, CA | Butte |  | 100,000,000 | 2,000 | 2,000 | 20 |
| Foothill Mill & Lumber Co | Paradise, CA | Butte |  | NA | NA | 2,000 | NA |
| P&M Cedar Products Inc. | Pioneer, CA | Amador | All 3 CA sites + 1 OR site: 95,000,000 | NA | NA | NA | 45 |
| Sierra Pacific Industries | Quincy, CA | Plumas |  | NA | 600,000 | 267,000 | 189 |
| Butler Forest Products | Redding, CA | Shasta |  | 8,600,000 | 120,000 | 60,000 | 24 |
| Wisconsin-California Forest Products | Redding, CA | Shasta |  | NA | 260,000 | 130,000 | 100 |
| Sierra Pacific Industries | Richfield, CA | Tehama |  | NA | NA | NA | NA |
| Sierra Pacific Industries | Shasta Lake, CA | Shasta |  | NA | NA | NA | NA |
| Deutschman Lumber Co. | Sonora, CA | Tuolomne |  | NA | NA | NA | 2 |
| Sierra Pacific Industries | Standard, CA | Tuolomne |  | NA | NA | NA | NA |
| Louisiana pacific Corp. | Ukiah, CA | Mendocino |  | NA | NA | NA | NA |
| Willits Redwood Co | Willets, CA | Mendocino |  | NA | 40,000 | 25,000 | 18 |

| Hardwood Plywood Manufacturing Facilities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Address | County | Ann. Capacity | Production | Employees |
| Pry Core Inc | Anderson, CA | Shasta | NA | NA | 20 |
| Louisiana-Pacific Corp. | Oroville, Ca | Butte | NA | NA | NA |

| Particleboard Manufacturing Facilities |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Address | County | Ann. Capacity (1997-8 WTNAF) | Ann. Capacity (1997 DWPI) | Production | Employees (1997 DWPI) | Employees (1998 DWPI) |
|  |  |  | bd. ft., 3/4 in. basis | sq.ft., 3/4 in. basis |  |  |  |
| Georgia Pacific | Martell, CA | Amador | 140,000,000 | 150,000,000 | NA | 150 |  |
| Sierrapine, Ltd. | Rocklin, CA | Placer |  | NA | NA |  | 170 |
| International Paper Company | Ukiah, CA | Mendocino |  | NA | NA |  | 420 |

| Hardboard Manufacturing Facilities |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Address | County | Ann. Capacity (1997-8 WTNAF) | Ann. Capacity (1997 DWPI) | 1995 Production (1997 DWPI) | Employees (1997 DWPI) | Employees (1998 DWPI) |
|  |  |  | sq.ft., 1/8 in. basis | sq.ft., 1/8 in. basis | sq.ft., 1/8 in. basis |  |  |
| Louisiana-Pacific Corp | Oroville, CA | Butte | 220,000,000 | NA | NA | 150 |  |
| International Paper Co. | Ukiah, CA | Mendocino |  | 525,000,000 | 371,000,000 | 420 |  |
| Louisiana-Pacific Corp | Oroville, CA | Butte |  | NA | NA |  | NA |
| Sierrapine, Ltd. | Rocklin, CA | Placer |  | NA | NA |  | NA |
| International Paper Co. | Ukiah, CA | Mendocino |  | NA | NA |  | 420 |

| Medium-Density Fiberboard Manufacturing Facilities |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Address | County | Ann. Capacity (1997-8 WTNAF) | Ann. Capacity (1997 DWPI) | Production | Employees (1998 DWPI) |
|  |  |  | bd. ft., 3/4 in. basis |  |  |  |
| Louisiana-Pacific Corp | Oroville, CA | Butte | 50,000,000 | NA | NA | NA |
| Sierrapine, Ltd. | Rocklin, CA | Placer | 90,000,000 | NA | NA | 170 |