Cargo Handling Equipment

Alternative Fuel Technologies

Preliminary research

| Equipment Type | Alternative Fuel or Engine | Potential Emission Reductions (per vehicle) | Cost | Cost-Effectiveness (per ton NOx) | Examples of Current Use |  |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Tractors | Propane, LNG, CNG, hydraulic hybrid, hydrogen-electric | Up to 100% emission reduction | N/A/ | N/A | Port of Los Angeles and Long Beach (Technology Advancement Program), Port of Rotterdam |  |
| Forklifts | Electric battery, propane, CNG/LNG/LPG , hydrogen-electric | Up to 100% emission reduction | N/A | N/A | commercially deployed in thousands of warehouses, airports, railyards and ports worldwide; demonstration projects happened |  |
| Straddle Carriers | Diesel-Electric Hybrid: Electric: Kalmar ESC450W12 Noell NSC E (variable electric AC drive)13 (top manufacturers: Noell, Kalmar) Diesel-Electri Hybrid: Noell SC644 E14 | Hybrid9: consume ~20% less fuel, emit up to 90% less; Kalmar hybrid package reduces fuel consumption 25-30% and 50 tonnes CO2 emissions per SC per year10 | Between $1.2 and $1.6 million each9, ~$1.0 – 1.3 million 10 | N/A |  | Ports of Barcelona, Spain; and Virginia Port Authority (Norfolk and Portsmouth included – Kalmar, Noell, and Konescranes) Ports of Auckland9, DP World Southampton, UK10, PSA HNN Antwerp, Belgium. Tilbury Container Services, England11, Port of Le Havre in France12, Durban Container Terminal in South Africa14, Hamburger Hafen und Logistik AG |
| Rubber Tired Gantry Crane | Electric, flywheel, Diesel-Electric Hybrid9, Electric motor (e-RTGC)9, Konecranes Fuel Saving System (FSS) - variable speed diesel alternator system11 Konescranes RTGC with FSS fitted with “cable reel system” 11 | Virtually 100% through electrification; Hybrid9: GHG and energy consumption reduction of 40% (66t-CO2/unit and 25kL/unit), 700kg of NOX and 520Kg of PM e-RTGC9: GHG emission reduced 64.4%, energy consumption cut down not less 84% ($22,000) than diesel engines ($0.13millions) per unit Konescranes FSS: reduce fuel consuption and emissions by up to 40% Konescranes FSS w/cable reel: claim, “reduces the local emissions by 95%” 11 | Retrofit Electrification: N/A Flywheel: N/A Hybrid: $400,000 USD9 Purchase of a new electrified RTG: N/A New e-RTGC: $300,000/unit9 | N/A |  | Retrofits and new cranes commercially available now, Hybrid: Port of Tokyo9, e-RTCG: Busan Container Port in Korea9 |
| Rail cargo handling machine | Electric9 | (Per month reductions) 5000-liter of diesel consumption and at least 13,900kg of greenhouse gases (including CO2, CH4 and N2O)9 | N/A | ~$1.7 million9 |  | Port of Keelung, Chinese Taipei9 |

Capacity TJ9000 CNG

http://www.doughertyequipment.com/new_vehicle_detail.asp?veh=221591#detail

Capacity TJ9000 LNG

http://www.doughertyequipment.com/new_vehicle_detail.asp?veh=221591#detail

Cargotec Kalmar Ottowa terminal truck

http://www.cargotec.com/cms/cargocms20.nsf/Documents/E8F510CA04CEFC34C22577D6004D16E1?openDocument&lang=1

and US Hybrid terminal truck

http://greenbigtruck.com/2011/01/new-york-terminal-gets-hybrid-tractors/

and Parker Hannifin RunWise Hydraulic Hybrid

http://epa.gov/cleandiesel/documents/et-letter-parker-hannifin-run-wise.pdf

and Eaton Hydraulic Launch Assist

http://www.eaton.com/Eaton/OurCompany/NewsEvents/NewsReleases/PCT_199871

Balqon/Capacity/Vision terminal truck ZETT/PHETT

http://fundamentalquestioning.blogspot.com/2011/08/board-of-harbor-commissioners-meeting.html

Toyota Electric Drive System

H2 Logic H2Drive/Dantruck 3000 Power Hydrogen

http://www.h2logic.com/com/dantruck-3000-power-hydrogen.asp

and Plug Power GenDrive forklift

http://www.plugpower.com/AboutUs/Documentation_Literature.aspx

Conductix Wampfler E-RTG retrofit

http://www.conductix.fr/public/docs/CX/KAT0000-0004-E_E-RTG_V2_Letter_Size%28web%29_20090128_170032.pdf

Kalmar E-one squared zero emissions & hybrid systems

http://www.kalmarind.com/show.php?id=1251535

Vycon REGEN

http://www.portvision.eu/repository/documents/a6afd792-c245-4c5e-83ee-3960e980b800RTGDocument.pdf

9 www.fmc.gov/userfiles/pages/file/GHGBestPractices.pdf

10 http://www.cargotec.com/cms/cargocms20.nsf/documents/B671895786B7F584C2257520003BAEA2?OpenDocument&lang=1

11 http://www.cargobusinessnews.com/Feb09/hybrid.html (www.konecranes.com )

12 Kalmar Around the World (Cargotec) Customer Magazine Issue 3-2010  http://www.kalmarind.co.uk/pages/news-details.php?id=46

13 http://www.noellmobilesystems.com/en/straddle-carrier-type-nsc-e-.126.html

14 http://www.noellmobilesystems.com/en/news.156.html