DRAFT 10-11-2007

Regulation to Control Emissions from In-Use On-Road Diesel-Fueled Heavy-Duty

Drayage Trucks

Adopt section 2027 of article 3, chapter 1, division 3 title 13, California Code of

Regulations to read as follows:

Section 2027 Purpose and Definitions of Control Measure

(a) Purpose. The purpose of this regulation is to reduce emissions and public

exposure to diesel particulate matter (diesel PM), oxides of nitrogen (NOx), and

other air contaminants by setting emission standards for in-use, heavy-duty

diesel-fueled vehicles that transport cargo to and from California’s ports and

intermodal rail facilities.

(b) Applicability

(1) This regulation applies to owners and operators of on-road diesel-fueled

heavy-duty drayage trucks operated at California ports and intermodal rail

yard facilities. This regulation also applies to “motor carriers,” “marine or

port terminals,” “intermodal rail yards,” “beneficial cargo owners”, and “rail yard and port authorities.”

(2) This regulation does not apply to:

(A) dedicated use vehicles;

(B) vehicles operating under an ARB authorized emergency decree;

(C) authorized emergency vehicles;

(D) military tactical support vehicles;

(E) vehicles that operate at port or rail yard properties in which the ARB

Executive Officer has granted an annual exemption under the

provisions of subsection (f) to local port or rail yard authorities; and

(F) yard trucks.

(c) Definitions. For purposes of this section, the definitions of Health and Safety

Code section 39010 through 39060 apply except to extent that such definitions

may be modified by the following definitions that apply specifically to this

regulation.

(1) “ARB” means the California Air Resources Board.

(2) “ARB Designees” are defined as those entities that ARB designates or

contracts with to perform certain functions or provide specific services on

its behalf under this regulation.

(3) “Authorized Emergency Vehicle” is as defined in Vehicle Code section

165.

(4) “Beneficial Cargo Owner” is a cargo owner, the person for whose account

the ocean or rail transportation is provided, the person to whom delivery is

to be made, a shippers' association, or an ocean or rail transportation

intermediary that accepts responsibility for payment of all applicable

charges.

(5) “Bill of Lading” is a document that states the terms of the contract between

a shipper and a transportation company. It serves as a document of title of

the goods shipped, a contract of carriage, and a receipt for goods.

(6) “CARB Diesel Fuel” is diesel fuel certified by ARB as meeting the fuel

specification standards set forth at title 13, California Code of Regulations

(CCR) section 2280 et seq.

(7) “Class I Railroad” is a freight railway based on large revenues ($250

million or more) in comparison to the revenues of Class II (which ranges

from greater than $20 million but less than $250 million) and Class III (less

than $20 million) railways, as defined by the Surface Transportation Board

(STB).

(8) “Compliance Label” is a tag issued by ARB or its designee under the

Drayage Truck Registry for heavy-duty drayage trucks operated at the

ports and intermodal rail yards that meet the requirements and compliance

schedules of subsection (d) of this regulation.

(9) “Dedicated Use Vehicles” are uni-body vehicles that do not have separate

tractor and trailers and include but are not limited to:

(A) Dedicated auto transports;

(B) Dedicated fuel delivery vehicles;

(C) Concrete mixers;

(D) On-road Mobile Cranes

(10) “Diesel Fuel” means any fuel that is commonly or commercially known,

sold, or represented by the supplier as diesel fuel, including any mixture or

primarily liquid hydrocarbons (HC) – organic compounds consisting

exclusively of the elements carbon and hydrogen – that is sold or

represented by the supplier as suitable for use in an internal combustion,

compression – ignition (CI) engine.

(11) “Diesel-Fueled” means a CI engine fueled by diesel fuel, CARB diesel

fuel, or jet fuel, in whole or part, including liquid natural gas (LNG) engines

using diesel-fuel for pilot injection are subject to the requirements of this

regulation.

(12) “Diesel particulate matter (diesel PM)” means the particles found in the

exhaust of diesel-fueled compression ignition engines. Diesel PM may

agglomerate and adsorb other species to form structures of complex

physical and chemical properties. ARB has identified diesel PM as a toxic

air contaminant.

(13) “Drayage Truck” means any in-use on-road vehicle with a gross vehicle

weight rating (GVWR) of 26,001 pounds or greater operating on or

transgressing through port or intermodal rail yard property for the purpose

of loading, unloading or transporting cargo, such as containerized, bulk or

break-bulk goods.

(14) “Drayage Truck Owner” means:

(A) the person registered as the owner of a drayage truck as shown by

the Department of Motor Vehicles, or its equivalent in another state,

province, or country; or the International Registration Plan.

Or

(B) the lessee of the truck, as indicated on the drayage truck’s

registration pursuant to Vehicle Code section 4453.5.

(15) “Drayage Truck Driver” means a driver who possesses a valid commercial  driver’s license and any required endorsements and drives a drayage truck for a motor carrier, regardless whether that person is an employee of the motor carrier or an independent contractor who operates under the operating authority of the motor carrier pursuant to federal law.

(16) “Drayage Truck Registry (DTR)” is an ARB database that contains

information on all trucks that conduct business at California ports and

intermodal rail yards.

(17) “Drayage Truck Registry Number” is a unique identifier issued to the

owner of a drayage truck upon registering in the DTR and corresponds to

the truck registered.

(18) “Emergency Event” means any situation arising from sudden and

reasonably unforeseen natural disaster such as earthquake, flood, fire, or

other acts of God, or other unforeseen events beyond the control drayage

truck owners and operators that threatens public health and safety or the

reasonable flow of goods movement.

(19) “Emergency Decree” means a determination by the Executive Officer that

an emergency event has occurred that requires the immediate temporary

operation of drayage trucks at ports and rail yard facilities.

(20) “Executive Officer” is the Executive Officer of ARB or his/her authorized

representative.

(21) “Gross Vehicle Weight Rating (GVWR)” is as defined in Vehicle Code

Section 350.

(22) “Heavy-Duty” is a manufacturer’s gross vehicle weight rating of greater

than 26,001 or more pounds.

(23) “Intermodal Rail Yard” is any rail facility within 50 miles of a port where

cargo is transferred from truck to train or vise versa. Intermodal rail yards

include, but are not limited to, the following facilities: Union Pacific (UP)

Oakland, Burlington Northern Santa Fe (BNSF) Hobart, LATC Union

Pacific, Commerce UP, Richmond BNSF, Commerce Eastern BNSF, ICTF

UP, San Bernardino, Stockton Intermodal BNSF, Lathrop Intermodal UP,

and BNSF Oakland.

(24) “International Registration Plan” is a registration reciprocity agreement

among states of the United States and provinces of Canada providing for

payment of license fees on the basis of total distance operated in all

jurisdictions.

(25) “Lessee” has the same meaning as in Vehicle Code section 371.

(26) “Liquid Natural Gas (LNG) Fueled Trucks” are drayage trucks that utilize a

heavy-duty pilot ignition engine that is designed to operate using an

alternative fuel, except that diesel fuel is used for pilot ignition at an

average ratio of no more than one part diesel fuel to ten parts total fuel on

any energy equivalent basis. An engine that can operate or idle solely on

diesel fuel at any time does not meet this definition.

(27) “Marine or Port Terminals” means wharves, bulkheads, quays, piers,

docks and other berthing locations and adjacent storage or adjacent areas

and structures associated with the primary movement of cargo or

materials from vessel to shore or shore to vessel including structures

which are devoted to receiving, handling, holding, consolidating and

loading or delivery of waterborne shipments or passengers, including

areas devoted to the maintenance of the terminal or equipment. For the

purposes of this regulation, the term includes but is not limited to

production or manufacturing areas, warehouses, storage facilities, and

private or public businesses or entities located on or surrounded by port

property.

(28) “Military Tactical Support Vehicles” is as defined in title 13, CCR, section

1905.

(29) “Motor Carrier” is a business entity which possesses valid interstate operating authority issued by the United States Department of Transportation that contracts with beneficial

cargo owners, ship companies, port terminals or Class I railroads for pickup

and delivery of goods and dispatches drayage truck drivers

to ports, intermodal rail yards, warehouses, distributions centers, or any other location to pick up and deliver such goods for import or export through a port.  A motor carrier may be either a motor carrier for hire or a private motor carrier.

(30) “On-road” means a vehicle that is designed to be driven on public

highways and roadways and that is registered or is capable of being

registered by the California Department of Motor Vehicles (DMV) under

Vehicle Code sections 4000 et seq. – or DMV’s equivalent in another

state, province, or country; or the International Registration Plan. A

vehicle covered under ARB’s In-Use Off-Road Regulation, title 13, CCR,

section 2449 is not an on-road vehicle.

(31) “Oxides of nitrogen (NOx)” means compounds of nitric oxide, nitrogen

dioxide, and other oxides of nitrogen. Nitrogen oxides are typically created

during combustion processes and are major contributors to smog

formation and acid deposition.

(32) “Port” is any facility used for water-borne commerce which typically

consists of different terminals, where cargo is loaded onto and unloaded

from ocean-going vessels. For the purposes of this regulation, ports

include, but are not limited to, the Port of Long Beach, Port of Los

Angeles, Port of Humboldt Bay, Port of San Diego, Port of Hueneme, Port

of Oakland, Port of San Francisco, Port of Sacramento, Port of Stockton,

Port of Redwood City, Port of Crockett, Port of Richmond, Port of

Pittsburg, and the Port of Benicia.

(33) “Port Authority” means those entities, either public or private, that are

responsible for the operation of the ports.

(34) “Port Property” means the property constituting the physical boundaries,

either contiguous or non-contiguous, of a port. For the purposes of this

regulation, port property also includes privately owned property located

within port boundaries.

(35) “Rail Yard Authority” means those entities, either public or private, that are

responsible for the operation of Class I rail yards.

(36) “Rail Yard Property” means the property constituting the physical

boundaries of intermodal rail yards. For the purposes of this regulation,

rail yard property also includes privately owned property located within rail

yard boundaries.

(37) “Uni-Body Vehicles” are vehicles that do not have separate tractor and

trailer and include but are not limited to:

(A) concrete mixers;

(B) on-road mobile cranes;

(C) on-road construction equipment.

(38) “Vehicle” is as defined in Vehicle Code Section 670.

(39) “Verified Diesel Emission Control Strategy (VDECS)” is an emission

control strategy that has been verified pursuant to the “Verification

Procedure, Warranty and In-Use Compliance Requirements for In-Use

Strategies to Control Emissions from Diesel Engines” in Title 13, California

Code of Regulations, commencing with section 2700, and incorporated by

reference.

(40) “Yard Truck” means an off-road mobile utility vehicle used to carry cargo

containers with or without chassis; also know as utility tractor rig (UTR),

yard tractor, yard goat, yard hustler, or prime mover.

(d) Requirements and Compliance Deadlines. Drayage trucks subject to this

regulation must meet the following requirements by the compliance deadlines

detailed in both Phase 1 AND Phase 2.

(1) Phase 1: By December 31, 2009, all drayage trucks must be equipped

with a:

(A) 1994 – 2003 model year engine certified to California or federal

emission standards and a level 3 VDECS for PM emissions;

or,

(B) 2004 or newer model year engine certified to California or federal

emission standards.

(2) Phase 2: By December 31, 2013, all drayage trucks must be equipped

with an engine that:

(A) meets or exceeds 2007 model year California or federal heavy-duty

diesel-fueled on-road emission standards;

or,

(B) is certified to 2004 or newer model year California or federal

emission standards.

(3) Drayage Truck Owner requirements

(A) Drayage truck owners shall:

1. meet all applicable requirements and deadlines set forth in

Phases 1 and 2 above;

2. register with the DTR, according to subsection (e);

3. upon receipt of ARB issued DTR compliance label, affix label

as required under subsection (e)(5);

4. ensure that all emission control devices are functioning

properly;

5. maintain all installed VDECS per manufacturer’s

specifications;

6. maintain and keep VDECS maintenance log in the drayage

truck and available upon request;

7. ensure that the drayage truck(s) has all information required

under subsection (d)(5)(A)(4) for the dispatching motor

carrier available and accessible in the vehicle and that the

driver of the vehicle be instructed to provide the information

upon demand to any enforcement personnel listed in

subsection (i).

(B) Phase 1 compliance deadline extension:

1. Drayage truck owners may apply for a one-time, one-year,

per-truck Phase 1 compliance deadline extension. The

compliance deadline application must be either electronically

filed or postmarked by June 1, 2009. To receive the Phase

1 compliance deadline extension, a drayage truck owner

must demonstrate all of the following:

i. the engine installed on his/her current truck is a

California or federally certified 1994 – 2003 model

year engine;

ii. the truck was registered with the DTR prior to June 1,

2009;

iii. no Level 3 diesel emission control technology verified

by ARB for use on that combination of truck and

engine was available at the time the extension was

filed.

2. Compliance extension applications shall be submitted to

ARB at:

California Air Resources Board

c/o Drayage Truck Phase1 Extension

P.O. Box 2815

Sacramento, CA, 95812

or electronically through ARB’s drayage truck website;

http://www.arb.ca.gov/drayagetruck

3. If after the one-year extension ARB verified technology is

still unavailable, the truck owner must comply with the

regulation within 90 days of the expiration of the extension

by replacing the existing heavy duty truck and / or engine

with a truck or engine that meets or exceeds the Phase 1

requirements .

(4) Drayage Truck Driver Requirements

Drayage truck drivers shall, upon demand, provide the following

information to authorized enforcement personnel as set forth in

subsection (i):

(A) the dispatching motor carrier’s contact information as detailed in

subsection (d)(5)(A)(4);

(B) the VDECS maintenance log of the drayage truck.

(5) Motor Carrier Requirements

(A) Each motor carrier shall:

1. provide a copy of this regulation or an ARB approved

summarized version to each drayage truck drivers that it

employs or contracts with for deliveries to ports and intermodal rail

yards;

2. ensure that all trucks dispatched to a port or intermodal rail

yard meet emission standards and compliance deadlines set

forth in Phases 1 and 2 in subsection (d);

3. ensure that all drayage trucks dispatched to ports and

intermodal rail yards are registered and in good standing

with the Drayage Truck Registry (DTR) and are properly

affixed with an ARB issued compliance label according to

subsection (e);

4. ensure the motor carrier information listed below is available

and accessible on each drayage truck covered by this

regulation that it dispatches and that the driver of the

vehicle is instructed to provide a copy of the information,

upon demand, to enforcement personnel, as listed in

subsection (i).

i. the motor carrier’s business name;

ii. contact person’s name;

iii. motors carrier’s street address, state, and zip code;

iv. contact person’s business phone number.

5. keep a record of all dispatched drayage trucks containing the

information set forth in i through iv below for a minimum of

five years from the dispatch date. Dispatch records are to

be made available to enforcement personnel within 72 hours

of an official written or oral request.

i. truck dispatch date and time;

ii. bill of lading or tracking number;

iii. truck license plate number and issuing state;

iv. Drayage Truck Registry number.

6.  No later than January 1, 2009 and annually thereafter, every motor carrier shall register with the ARB on an approved form which shall include, but not be limited to, the following information:

i. The name and commercial driver’s license number of all drayage truck drivers who were dispatched by the motor carrier in the previous 12 calendar months;

ii. the Drayage Truck Registry number of each drayage truck dispatched by the motor carrier in the previous 12 calendar months.

7. Every motor carrier shall make available for inspection by the ARB every drayage truck that it dispatches and all relevant records for those vehicles.

___________

(6) Marine or Port Terminals and Rail Yard Requirements

(A) Starting January 1, 2009, marine or port terminals and rail yards

shall collect the following information for each dispatching motor

carrier and each drayage truck subject to this regulation that attempts to enter

the facility not displaying a valid and current compliance label

.

1. Dispatching motor carrier:

i. business name of dispatching motor carrier;

ii. contact person’s name;

iii. street address, state, zip code of the dispatching

motor carrier;

iv. phone number of the dispatching motor carrier;

v. bill of lading or tracking number.

2. Drayage truck:

i. entry date and time;

ii. registered owner’s name;

iii. driver’s name;

iv. driver’s license number;

v. drayage truck’s license plate number and state of

issuance;

vi. drayage truck’s vehicle identification number (VIN).

All information collected in subsection (d)(6) shall be kept for a

period of not less than five years from the truck entry date and is to

be made available to enforcement personnel within 72 hours of an

official written or oral request.

(B) Marine or port terminals and rail yards shall report the information

collected in subsection (A) above to their respective authorities

according to schedule (A) below and in a format acceptable to their

respective authority.

(C) No marine or port terminal or rail yard shall permit a port drayage truck that does not display a valid Compliance Label to pick up or drop off freight at its facility.

Schedule A: Terminal Reporting Schedule

(7) Port Authorities and Rail Yard Authorities Requirements

(A) Port and rail yard authorities shall respectively report the

information collected by the port terminals and rail yards, as

detailed in subsection (d)(6), to, and in a manor and format

prescribed by, ARB according to Schedule B below. ARB reporting

parameters are detailed on ARBs website

http://www.arb.ca.gov/drayagetruck.

Date Truck Enters

Terminal or Rail Yard

Date by which Information is to

be Reported to

Port or Rail Authority

January 1 – March 31 April 15

April 1 – June 30 July 15

July 1 – September 30 October 15

October 1 – December 31 January 15

Schedule B: Port and Rail Yard Authority Reporting Schedule

(B) Port and rail yard authorities shall ensure their respective terminals

and/or rail yards abide by all Schedule A reporting deadlines.

(8)  Beneficial Cargo Owner Requirements

(A) No beneficial cargo owner or any person or entity acting as an agent for a beneficial cargo owner shall contract with a motor carrier to provide port drayage services that is not in compliance with the provisions of this regulation.

(B)  Prior to entering into an agreement with a motor carrier, the beneficial cargo owner or agent thereof shall check the ARB’s registry that the drayage truck operated by the motor carrier is in compliance which will provide port drayage is in compliance with this regulation.

(e) Drayage Truck Registry and Compliance Label Requirements

(1) Truck Owner Requirements

(A) Owners of all drayage trucks doing business at a port or intermodal

rail yard prior to September 30, 2009 and intending to continue

operations after that date must register with the DTR database by

September 30, 2009.

(B) Drayage trucks intending to begin operations at a port or intermodal

rail yard after September 30, 2009 must be registered with the DTR

database prior to commencing operations.

(C) Owners of all drayage trucks covered by the regulation must

provide the following information to ARB or its designee by mail to

the address in subsection (e)(2) or electronically through ARB’s

DTR website http://www.arb.ca.gov/drayagetruck. The information

shall include but may not be limited to:

1. truck owner name, address, and contact information (e.g.

phone number, email address, fax number);

2. engine make, model, and model year;

3. vehicle identification number (VIN);

4. vehicle license number and state of issuance;

5. compliance status, which shall include:

i. identifying whether the drayage truck has complied

with the requirements of Phases 1 and 2, set forth in

subsection (d) above;

ii. if so, how was compliance achieved (e.g. new

compliant truck or description of the level 3 VDECS

Date by which Information is to be Reported to the

California Air Resources Board

May 15

August 15

November 15

February 15

that was used), who did the installation work, and

when was it completed;

iii. if not, identifying when the drayage truck is scheduled

to come into compliance under Phases 1 or 2.

(D) After filing the initial application, the drayage truck owner shall

within 30 days of bringing a truck into compliance with Phase 1 or

2, update the DTR with the vehicle’s compliance status information

and any other changes to the vehicle’s ownership, DMV registration

status, or participation status in IRP.

(E) Upon receipt of a DTR compliance label from ARB or its designee,

the drayage truck owner must affix the label in accordance with

subsection (e)(5).

(2) Mailing Address for Filing Initial Applications and Updates. Drayage truck

owners shall submit DTR applications and motor carriers shall submit registrations and any updated information to

ARB at:

California Air Resources Board

c/o Drayage Truck Registry

P.O. Box 2815

Sacramento, CA, 95812

(3) Failure to register with the DTR or submittal of false information is a

violation of state law and subject to civil or criminal penalty.

(4) ARB or its designee shall issue a DTR compliance label upon verification

of compliance with the requirements of this regulation. The DTR label will

be number coded by year to show compliance through that year. For

example: a compliant truck issued a 2013 coded label will be able to

access ports and rail yards through 2013. After 2013, the truck will again

have to demonstrate regulatory compliance and apply for a new

compliance label.

(5) All DTR compliance labels shall be:

(A) located on or near the lower left hand corner on the outside of the

driver’s side door; and,

(B) affixed to the truck in clear view, correct side up, un-obstructed; and

kept and maintained in a manner that retains legibility.

(f) Annual Port or Rail Yard Exemption

(1) Annual Exemption. An annual exemption may be granted, under limited

circumstances, by the ARB Executive Officer to ports or rail yards. An

exemption may cover a clearly defined portion or the entirety of a port or

rail yard. The Executive Officer has sole discretion in issuing an

exemption, which will be issued to ports or rail yards that are able to

demonstrate one or more of the following:

(A) port or rail yard land is not typically used for truck traffic and its

primary function or location does not include or attract drayage

trucks covered under this regulation (e.g. a shoreline animal

sanctuary);

(B) the overwhelming majority of trucks accessing the port or rail yard

are exempted under this regulation (e.g. a port where only

dedicated auto transports are in service).

(2) The Exemption Application

(A) may be obtained from the ARBs’ website

http://www.arb.ca.gov/drayagetruck or by mail from;

California Air Resources Board

c/o Drayage Truck Port / Rail Yard Exemption App.

P.O. Box 2815

Sacramento, CA, 95812

(B) must be completed and submitted annually (via the same website

or address listed above) no later than January 1 of the year prior to

the exemption year (e.g. a 2009 year exemption application must

be completed and submitted by January 1, 2008);

(C) will be approved or disapproved by the Executive Officer no later

than July 1, of the year prior to the exemption year. The Executive

Officer will then issue an exemption to be valid for the specified port

or rail yard for the specified exemption year.

(g) Penalties. Any person who fails to comply with the performance requirements of this regulation, who fails to submit any information, report, or statement required by this regulation, or who knowingly submits any false statement or representation in any application, report, statement, or other document filed, maintained, or used for the purposes of compliance with this regulation may be subject to civil or criminal penalties under sections 39674, 39675, 42400,

42400.1, 42400.2, 42402,.2, and 43016 of the Health and Safety Code. In

assessing penalties, the Executive Officer will consider factors, including but not limited to the willfulness of the violation, the length of time of noncompliance,

whether compliance was attempted, and the magnitude of noncompliance.

(h) Right of Entry. For the purpose of inspecting on-road vehicles covered in this regulation, and their records to determine compliance with these regulations, an agent or employee of ARB, upon presentation of proper credentials, has the right to enter any facility (with any necessary safety clearances) where on-road

vehicles are located or on-road vehicle records are kept.

(i) Enforcement. Enforcement of this section may be carried out by authorized

representatives of ARB, port and rail yard authorities; peace officers as defined in

California Penal Code, Title 3, chapter 4.5, sections 830 et seq. and their

respective law enforcement agencies; and authorized representatives of air

pollution control or air quality management districts.

(j) Relationship to Other Law. Nothing in this section allows drayage trucks to

operate in violation of other applicable law, including, but not limited to:

(1) California Vehicle Code;

(2) California Health and Safety Code;

(3) division 3, title 13, California Code of Regulations;

(4) any applicable ordinance, rule, or requirement as stringent as, or more

stringent than, than the requirements of subsection (d) of this regulation.

(k) Severability. If any subsection, paragraph, subparagraph, sentence, clause,

phrase, or portion of this regulation is, for any reason, held invalid,

unconstitutional, or unenforceable by any court of competent jurisdiction, such

portion shall be deemed as a separate, distinct, and independent provision, and

such holding shall not affect the validity of the remaining portions of the

regulation.

Authority Cited: Sections 39600, 39601, 39650, 39658, 39659, 39666, 39667, 39674,

39675, 42400, 42400.1, 42400.2, 42402.2., 42410, 43013, 43016,

43018, 43023, 43600, California Health and Safety Code.

Reference: Sections 39650, 39658, 39659, 39666, 39667, 39674, 39675, 42400,

42400.1, 42400.2, 42402,.2, 42410, 40717.9, 43013, 43016, and 43018,

43023, 43600, California Health and Safety Code.