VIA ELETRONIC FILING		Londrina, August 17, 2009

E-DOCKET

California Environmental Protection Agency

Air Resources Board – CARB

California

Re: Comments of the Detailed Calofornia-Modified Greet Pathways for Brazilian

Sugarcane Ethanol: Average Brazilian Ethanol, with Mechanized Harvesting and

Electricity Co-product Credit, with Electricity Co-product Credit (July 20, 2009).

Dear Sir or Madam:

I submit the enclosed comments and correction on the Detailed Calofornia-Modified Greet Pathways for Brazilian Sugarcane Ethanol, published by CARB on July 20, 2009.

- The Input Values for Ethanol from Brazilian Sugarcane (Appendix B).

- The input value for Sugarcane Harvest Yield of 75 tonne per hectare (CA-GREET Default) is not correct.

- The current national harvest yield for ethanol sugarcane production in Brazil is 82.7 tonne per hectare, as shown in Table 1.00 (CONAB, 2009 < www.conab.gov.br  >):

2. The average truck load capacity for sugarcane transportation is 34 tonne or more, not the CARB value of 17 tonne. In this case the CO2e per MJ for cane transportation should be reduced by half (CARB Document Table 2.04).

Table 1.00. Current national harvest yield for sugarcane in Brazil.

| Region | Production (thousand tonne) | National Fraction (%) | Yield (tonne per ha) | Corrected Yield (tonne per ha) |
| --- | --- | --- | --- | --- |
| North East | 27,215 | 7.78 | 59.6 | 4.64 |
| Center West | 62,498 | 17.90 | 80.1 | 14.33 |
| South East | 231,344 | 66.23 | 85.9 | 56.91 |
| South | 28.270 | 8.10 | 84.3 | 6.82 |
| Brazil | 349,327 | 100.0 |  | 82.7 |

Source: Conab/MARA 2009 accessed on 13 August 2009.

http://www.conab.gov.br/conabweb/download/safra/1cana_de_acucar.pdf

Since the correct harvest yield of sugarcane in Brazil is 82.7 tonne per ha, all the Tables of the Document - Detailed Calofornia-Modified Greet Pathways for Brazilian Sugarcane Ethanol: Average Brazilian Ethanol, with Mechanized Harvesting and Electricity Co-product Credit, with Electricity Co-product Credit (July 20, 2009) should be reviewed and corrected.

Table C. GHG Emissions Summary for Sugarcane Ethanol

| Sugarcane Ethanol Components | GHG Emissions (gCO2e/MJ) |
| --- | --- |
| Sugarcane Farming (incl. Straw burning) | 7.0 |
| Ag Chemical Production and Use Impacts | 7.6 |
| Sugarcane Transportation | 1.0 |
| Ethanol Production | 1.8 |
| Ethanol T&D | 4.1 |
| Total Well –to-Tank | 21.5 |
| Total Tank-to-Wheel | 0 |
| Total Well-to-Wheel | 21.5* |

Note: The value of 21.5gCO2e/MJ does not include contributions from CH4 and N2O when ethanol is blended with CARBOB and used as Reformulated Gasoline in a light-duty gasoline engine.

Table A provides a summary of the WWT GHG emissions for the baseline pathway and the two additional scenarios described in this document.

Table A. Summary of Baseline Pathway and two Additional Scenarios.

| Pathway Description | WTW GHG* Emissions (gCO2e/MJ) |
| --- | --- |
| Baseline Pathway Brazilian sugarcane using average production processes | 22.1 |
| Scenario 1 Brazilian sugarcane using average production process, mechanized harvesting and electricity co-product credit | 9,6 |
| Scenario 2 Brazilian sugarcane using average production process, and electricity co-product credit | 15.1 |

*These values do not include contributions from Land Use Change.

For instances see Table 2.06 (page 26):

Impact of soil N2O emissions resulting from nitrogen fertilizer use on WTT GHG emissions.

Table 2.06 (corrected by harvest yield). Inputs and Calculated Emissions for Soil NO and N2O from Sugarcane.

|  | Fertilizer N input (g/tonne) | Percent conversion to N2O-N | N2O formed/ N2O-NO (g/g) | N Converted (g/tonne) | N2O or NO Emissions (g/tonne) | GHG Emissions (g/mmBtu) | GHG Emissions (gCO2e/MJ) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N2O | 990.0 | 1.3% | 44/28 | 12.87 | 20.2 | 3,267.8 | 3.1 |

N2O GWP relative to CO2 = 296.